To configure D-Link DI-106, DI-300 and DI-300M, please follow step by step the following menu, help yourself looking at the note for each voice of the menu. Some voice of the menu are filled as example.

Menu 1 - General Setup

System Name= DLINK-IT	(Insert system name)

Location= BARCELONA	(Insert where the router is located.)

Contact Person's Name= C Rossi	(Insert a contact name)

Route IP= Yes                                                                (For DI-106 router only: Route IP=Yes)

Route IPX= No

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

Menu 2 - ISDN Setup

Switch Type: DSS-1	(ISDN line connection)

B Channel Usage= Switch/Switch	(Swicthed Mode Communication)

ISDN Data     = 11223344 (own telephone number with ISDN code)	Subaddress=

Dial Prefix to Access Outside Line=

PABX Number (Include S/T Bus Number)=

Incoming Phone Number Matching= Multiple Subscriber Number (MSN)

Press ENTER to Confirm or ESC to Cancel:

Press Space Bar to Toggle.

Menu 3 - Ethernet Setup

1. General Setup

2. TCP/IP and DHCP Setup

3. Novell IPX Setup

4. Bridge Setup

Enter Menu Selection Number:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:	(if you do not have an DHCP server do not fill )

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 192.170.5.10	(Router IP Address)

IP Subnet Mask= 255.255.255.0	(Subnet Mask of the network)

RIP Direction= Both

Version= RIP-2B

Press ENTER to Confirm or ESC to Cancel:

Press Space Bar to Toggle.

Menu 4 - Internet Access Setup

ISP's Name= TIN	(Insert name of your provider)

Pri Phone #= 11223344		(Insert your provider phone number with prefix too )

Sec Phone #=

My Login= username	(Insert your login name)

My Password= ********	(Insert your password)

Single User Account= Yes

IP Addr= 0.0.0.0                    (Insert zero values if you have dynamic IP; in case of static IP

Insert the IP given by your provider)

Telco Option:

Transfer Type= 64K

Multilink= Off

Press ENTER to Confirm or ESC to Cancel:

Menu 11 - Remote Node Setup

1. TIN (ISP, SUA)                    (Insert to access submenu 11.1 -1 + ENTER)

2. ________

3. ________

4. ________

Enter Node # to Edit:

Menu 11.1 - Remote Node Profile

Rem Node Name= TIN	Route= IP

Active= Yes	Bridge= No

Call Direction= Outgoing

Edit PPP Options= No

Incoming:	Rem IP Addr= 0.0.0.0

Rem Login=	Edit IP/IPX/Bridge= No

Rem Password= ********	Telco Option:

Rem CLID= N/A	Transfer Type= 64K

Call Back= N/A	Allocated Budget(min)= 0

Outgoing:	Period(hr)= 0

My Login= username (Insert login name)	Session Options:

My Password= ******** (Insert password)	Input Filter Sets=

Authen= CHAP/PAP	Output Filter Sets=

Pri Phone #= 11223344 (Insert ISP phone number)	Call Filter Sets=

Sec Phone #=	Idle Timeout(sec)= 60 (Set timer to disconnect)

Press ENTER to Confirm or ESC to Cancel:

Once you made this configuration you need to set the “Network” on every workstation following the next steps:

- START  SETTINGS  CONTROL PANEL  NETWORK NEIGHBORHOOD

- Install Microsoft TCP/IP Protocol

- Your network card TCP/IP protocol  PROPERTIES

- Set an IP Address and a subnet mask for the workstation

- GATEWAY   Add as GATEWAY the IP Address given to the router

- DNS CONFIGURATION   DNS ON

- HOST  Insert the name of your ISP

- DOMAIN Insert your ISP domain

- DNS SERVER Search Order    Add DNS IP Addresses given by your ISP

- “Do you want to reboot your PC?”  OK

To configure your browser follows: this example is for MS Explorer but it’s the same for Netscape Navigator

From Explorer Menu Bar:

- INTERNET OPTIONS  CONNECTION

- Set connection by your LAN    APPLY OK

- Close your browse

When you will run your browser again D-Link Router will dial-up the call toward your ISP and let you share the Internet access throughout your network.

When the connection is on check that B1 or B2 led is lighted and D-Link led too.