Last modified: June 13, 1997

LAN to LAN Connection Routing IPX with Two NetWare Servers

NetWare

Server 1

NetWare

Client A

DI-300(M) 1

DI-300(M) 2

‘Server_1’

Network # =                                                          Network # =

NetWare

Server 2

00004444                                                                    00003333

‘Server_2’                                                                                                                                                                                                Internal Net # = 00001111

Internal Net # = 00002222

Note: The IPX network numbers shown above are examples only!

This configuration note explains how to set up a workstation on one network to use a DI-300(M) to make a connection to another DI-300(M) to route IPX.    In this configuration, the workstation, ‘NetWare Client A’ will be able to log into a NetWare server, ‘Server_1’.    There will be three items that you need to set up.    These are the workstation and the two DI-300(M)s.

I.	Workstation Setup (NetWare Client)

To set up the workstation (W/S), you will need to set the following parameters:

- IPX Frame Type - First determine the frame type that the NetWare server that you wish to login to is using (this would be the remote server, ‘Server_1’), and set the same frame type on the NetWare client workstation.

- Server Name - In some cases, you may need the name that has been configured for the NetWare server you wish to login to.    In the previous example, that would be ‘Server_1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of IPX client software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- DOS workstation w/ NetWare Client (VLM) software

- Windows 95/NT - see section ‘LAN-to-LAN’

II.	DI-300(M) 2 Setup

In order to configure DI-300(M) 2 for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_2

Location= Taipei

Contact Person's Name= CC

Route IP= No

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IPX field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.3:

Menu 3.3 - Novell IPX Ethernet Setup

Seed Router= No

Frame Type 802.2= No

IPX Network #= N/A

Frame Type 802.3= Yes

IPX Network #= N/A

Frame Type Ethernet II= No

IPX Network #= N/A

Frame Type SNAP= No

IPX Network #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Set the Seed Router field to ‘No’, since there is a Novell Server in DI-300(M)2’s LAN.

- Make sure that you have set the correct    Frame Type(s) to ‘Yes’.    You should enable the frame type or types that the NetWare server you want to logon to is running (‘Server_1’ in our example).    Please note that it is possible to set more than one frame type to ‘Yes’.

Menu 11.1:

Menu 11.1 - Remote Node Profile

Rem Node Name= ToPres1               Route= IPX

Active= Yes                          Bridge= No

Call Direction= Outgoing

Edit PPP Options= No

Incoming:                            Rem IP Addr= N/A

Rem Login=                         Edit IP/IPX/AppleTalk/Bridge= Yes

Rem Password=

Rem CLID=                          Telco Option:

Call Back= N/A                       Transfer Rate= 64K or 56K

Outgoing:                            Session Options:

My Login= username                   Input Filter Sets=

My Password= ******                  Output Filter Sets=

Pri Phone #= 5551212                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Outgoing’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- My Login and My Password is the login information that this DI-300(M) will use to connect to the remote router.

- The Pri Phone # field should be set to the correct phone number to dial.

·	Route should be set to ‘IPX’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr: N/A                     Dial-On-Query= Yes

Rem Subnet Mask= N/A                 Rem LAN Net #= 00003333

My WAN Addr= N/A                     My WAN Net #= 00000000

Single User Account= N/A             Hop Count= 1

Server IP Addr= N/A                Tick Count= 2

Metric= N/A                          W/D Spoofing(min)= 3

Private= N/A                         SAP/RIP Timeout(min)= 3

RIP= N/A

Bridge Options:

Dial-On-Broadcast= N/A

Ethernet Addr Timeout(min)= N/A

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Query should be ‘Yes’.

- The Rem LAN Net # field should be set to equal the external network number of the remote NetWare server station.    This number can be obtained by that network’s system administrator.    In our example, this value is ‘00003333’.

- SAP/RIP Timeout can be set to a greater value, so if your ISDN connection drops for an extended period of time, the DI-300(M) will still retain the proper SAP and RIP tables.    This value should be the same as the W/D Spoofing field in the server-side DI-300(M).

Menu 12.2:

Menu 12.2 - Edit IPX Static Route

Route #= 11

Server Name= Server_1

Active= Yes

Network #= 00001111

Node #= 000000000001

Socket #= 0451

Type #= 0004

Hop Count= 2

Tick Count= 3

Gateway Node= 1

Press ENTER to Confirm or ESC to Cancel:

- Set Server Name equal to name assigned to the remote NetWare server you want to login to.    In our example, this name is ‘Server_1’.

- The Active field should be set to ‘Yes’.

- Set Network # equal to the unique internal network number of the NetWare server you want to login to.    In our example, this number is ‘00001111’.

- Gateway Node should be set equal to the number of remote node (1-4) that has been configured for this application.    If you are not sure of the remote node, check menu 11.

- You should be able to make the connection by using the default values for the remaining fields.    Please note that the Socket # and Type # fields should correspond to the server you want to login to.

III.	DI-300(M) 1 Setup

In order to configure DI-300(M) 1, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_1

Location= Taipei

Contact Person's Name= CC

Route IP= No

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IPX field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551214

SPID #= 408555121400

2nd Phone #= 5551215

SPID #= 408555121500

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.3:

Menu 3.3 - Novell IPX Ethernet Setup

Seed Router= No

Frame Type 802.2= No

IPX Network #= N/A

Frame Type 802.3= Yes

IPX Network #= N/A

Frame Type Ethernet II= No

IPX Network #= N/A

Frame Type SNAP= No

IPX Network #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Set Seed Router field to ‘No’, since there is no Novell Server in DI-300(M)1’s LAN.

- Make sure that you have set the correct    Frame Type(s) to ‘Yes’.    You should enable the frame type or types that the NetWare server is running.    Please note that it is possible to set more than one frame type to ‘Yes’.

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= ToPres2               Route= IPX

Active= Yes                          Bridge= No

Call Direction= Incoming

Edit PPP Options= No

Incoming:                            Rem IP Addr= N/A

Rem Login=                         Edit IP/IPX/AppleTalk/Bridge= Yes

Rem Password=

Rem CLID=                          Telco Option:

Call Back= N/A                       Transfer Rate= 64K or 56K

Outgoing:                            Session Options:

My Login= username                   Input Filter Sets=

My Password= ******                  Output Filter Sets=

Pri Phone #= 5551212                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Incoming’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- Rem Login and Rem Password is the login information that the remote router will use to connect to this DI-300(M).

·	Route should be set to ‘IPX’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr: N/A                     Dial-On-Query= No

Rem Subnet Mask= N/A                 Rem LAN Net #= 000044444

My WAN Addr= N/A                     My WAN Net #= 00000000

Single User Account= N/A             Hop Count= 1

Server IP Addr= N/A                Tick Count= 2

Metric= N/A                          W/D Spoofing(min)= 3

Private= N/A                         SAP/RIP Timeout(min)= 3

RIP= N/A

Bridge Options:

Dial-On-Broadcast= N/A

Ethernet Addr Timeout(min)= N/A

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Query should be ‘No’.

- W/D Spoofing can be set to a greater value, so if your ISDN connection drops for an extended period of time, the DI-300(M) will still retain the proper SAP and RIP tables.    This value should be the same as the SAP/RIP Timeout field in the client-side DI-300(M).

- You should be able to make this connection by using the default values for the remaining fields.