Last modified: June 13, 1997

LAN to LAN Connection w/Cisco

Routing IPX with PC client on Cisco side and Novell Server on DI-300(M) side

NetWare

Client A

Cisco

NetWare

Server 1

DI-300(M) 1

‘Server_1’

Network # =

Network # =                                                                                                                          00003333

23456789

Internal Net # = 00001111

Note: The IPX network numbers shown above are examples only!

This configuration note explains how to set up a workstation on a network to use a Cisco 2503 to make a connection to a DI-300(M) to route IPX.    In this configuration, the workstation has NetWare client software so that you will be able to log into the NetWare server on the remote network.    There will be three items that you need to set up.    These are the workstation, Cisco 2503 and the DI-300(M).

I.	Workstation Setup (NetWare Client)

To set up the workstation (W/S), you will need to set the following parameters:

	IPX Frame Type - First determine the frame type that the Cisco router is using, and set the same frame type on the NetWare client workstation.

	Server Name - In some cases, you may need the name that has been configured for the NetWare server you wish to login to.    In the previous example, that would be ‘Server_1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of IPX client software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

	DOS workstation w/ NetWare Client (VLM) software

- Windows 95/NT - see section ‘LAN-to-LAN’

II.	DI-300(M) Setup

In order to configure the DI-300(M) on the Novell server side for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

	In menu 1, make sure the Route IPX field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

	In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

	After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.3:

Menu 3.3 - Novell IPX Ethernet Setup

Seed Router= No

Frame Type 802.2= No

IPX Network #= N/A

Frame Type 802.3= Yes

IPX Network #= N/A

Frame Type Ethernet II= No

IPX Network #= N/A

Frame Type SNAP= No

IPX Network #= N/A

Press ENTER to Confirm or ESC to Cancel:

	Since DI-300(M) is connected on the Novell server side, set that Seed Router field to ‘No’.

	Make sure that you have set the correct    Frame Type(s) to ‘Yes’.    You should enable the frame type that the NetWare server is running.    ( Please note that it is possible to set more than one frame type to ‘Yes’ if you have more than one server and they are using different frame types.)

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= ToCisco               Route= IPX

Active= Yes                          Bridge= No

Call Direction= Incoming

Edit PPP Options= No

Incoming:                            Rem IP Addr= N/A

Rem Login=                         Edit IP/IPX/AppleTalk/Bridge= Yes

Rem Password=

Rem CLID=                          Telco Option:

Call Back= N/A                       Transfer Rate= 64K or 56K

Outgoing:                            Session Options:

My Login= username                   Input Filter Sets=

My Password= ******                  Output Filter Sets=

Pri Phone #= 5551212                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

	The Call Direction should be set to ‘Incoming’.

	Make sure the remote node is enabled by setting Active to ‘Yes’.

	Rem Login and Rem Password is the login information that this DI-300(M) will use to authenticate Cisco.

·	Route should be set to ‘IPX’.

	Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr: N/A                     Dial-On-Query= No

Rem Subnet Mask= N/A                 Rem LAN Net #= 2345678

My WAN Addr= N/A                     My WAN Net #= 00000000

Single User Account= N/A             Hop Count= 1

Server IP Addr= N/A                Tick Count= 2

Metric= N/A                          W/D Spoofing(min)= 3

Private= N/A                         SAP/RIP Timeout(min)= 3

RIP= N/A

Bridge Options:

Dial-On-Broadcast= N/A

Ethernet Addr Timeout(min)= N/A

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Query should be ‘No’.

- The Rem LAN Net # field should be set to the number that Cisco is using on LAN. This number can be seen in your Cisco router.    In our example, this value is ‘23456789’.

- W/D Spoofing can be set to a greater value, so if your ISDN connection drops for an extended period of time, the DI-300(M) will still retain the proper SAP and RIP tables.

Menu 13:

Menu 13 - Default Dial-in Setup

Telco Options:                       IP Address Supplied By:

CLID Authen= None                    Dial-in User= Yes

IP Pool= No

PPP Options:                             IP Start Addr= N/A

Recv Authen= CHAP/PAP                  IP Count(1,2)= N/A

Compression= Yes

Mutual Authen= Yes                 IPX Net Num Supplied By:

PAP Login=                         IPX Pool= No

PAP Password= ********               IPX Start Net Num= N/A

Multiple Link Options:                 IPX Count(2,16)= N/A

Max Trans Rate(Kbps)= 128

Session Options:

Callback Budget Management:            Input Filter Sets=

Allocated Budget(min)= 0             Output Filter Sets=

Period(hr)= 0                        Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

	Mutual Authen should be ‘Yes’.

	The PAP login/Password fields are the name and password that DI-300(M) will send to Cisco when Cisco calls into DI-300(M) and use PAP as authentication. In this example, the name is DI-300(M) and password is hidden.

III.	Cisco 2503 Setup

Following is a capture of 'wr t' command. Please note those the fields are indicated in bold type.

####

Current configuration:

!

version 10.2

service password-encryption

!

hostname csco2503

!

enable password 7 03085A09

!

username DI-300(M) password 7 08701E1D5D4C

ip subnet-zero

ipx routing

isdn switch-type basic-dms100

!

interface Ethernet0

ip address 192.68.135.175 255.255.255.0

ipx network 23456789

!

interface Serial0

no ip address

shutdown

!

interface Serial1

no ip address

shutdown

!

interface BRI0

ip unnumbered Ethernet0

encapsulation ppp

ipx network 22221111

dialer idle-timeout 300

dialer map ip 204.247.203.176 name DI-300(M) 5551212

dialer map ipx 22221111.0003.0004.0005 name DI-300(M) broadcast 5551212

dialer-group 1

ppp authentication pap

!

ip classless

ip route 204.247.203.0 255.255.255.0 BRI0

ip route 204.247.203.156 255.255.255.255 BRI0

ipx route 00001111 22221111.0003.0004.0005

!

ipx sap 4 1ST_SERVER 00001111.0000.0000.0001 451 2

!

dialer-list 1 protocol ip permit

dialer-list 1 protocol novell permit

!

:

:

!

end