Last modified: 4/9/96

Configuring Win95 Dial in User to DI-300(M) to use Microsoft Network

(remote network)

NT

W95

ISDN

TA or modem

DI-300(M)

W95

Special Note:

Win95 must specify it's own IP address that's within the range of the remote network when dialing in; Win95 can not use an address from the DI-300(M)'s IP pool.

Using TCP/IP

I. Setting up dial-in Win95 Network Configuration

A. Click “Start”.

B. Select “Setting”.

C. Select “Control Panel”.

D. In the “Control Panel” panel, double click “Network”.

E.    Add these components if they are not presently installed.    Click “Add...” to add.

1. “Client for Microsoft Network”

a. Double click “Client”

b. Select “Microsoft” under Manufactures

c. Double click “Client for Microsoft Network”

2. “Dial-Up Adapter”

a. Double click “Adapter”

b. Select “Microsoft” under Manufactures

c. Click “Dial-Up Adapter” under Network Adapters

3. “TCP/IP”

a. Double click “Protocol”

b. Select “Microsoft” under Manufactures

c. Double click “TCP/IP” under Network Protocols

4. “File and printer sharing for Microsoft Network”

a. Double click “Service”

b. Double click “File and printer sharing for Microsoft Network”

F. Configure components

1. "Client for Microsoft Network"

a. Pick "Logon and restore network connection" under Network logon options

2. "TCP/IP"

a. In "IP Address" Menu

i. Pick "Specify an IP Address"

ii. Choose a valid address and net mask from the remote network.    The address 				must be 	within the subnet of the remote network.

b. In "Bindings" Menu

i. Pick "Client for Microsoft Network"

ii. Pick "File and printer sharing for Microsoft Network"

G. Click "Access Control" in Network Menu

1. Choose "Share-level access control"

II. Configuring dial-in Win95 Dial-up Network

A. Click “Start”

B. Select “Programs”

C. Select “Accessories"

D. Select "Dial-up Networking"

E. Create new connection if one doesn't already exist.

1. Click "Make New Connection"

2. Follow directions

F. Configurate connection

1. Right click connection icon only once.

2. Click "Properties" in pop up menu.

3. In connection screen, click "Server type"

a. Choose Type of Dial-up Server as "PPP. W95, NT3.5, Internet"

b. Pick "Log on to network"

c. Pick "TCP/IP"

d. Click TCP/IP Setting

i. Pick "Server assigned IP address"

ii. Pick "Server assigned name server addresses"

iii. Pick "Use IP header compression"

iv. Pick "Use default gateway on remote network"

III. Setting up Win95 on remote network

A. Click “Start”.

B. Select “Setting”.

C. Select “Control Panel”.

D. In the “Control Panel” panel, double click “Network”.

E.    Add these components if they are not presently installed.    Click “Add...” to add.

1. “Client for Microsoft Network”

a. Double click “Client”

b. Select “Microsoft” under Manufactures

c. Double click “Client for Microsoft Network”

2. Network Adapter

a. Double click “Adapter”

b. Click your network adapter's manufacturer

c. Click your network adapter's name

3. “TCP/IP”

a. Double click “Protocol”

b. Select “Microsoft” under Manufactures

c. Double click “TCP/IP” under Network Protocols

4. “File and printer sharing for Microsoft Network”

a. Double click “Service”

b. Double click “File and printer sharing for Microsoft Network”

F. Configure components

1. "Client for Microsoft Network"

a. Pick "Logon and restore network connection" under Network logon options

2. "TCP/IP"

a. In "IP Address" Menu

i. Pick "Specify an IP Address"

ii. Choose a valid address and net mask from the remote network.

b. In "Binding" Menu

i. Pick "Client for Microsoft Network"

ii. Pick "File and printer sharing for Microsoft Network"

G. Click "Access Control" in Network Menu

1. Choose "Share-level access control"

IV. Configure the DI-300(M)

Menu 1 - General Setup System Name= DI300 Location= Taipei Contact Person's Name= CC Route IP= Yes Route IPX= Yes Bridge= No Press ENTER to Confirm or ESC to Cancel:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup DHCP Setup: DHCP= None Client IP Pool Starting Address= N/A Size of Clent IP Pool= N/A Primary DNS Server= N/A Secondary DNS Server= N/A TCP/IP Setup: IP Address= 192.68.135.14 IP Subnet Mask= 255.255.255.192 RIP Direction= Both Version= RIP-v1 Press ENTER to Confirm or ESC to Cancel:

Menu 13 - Default Dial-in Setup Telco Options: IP Address Supplied By: CLID Authen= None Dial-in User= Yes (This field must be yes) IP Pool= Yes (This field doesn't matter) PPP Options: IP Start Addr= 192.68.135.20 Recv Authen= PAP IP Count(1,2)= 2 Compression= No Mutual Authen= No IPX Net Num Supplied By: PAP Login= N/A IPX Pool= No PAP Password= N/A IPX Start Net Num= N/A Multiple Link Options: IPX Count(2,16)= N/A Max Trans Rate(Kbps)= 128 Session Options: Callback Budget Management: Input Filter Sets= Allocated Budget(min)= 0 Output Filter Sets= Period(hr)= 0 Idle Timeout= 300 Press ENTER to Confirm or ESC to Cancel:

Menu 14.1 - Edit Dial-in User User #: 3 User Name= sherwin Active= Yes Passwd= ******** Callback= No Phone # Supplied by Caller= N/A Callback Phone #= N/A Rem CLID= Idle Timeout= 300 Press ENTER to Confirm or ESC to Cancel: