Last modified: June 12, 1997

Bridging IPX Between Two DI-300(M)

NetWare

Server

NetWare

Client

DI-300(M) 1

DI-300(M) 2

Note: The IPX network numbers shown above are examples only!

This configuration note explains how to set up a workstation on a network to use a DI-300(M) to make a connection to another DI-300(M) to bridge IPX.    In this configuration, the workstation has NetWare client software so that you will be able to log into the NetWare server on the remote network.    There will be three items that you need to set up.    These are the workstation and the two DI-300(M).

I.	Workstation Setup (NetWare Client)

To set up the workstation (W/S), you will need to set the following parameters:

- IPX Frame Type - First determine the frame type that the NetWare server that you wish to login to is using, and set the same frame type on the NetWare client workstation.

- Server Name - In some cases, you may need the name that has been configured for the NetWare server you wish to login to.    In the previous example, that would be ‘Server_1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of IPX client software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- DOS workstation w/ NetWare Client (VLM) software

- Windows 95/NT LAN Setup

II.	DI-300(M) 2 Setup

In order to configure the DI-300(M) on the NetWare client side for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_2

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= No

Bridge= Yes

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Bridge field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.4:

Menu 3.4 - Bridge Ethernet Setup

Handle IPX= Client

Press ENTER to Confirm or ESC to Cancel:

- Set Handle IPX to ‘Client’.

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= My_ISP                Route= None

Active= Yes                          Bridge= Yes

Call Direction= Outgoing

Edit PPP Options= No

Incoming:                            Rem IP Addr= N/A

Rem Login=                         Edit IP/IPX/Bridge= Yes

Rem Password= ********             Telco Option:

Rem CLID= N/A                        Transfer Type= 64K

Call Back= N/A                       Allocated Budget(min)= 0

Outgoing:                              Period(hr)= 0

My Login= username                 Session Options:

My Password= ********                Input Filter Sets=

Authen= CHAP/PAP                     Output Filter Sets=

Pri Phone #= 5551214                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Outgoing’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- My Login and My Password is the login information that this DI-300(M) will use to connect to the remote router.

- The Pri Phone # field should be set to the correct phone number to dial.

·	Route should be set to ‘None’

- The Bridge field should be set to ‘Yes’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr:                         Dial-On-Query= N/A

Rem Subnet Mask= N/A                 Rem LAN Net #= N/A

My WAN Addr= N/A                     My WAN Net #= N/A

Single User Account= N/A             Hop Count= N/A

Server IP Addr= N/A                  Tick Count= N/A

Metric= N/A                          W/D Spoofing(min)= N/A

Private= N/A                         SAP/RIP Timeout(min)= N/A

RIP Direction= N/A

Version= RIP-v1                  Bridge Options:

Dial-On-Broadcast= Yes

Ethernet Addr Timeout(min)= 0

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Broadcast should be ‘Yes’.

III.	DI-300(M) 1 Setup

In order to configure the DI-300(M) on the NetWare server side, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_1

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= No

Bridge= Yes

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Bridge field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551214

SPID #= 408555121400

2nd Phone #= 5551215

SPID #= 408555121500

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.4:

Menu 3.4 - Bridge Ethernet Setup

Handle IPX= Server

Press ENTER to Confirm or ESC to Cancel:

- Set Handle IPX to ‘Server’.

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= My_user               Route= None

Active= Yes                          Bridge= Yes

Call Direction= Incoming

Edit PPP Options= No

Incoming:                            Rem IP Addr= N/A

Rem Login= username                Edit IP/IPX/Bridge= Yes

Rem Password= ********             Telco Option:

Rem CLID=                            Transfer Type= 64K

Call Back= N/A                       Allocated Budget(min)= 0

Outgoing:                              Period(hr)= 0

My Login= username                 Session Options:

My Password= ********                Input Filter Sets=

Authen= N/A                          Output Filter Sets=

Pri Phone #= N/A                     Call Filter Sets=

Sec Phone #= N/A                     Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Incoming’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- Rem Login and Rem Password is the login information that this DI-300(M) will expect from the remote router.

·	Route should be set to ‘None’

- The Bridge field should be set to ‘Yes’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr:                         Dial-On-Query= N/A

Rem Subnet Mask= N/A                 Rem LAN Net #= N/A

My WAN Addr= N/A                     My WAN Net #= N/A

Single User Account= N/A             Hop Count= N/A

Server IP Addr= N/A                Tick Count= N/A

Metric= N/A                          W/D Spoofing(min)= N/A

Private= N/A                         SAP/RIP Timeout(min)= N/A

RIP= N/A

Bridge Options:

Dial-On-Broadcast= No

Ethernet Addr Timeout(min)= 0

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Broadcast should be ‘No’.