Last modified: June 12, 1997

LAN to LAN Connection Routing IP

204.5.1.0/24	          							192.68.135.0/24                204.247.203.0/24

204.5.1.2

W/S with

TCP/IP

DI-300(M) 1

FTP

Server

DI-300(M) 2

192.68.135.1

204.5.1.1                                                                                                                                                                                                                                                                                  204.247.203.1

Note: The IP addresses shown above are examples only!

This configuration note explains how to set up a workstation on one network to use a DI-300(M) to make a connection to another DI-300(M) to route IP.    In this configuration, the workstation has the TCP/IP software installed so that it will be able to run any applications with stations on the remote network (e.g., FTP, telnet, etc.).    There will be three items that you need to set up.    These are the workstation and the two DI-300(M)s.

I.	Workstation Setup

To set up the workstation (W/S), you will need to set the following parameters:

- IP Address - the IP address assigned to the workstation itself, in the above example, ‘204.5.1.1’

- Subnet Mask - the subnet mask used for your network.    Class C networks generally use a 24-bit netmask, ‘255.255.255.0’.

The procedure for configuring these parameters for your workstation may differ depending on the type of TCP/IP networking software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- Trumpet LAN Setup - Setting up Trumpet TCP/IP

- Chameleon LAN Setup - Setting up Chameleon TCP/IP

- Windows 95/NT TCP/IP LAN Setup

- MacTCP Setup

II.	DI-300(M) 2 Setup

In order to configure DI-300(M) 2 for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_2

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= No

Bridge= Yes

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 204.5.1.2

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-v1

Press ENTER to Confirm or ESC to Cancel:

- In menu 3.2,    the IP Address and IP Subnet Mask field must be set correctly.    In our example, the IP address of this DI-300(M) should be set to ‘204.5.1.2’.

- The RIP Direction field must be set to either ‘Both’ or ‘Out’ in order for your workstation to receive the route to the remote network (destination 192.68.135.0/24, gateway, 204.5.1.2).    If this field is not set in this way, you will need to create a static route in your workstation or another router that sends this RIP information.

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= IP_user               Route= IP

Active= Yes                          Bridge= No

Call Direction= Outgoing

Edit PPP Options= No

Incoming:                            Rem IP Addr= 192.68.135.1

Rem Login=                         Edit IP/IPX/Bridge= Yes

Rem Password= ********             Telco Option:

Rem CLID= N/A                        Transfer Type= 64K

Call Back= N/A                       Allocated Budget(min)= 0

Outgoing:                              Period(hr)= 0

My Login= username                 Session Options:

My Password= ********                Input Filter Sets=

Authen= CHAP/PAP                     Output Filter Sets=

Pri Phone #= 5551214                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Outgoing’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- My Login and My Password is the login information that this DI-300(M) will use to connect to the remote router.

- The Pri Phone # field should be set to the correct phone number to dial.

·	Route should be set to ‘IP’.

- Set Rem IP Addr equal to the IP address of the remote router.    In our example, this would be ‘192.68.135.1’.

Menu 12.1 (optional):

Menu 12.1 - Edit IP Static Route

Route #: 1

Route Name= To_FTP

Active= Yes

Destination IP Address= 204.247.203.1

IP Subnet Mask= 255.255.255.0

Gateway IP Address= 204.5.1.1

Metric= 2

Private= No

Press ENTER to Confirm or ESC to Cancel:

If you need to set up a static route to reach a station or device that is not directly connected to the remote network you are accessing, you can configure menu 12.    In our example, let’s say the workstation wants to log into the FTP server on the 204.247.203.0/24 network.    He will need to create a static route to access this device.

- In menu 12, make sure to set the Active field to ‘Yes’.

- Set the Destination IP Address field to the IP address of the device/network you want to access.    In our example, this would be ‘204.247.203.1’.

- Set the Gateway IP Address field to the first hop in which to route your packets to the desired destination address; in this case, it would be ‘192.68.135.1’.

III.	DI-300(M) 1 Setup

In order to configure DI-300(M) 1 for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300_1

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= N/A

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 1234567

SPID #= 408123456700

2nd Phone #= 1234568

SPID #= 408123456800

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 192.68.135.1

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-v1

Press ENTER to Confirm or ESC to Cancel:

- In menu 3.2,    the IP Address and IP Subnet Mask field must be set correctly.    In our example, the IP address of this DI-300(M) should be set to ‘192.68.135.1’.

- The RIP Direction field must be set to either ‘Both’ or ‘Out’ in order for your workstation to receive the route to the remote network (destination 204.5.1.0/24, gateway, 192.68.135.1).    If this field is not set in this way, you will need to create a static route in your workstation or another router that sends this RIP information.

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= IP_user               Route= IP

Active= Yes                          Bridge= No

Call Direction= Incoming

Edit PPP Options= No

Incoming:                            Rem IP Addr= 204.5.1.1

Rem Login= username                Edit IP/IPX/Bridge= Yes

Rem Password= ******               Telco Option:

Rem CLID=                            Transfer Type= 64K

Call Back= N/A                       Allocated Budget(min)= 0

Outgoing:                              Period(hr)= 0

My Login= usermame                 Session Options:

My Password= ********                Input Filter Sets=

Authen= N/A                          Output Filter Sets=

Pri Phone #= N/A                     Call Filter Sets=

Sec Phone #= N/A                     Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- The Call Direction should be set to ‘Incoming’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- Rem Login and Rem Password is the login information that this DI-300(M) will expect from the remote router.

·	Route should be set to ‘IP’.

- Set Rem IP Addr equal to the IP address of the remote router.    In our example, this would be DI-300(M) 2 which is ‘204.5.1.2’.

Menu 13:

Menu 13 - Default Dial-in Setup

Telco Options:                       IP Address Supplied By:

CLID Authen= None                    Dial-in User= Yes

IP Pool= No

PPP Options:                             IP Start Addr= N/A

Recv Authen= PAP/CHAP              IP Count(1,2)= N/A

Compression= No

Mutual Authen= No                  IPX Net Num Supplied By:

PAP Login= N/A                     IPX Pool= No

PAP Password= N/A                    IPX Start Net Num= N/A

Multiple Link Options:                 IPX Count(2,16)= N/A

Max Trans Rate(Kbps)= 128

Session Options:

Callback Budget Management:            Input Filter Sets=

Allocated Budget(min)= 0             Output Filter Sets=

Period(hr)= 0                        Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- Set the Recv Authen field to the type of authentication you want use (CHAP, PAP, or none).