Filtro Provider DI-300 (già presenti filtri Netbios)

A)Selezionate il menù 21-Filter Set Configuration

B)Selezionate poi  il primo punto libero (in questo caso il punto 3) e assegnate un nome (Filter)

Menu 21 - Filter Set Configuration

Filter                                              Filter

Set #        Comments                     Set #        Comments

------  -----------------                      ------  -----------------

1      NetBIOS#1                          7      _______________

2      NetBIOS#2                          8      _______________

3      Filter                                    9      _______________

4      _______________              10      _______________

5      _______________              11      _______________

6      _______________              12      _______________

# Enter Filter Set Number to Configure= 3

Edit Comments= Filter

Press ENTER to Confirm or ESC to Cancel:

C)Inserite ora  il seguente filtro:

Menu 21.3.1 - TCP/IP Filter Rule

Filter #: 3,1

Filter Type= TCP/IP Filter Rule

Active= Yes

IP Protocol= 1     IP Source Route= No

Destination: IP Addr= 0.0.0.0

IP Mask= 0.0.0.0

Port #= 0

Port # Comp= None

Source: IP Addr= 0.0.0.0

IP Mask= 0.0.0.0

Port #= 0

Port # Comp= None

TCP Estab= N/A

More= No           Log= None

Action Matched= Drop

Action Not Matched= Forward

Press ENTER to Confirm or ESC to Cancel:

Menu 21.3 - Filter Rules Summary

# A Type                       Filter Rules                              M m n

- - ---- --------------------------------------------------------------- - - -

1 Y IP   Pr=1, SA=0.0.0.0, DA=0.0.0.0                                    N D F

2 N

3 N

4 N

5 N

6 N

Enter Filter Rule Number (1-6) to Configure:

D)Infine , per attivare il filtro, inserite in Output Filter Sets il numero 3 relativo al numero di Filter set ( nel menù 11.1 relativo alla connessione)

Menu 11.1 - Remote Node Profile

Rem Node Name= INET                  Edit PPP Options= No

Active= Yes                          Rem IP Addr= 0.0.0.0

Call Direction= Outgoing             Edit IP= No

Incoming:                            Telco Option:

Rem Login=                           Transfer Type= 64K

Rem Password= ********               Allocated Budget(min)= 0

Rem CLID= N/A                        Period(hr)= 0

Call Back= N/A

Outgoing:                            Session Options:

My Login= dlink                      Input Filter Sets=

My Password= ********                Output Filter Sets= 3

Authen= CHAP/PAP                     Call Filter Sets= 1

Pri Phone #= 0240918504              Idle Timeout(sec)= 60

Sec Phone #=

###### Press ENTER to Confirm or ESC to Cancel: