Silicon Image SuSE 9.0 Series Driver Installation

(SATA 3114 Controller)

# OVERVIEW

This driver disk adds support for the SiI 3114 SATA controller and overrides the Open Source Driver. The installation is NOT trivial since it requires unloading the default Open Source Driver already contained in the CD-ROM.

# CREATE FLOPPY before INSTALLATION

1) The easiest way to create a floppy containing the drivers is to us
   “dd” as follows:

dd if=file_name.img of=/dev/fd0  (In a Linux System)
      where file_name will change with the distribution used.

2) Configure RAID SET using the controller BIOS if you want RAID
   functionality

3) The following BIOS BOOT sequence is recommended => CD, Hard Disk,
   Floppy

# INSTALL THE SuSE DRIVER

## Phase 1

By default SuSE wants to boot from the Hard Disk.  At Boot Time manual intervention is required to choose Installation.  See figure 1.

For the 3114 series controller DO NOT insert the floppy disk and DO NOT type F6 at this time. Let the system continue installation until you get the first YAST screen. You  can also wait until the second screen shown in figure 2.

FIGURE 1.

FIGURE 2.

Now you need to MOUNT the Floppy Drive and run a script.  In order to do that you need to get a console window. This is done by typing CONTROL-ALT-F2 all 3 keys at the same time. To go back to the graphical interface type CONTROL-ALT-F1 or F7.

The script is executed as follows:

# mkdir f

# mount /dev/fd0 /f 
# cd /f

# ./initial_install.sh

(sh ./initial_install should also work)

“initial_install” bumps the open source driver and install this driver. However, this driver is the default driver and is not the final driver.

Now continue with the installation.  At some point the system will AUTOMATICALLY reboot.  Unfortunately if you do not catch the reboot the system will load the Open Source Driver on the second reboot.  In any case on the second reboot you MUST repeat the same steps described above.  So the initial_install.sh script must be executed twice.  (Note: on the second reboot you must have CD #1 ready and NO floppy like the first time. However, the second time figure # 3 will appear as shown on Phase 2.

Phase 2

On the second reboot you must choose “Boot Installed System” since the system software is already partially on the hard disk.                      .

FIGURE 3.

This time complete installing the software until you get to the “finish” screen or you get to login as shown in figure 4. Last, you still need to run one final script.  Get a console window as before and execute the following:

# mkdir /tmp/f ; mount /dev/fd0 /tmp/f

# cd /tmp/f; ./upgrade_driver.sh

("sh ./upgrade_driver.sh" if minimum install)

At this point you are done. You can now reboot the system for normal operation.

FIGURE 4.