DOCUMENTATION DATABASE

DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 2

PRODUCT    : PC

MODEL      : M4-40,M4-60

TYPE       : TIB

DATE BEGIN : 06/30/94

PAGES      : 3

TITLE      : HDU, FDU EVOLUTION NEWS SIMM FOR M4-4X/6X PCS.

HDU AND FDU UPGRADING

ALTERNATIVE SIMM's

STARTER KIT AND SYSTEM REGION UPGRADING

ORGANIZATION

SIMM MEMORY

Alternative SIMMs  1Mx36 70 ns Quad Cas code  4897691 K, and 2Mx36 70ns code 4897690 J are being introduced.

MFDU

The MFDU YD-702B  - 6037B  drives  from supplier  Y-E Data,  previous  to December 93 were furnished with an isolated mounting staff which did  not guarantee electrical contact with the drive  casing.  The drive code  has been updated to level 01.

HDU

The AC1170  drive supplied  by  Western Digital,  from  November  93  has changed buffer size from 64KB to 32 KB.Its code changes tolevel 02.

The AC2250 drive supplied by Western Digital, from March 94 has changed Firmware from A7 to A9 to implement new FAST IDE functionality.

CFA170A and CFA340A  drives supplied  by Conner,  from November  93  have changed niwot code from rev. 1.67 to 1.70.

Due to a  compatibility problem  found on Pike1  PCs, the  CFA340A  drive supplied by  Conner has  introduced  a new  printed circuit  board  level 1380-001.

The CFA540A drive supplied by CONNER has  introduced a new version of  FW 8FTO54 for functional improvements; and a new version of the data channel chip, causing an evolution to 09570-003 of the board code.

In the 560455E code groupings,  the CFA540A drive was mistakenly  entered with code 560456G, the correct code is 558516W.

STARTER KIT

For the changes  implemented in  some of  the SW  shown below,  read  the information included on the diskettes.

EVD the latest release issued is 1.06

USER DISK

Code 2690577Y has been changed  in time from rel 1.01  to rel. 1.02  code 2690615L, then to 1.04 update 1 Code 2690664M (Release 1.04 requires BIOS version 2.06) for modifications to the following test:

Memory Check presence, Floppy (modified to optimize the handling of  the 1.2 MB unit), Keyboard to correctly  recognize the type of controller  as PS/2 or AT, Mouse,  CPU modified  the cache test, HDU configuration for high capacity, SETUP updated presettings for SHADOW, Video introduced new monitor type, the logo for the M4-60/65 introduced.

And finally rel. 1.05 code 2690787 D, where the modified tests are:

Keyboard, Interrupt subtest; Setup, introduction of the multisector RD/WR functions on the HDU.

ON-LINE DOCUMENTATION

Change with the same releases and for the same reasons as the

USER DISK

SYSTEM REGION SET-UP

Trota 1

The first rel. supplied was 1.04, the evolution to 1.06 was made for  the same reasons described for the  USER DISK rel. 1.04;  the latest  release issued is 1.07,  DIR/DOC code:  disk  1, H06038;  disk 2,  H06039; which implement the same modifications as USER DISK rel 1.05.

Trota 1.5

The first rel. supplied was 1.01, subsequently release 1.02 was issued to update the  HDU  test,  cache  and  monitor  utilities;  and  the latest distribution is rel.  1.03, DIR/DOC  code: disk 1,  H06040; and  disk  2, H06041; to implement the same modifications as USER DISK rel 1.05.