DOCUMENTATION DATABASE

DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 501/B

PRODUCT    : PC

MODEL      : M4-40,M4-46,M4-60,M4-62,M4-64,M4-65,M4-66

TYPE       : FCO

DATE BEGIN : 05/13/94

PAGES      :

TITLE      : MB TROUT 1 TROUT 1.5 HW EVOLUTON AND LEVELS.

FOR PROBLEM DESCRIPTIONS SEE THE SPECIFIC SECTIONS

ORGANIZATION

During the production period from February to  April 94, the trota 1  and trota 1.5 boards have undergone various modifications which result in the evolution of their hardware revision levels.

Following is a list of the modifications and the corresponding  revision levels.

It should be noted that the HW level of the trota 1 board will follow one of two different evolutions, depending on whether or not the modification introduced to correctly handle OS/2 version 1.3 was implemented.

Also note that the trota 1.5 boards have been produced on two  different motherboards, for which there have been two different evolutions with the motherboards undergoing the same modifications but  assigned different revision levels.

Evolution of trota 1.5 and  rota 1 boards which have implemented modifications for OS/2 ver. 1.3

Motherboards:

- BA2039 and BA2034 change to revision level 11

- BA2032 changes to revision level 10

- BA2050/BA2051/BA2068/BA2063/BA2064/BA2076/BA2077/BA2078/BA2079

Change to level 02

A change in  the values  of 4 resistors and  3 capacitors necessary  to re-apdapt the  impedence on the RGB  signals, thus recalibrating the

circuit that generates the reference current  to the DAC, making the  RGB signal edges sharper, and  improving the compensation  between point  and horizontal line.

NOTE: The change to revision level 2 was announced in FCO 502 for

BA2050/BA2051/BA2068/BA2063/BA2064 boards corresponding to   the introduction of BIOS rev. 2.03. The modification above was made in

time to incorporate this change on the boards simultaneously  with the BIOS change.

The introduction of BIOS version 2.04 corresponds to revision level

changes as follows:

BA2034 and BA2039 go from level 11 to level 12

BA2032 goes from level 10 to level 11

BA2076/BA2077/BA2078/BA2079 go from level 02 to level 03

This BIOS version is available from DRS with function name PD2R and  code 588794 U.

The following boards:

BA2034 and BA2039 go from level 12 to level 13

BA2032 goes from level 11 to level 12

BA2076/BA2077/BA2078/BA2079 go from level 04 to level 05

BA2050/BA2051/BA2064/BA2063/BA2068 go from level 03 to level 04

Due to:

- the introduction of BIOS version 2.06

- diodes in positions  D4 and  D5 must be  inverted, that  is diode  code 4800056 A mounted in position D4, must be mounted in pos. D5, and  vice versa the Schottky  diode code  4800067F  mounted in  pos. D5  must  be mounted in pos. D4

-diode D3 should be substituted with a 340ohm resistor code 4901569U,

mounted between pin 3 and pins 1 or 2 (which are connected on the

printed circuit board)

This BIOS version is available from DRS with function name PDK0 and  code 588815 R.

These boards:

- BA2050/BA2051/BA2064/BA2063/BA2068 go to revision level 03

- BA2076/BA2077/BA2078/BA2079 go to level 04

with the accomplishment of one PCB track cut and one wire splice to allow correct operation with an LPU board.

These boards:

BA2034 and BA2039 go from revision level 13 to level 14

BA2032 goes from level 12 to level 13

BA2050/BA2051/BA2064/BA2063/BA2068 go from level 04 to level 05

BA2076/BA2077/BA2078/BA2079 go from level 05 to level 06.

With the introduction of:

One 470 ohm axial resistor code 4900673C

2 track cuts

3 wire splices (for TROTA 1 boards)

4 wire splices (for TROTA 1.5 boards)

BA2076/BA2077/BA2078/BA2079 go from revision level 06 to level 07.

Due to wiring optimizations on  the printed circuit board that  eliminate

the cut and wire splice that  were introduced due to incorrect  operation

with the LPU board.

As a result of this  the printed  circuit board code  654199H changes  to

level 01

On boards:

- BA2034/BA2063/BA2064/BA2078/BA2079 the Intel microprocessor 486SX-25MHz

SL Enhanced, code 4893098 M., function  name PQFP, is introduced as  an

alternative CPU.

- BA2032/BA2039/BA2068 the Intel microprocessor  486SX-33MHz SL  Enhanced PGA version, code 4893092 T,  function name CSEG, is  introduced as an alternative CPU.

Note that BIOS Rel. 2.06 is required for correct identification of the SL Enhanced CPUs by the POD.

Trota 1 boards which have not implemented the modification for OS/2  ver. 1.3

BA2039 and BA2034 go to  level 10/A; BA2032  goes to level  9/A (for  the same reason BA2034/BA2039 are level 11 and BA2032 level 10).

BA2039 and BA2034 go to level 11/A; BA2032 goes to level 10/A (as  BA2034 and BA2039 level 14 and BA2032 level 13)

BA2039 and BA2034 go to level 12/A; BA2032 assumes level 11/A when:

- BIOS Rel. 1.13 is installed, available  from DRS with code 588827M  and function name PDK9

- diodes D4 and D5 are inverted, that is diode code 4800056 A mounted  in position D4 must be mounted  in pos.  D5, and vice  versa the  Schottky diode code 4800067F mounted in pos.D5 must be mounted in pos.D4

- the diode in position D3 is to be  substituted with a 340 ohm  resistor code 4901569U,  mounted between  pin  3  and  pin 1  or  2  (which  are connected on the circuit board)

The Intel microprocessor  80486SX 25  or 33MHz SL-Enhanced,can also  be used on these  boards on the  condition that  the BIOS is  updated to  at least rel. 1.13.

DESCRIPTION

Trota 1.5 and trota 1 boards where the modification for OS/2 ver. 1.3 has been impemented

These boards:

- BA2034/BA2039 modified to level 11

- BA2032 modified to level 10

- BA2050/ BA2051/ BA2068/ BA2063/ BA2064/ BA2076/ BA2077/ BA2078/  BA2079 modified to level 02

Have integrated the solution to two anomalies:

The first, that  horizontal lines  had greater luminosity  than  vertical lines, due to inaccurate compensation  between line modulation and point modulation.

The second, appeared in  the form  of a comet-like  effect, when  working with  small  font  and  1024  x  768  resolution  with  72  Hz   vertical synchronism.

These boards:

- BA2034/BA2039 level 12

- BA2032 level 11

- BA2076/BA2077/BA2078/BA2079 level 03

BIOS 2.04 was introduced which resolves the following problems:

- Allows correct display  of the  CPU type and  frequency in  the  banner during the POD phase.

- In 132 X 43 video mode the font was not displayed correctly, leaving a line of undriven pixels between one line and the next.

NOTE: This BIOS version does not permit the use of SL-Enhanced  CPUs 		even if they are recognized correctly in the POD banner.

These boards:

BA2034/BA2039 level 13

BA2032 level 12

BA2050/BA2051/BA2068/BA2063/BA2064 level 04

BA2076/BA2077/BA2078/BA2079 level 05

Have introduced BIOS Rel. 2.06 which corrects the following problems:

- An OVERSCAN problem in 640 X 480 video mode at 72 Hz, detected by video diagnostic tests, for which the lower side  of the the framing did  not appear.

- Correct handling of the Solitaire game of Windows 3.1, which caused the PC to lock-up at game termination.

- Support for an optional feature:  MULTI-SECTOR RD/WR during the   phase of HDU access.  (Configurable using the set-up utility included in  the system region)

- Correction of  programming  values  in  the FIFO  queue  of  the  video controller for  the  various  video  modes  for  the  vertical  refresh function.

The inversion of the diodes, and the introduction of the resistor,  serve to increase the margin of the VBATT  for the CMOS in stand-by,  resolving various problems of incorrect messages  of: CHECKSUM ERROR, BATTERY  LOW, CMOS ERROR that appeared during the POD phase.

These boards:

- BA2050/ BA2051/ BA2068/ BA2063/ BA2064 level 03

- BA2076/ BA2077/ BA2078/ BA2079 level 04

Have introduced the modification that  permits identification of the  LPU board and relative  SW, thus  allowing it  to work  correctly in  the PB environment.

These boards:

BA2034 and BA2039 level 14

BA2032 level 13

BA2050/ BA2051/ BA2068/ BA2063/ BA2064 level 05

BA2076/ BA2077/ BA2078/ BA2079 of level 06.

Correct a problem that would not  permit the boot-strap from floppy  when boards manufactured by WANG,COGENT and COFAX KF 9210, are present on the bus.

Trota 1 boards which have not implemented the modification for OS/2  ver. 1.3

BA2034/ BA2039 level 10/A and BA2032 level 9/A (like BA2034/ BA2039 level 11 and BA2032 level 10)

BA2034/ BA2039 level  11/A and  BA2032 level  10/A (like  BA2034/  BA2039 level 14 and BA2032 level 13)

BA2034/ BA2039 level 12/A and BA2032 level 11/A

The BIOS rel. 1.13 solves:

- An OVERSCAN problem in 640 X 480 video mode at 72 Hz, detected by video diagnostic tests, for which the lower side  of the the framing did  not appear.

- New Intel CPUs have  a different identification  from previous  models, therefore previous BIOS versions were not able to display CPU type and frequency correctly in the banner during the POD phase.

- In 132 X 43 video mode the font was not displayed correctly, leaving a line of undriven pixels between one line and the next.

- Correct handling of the Solitaire game of Windows 3.1, which caused the PC to lock-up at game termination.

- Support for an optional feature:  MULTI-SECTOR RD/WR during the   phase of HDU access. (Configurable using  the set-up utility included in  the "system region")

- Correction of  programming  values  in  the FIFO  queue  of  the  video controller for  the  various  video  modes  for  the  vertical  refresh function.

The inversion of the diodes, and the introduction of the resistor,  serve to increase the margin of the VBATT  for the CMOS in stand-by,  resolving various problems of incorrect messages  of: CHECKSUM ERROR, BATTERY  LOW, CMOS ERROR that appeared during the POD phase.

IMPLEMENTATION

BA2034/ 2039 levels 10 and 9/A

BA2032 levels 9 and 8/A

BA2050/ BA2051/ BA2068/  BA2063/ BA2064/ BA2076/  BA2077/ BA2078/  BA2079 level 01

Go to the next revision level substituting:

- In  positions  R144,R145,R146,  the  150  ohm  resistors  with  75  ohm resistors, code 4901533F.

- In position R118 the 360  ohm resistor  with a 270  ohm resistor,  code 4901561S

- In positions C265,266,267, the 68 pF capacitors with 15 pF  capacitors, code 5071713E.

BA2050/ BA2051/ BA2068/ BA2063/ BA2064 level 02

BA2076/ BA2077/ BA2078/ BA2079 level 03

Change to  the next  revision  level after  execution  of  the  following changes with reference to attachment B:

- A cut in the track of signal ETEQCLK1 on pin 1 of the component mounted in position U75 see attachment  B for references

- A wire splice between pin  9 of the component mounted  in pos. U50  and pin 1 of the component mounted in pos. U75

NOTE:For correct operation with LPU close jumper J47 in position 1-2.

On boards:

BA2034/ 2039 level 13 and 10/A and BA2032 level 12 and 9/A for  upgrading to successive revision levels, perform the following operations observing attachment A for references:

Side B

- mount a 470  ohm resistor between  pin 3  and pin 20  of the  component mounted in pos. U36

- cut the PCB track that connects to pin 19 of U36 (signal name VCC) [see reference C]

- add a wire between pin 19 of U36 and pin 3 of J8 (signal name AEN)

-  "  "  "     "      "  17 of U36 and  "  3 of U3

-  "  "  "     "      "  3 of  U36 and  "  31 of U31

Side A

- Cut the PCB track that connects to pin 56 of J8 (signal name IOCS16*)

On boards:

BA2050/ BA2051/ BA2068/ BA2063/ BA2064 of level 04

BA2076/ BA2077/ BA2078/ BA2079 of level 05

for upgrading  to  successive  revision  levels,  perform  the  following operations observing attachment B for references:

Side B

- mount a 470  ohm resistor between  pin 9  and pin 20  of the  component mounted in pos. U36

- raise pin 19 of the component mounted in position U36

- add a wire between pin 19 of U36 and pin 3 of J8 (signal name AEN)

-  "  "  "     "      "  11 of U36 and  "  56 of J8

-  "  "  "     "      "  31 of U31 and pin 9 of U36

Side A

- Cut the PCB track connected to pin 56 of J8 (signal name IOCS16*)

- Add a wire splice between pin 9 of U50 and pin 1 of U75

BA2034/2039 level 11 and BA2032 level 10

Pass to the next  revision level  substituting BIOS rel. 2.03 with  rel. 2.04

These boards:

BA2034 and BA2039 go from level 12 to level 13

BA2032 goes from level 11 to level 12

BA2076/ BA2077/ BA2078/ BA2079 go from level 04 to level 05

BA2050/ BA2051/ BA2068/ BA2063/ BA2064 go from level 03 to level 04

- Substituting BIOS version 2.04 with version 2.06

- Inverting the positions of diodes D4 and D5

- Substituting diode D3  with a  340 ohm resistor,  mounted in  the  same position and connected between pin 3 and either of pins 1 or 2 (because these pins are inter-connected on the printed circuit board)

BA2039 and BA2034 pass from revision level 11/A to level 12/A;

BA2032 passes from level 10/A to level 11/A

- Substituting BIOS version 1.10 with version 1.13

- Inverting the positions of diodes D4 and D5

- Substituting diode D3  with a  340 ohm resistor,  mounted in  the  same position and connected between pin 3 and either of pins 1 or 2 (because these pins are inter-connected on the printed circuit board)