| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D.Mineo |  |  | A.Niosi |  |  |  |  |  |  |  | Jul-97 |  |  |  |  | 2800287K |  |  |  |  | 24 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| PC-PORTABLE |  |  | ECHOS PRO 133/150/166 |  |  |  |  |  |  |  | BIOS |  |  |  |  | Mother Board |  |  |  |  |  |  |  |
| titolo / subject ––––– BIOS rel. 1.00.31 ––––– ––––– ––––– ––––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other ––––– |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

# DESCRIPTION

This document report information about the  new BIOS rel.1.00.31 for Echos Pro line.

This new BIOS release, is splitted in two versions:

1.00.31.1 (All models except ECHOS PRO 150D)

1.00.31.2 (ECHOS PRO 150D only)

BIOS 1.00.31  fixes the following problems:

- Freeze fails in battery low when the beep warning

- The hard disk detection fail after restart from WinNT when the IBM 2GB HDD and Toshiba-CDROM are installed.

- Suspend is performed even with AC-Adapter .

- Ghost problem: 2,4,6,8.

- Only 1.00.30.2 the feature of “ Maximum battery Life “ is removed.

- The message doesn’t change on setup but this hasn’t any effect.

- Improves the problem that LCD sleep time-out sometime wait any key and auto wake-up.

- Fixed the problem that suspend hotkey is not active when APM set to OFF.

- Fixed for system halt problem during resume from save-to-disk.

(Only for 150D, found on bios 1.00.30.2)

# Known Problems:

- Sometimes, if battery is very low, after last freeze it’s not possible to boot the system even after inserting AC Adapter: the machine turns ON LCM but LCD remains dark and soon afterward system turn off by itself.  It needs to wait a few minutes or remove battery from inside the machine.

- Sometimes system doesn’t enter LCD sleep by time out. This problem has been improved with this bios release but not completely fixed. I noticed this problem in WfW, too.

- System performs suspend even with AC adapter.

This problem has been improved but not completely fixed; following situation still has the problem: in WfW, after bootstrap, leaving system idle it enters suspend even with AC adapter.

- LCD sleep and suspend time out do not correspond to the system setup value:

LCD Sleep is done after 8min. instead of 4 and suspend is done after 16min instead of 9 (5+4 of LCD sleep).

This problem was present already in the previous bios but it was less evident because of shorter default time out set.

- Default values have been modified on “Normal” Power settings while on “Max. battery life” they are still 1min HDU sleep and 2min. LCD sleep.

- Nationalized System setup messages are not correct (present in older bios, too).

PROCEDURE

Before carrying out the updating operation it is important: to have the machine with the power supply connected, to avoid possible accidental turn off due to the batteries running out during the writing phase of the flash EPROM.

To upgrade the BIOS follow these instructions:

- Boot the PC from the diskette.

- At the A:> type :

- "f199 90x_100.311 (for all ECHOS PRO models except ECHOS PRO150D)".

- or

"f199 90x_100.312 (for  ECHOS PRO 150D only)".

NOTE: If the resident BIOS of your PC is the 1.009 Version (present only on EchosPro133E and EchosPRO133S), when you upgrade it, after restarting the pc hangs showing the  following message:

"Press F1 to resume or press F2 to enter setup";

It is necessary to enter in Setup; now a new message like "All C-MOS data must be re-entered" appears.

This message disappear after press ENTER key, choose in SetUP the EXIT Field than choose "Save settings and exit (firts voice)".

The sistem will restart again without problem.

Be clear that this happen only the first time you upgrade the Bios 1.00.9 version.