| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V.Aragno |  |  | M.Merlone |  |  |  |  |  |  |  | Dec-96 |  |  |  |  | 2800267H |  |  |  |  | 03 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| SNX-Netstrada |  |  | Netstrada 5000 |  |  |  |  |  |  |  | ––––– |  |  |  |  | ––––– |  |  |  |  |  |  |  |
| titolo / subject Technical informations about Netstrada 5000 System ––––– ––––– ––––– ––––– ––––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other General Informations |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

Please, find attached technical informations regarding the Netstrada 5000 system.

A detailed description and the limitations/warning present at the start-up phase are also reported.

This document is issued only in English language.

# Table of contents

1. Table of contents

2. FOREWORD

3. System Overview

3.1. Motherboard Architecture

3.1.1. Block Diagram

3.2. CPU Board

3.2.1. PPRO processor

3.2.2. 82440FX chip set.

3.2.3. System memory.

3.3. Mechanical Structure

3.4. Resilience and Server Management

3.5. Power Supply

4. SYSTEM  BOARDS

4.1. SYSTEM CORE

4.2. I/O SUBSYSTEM

5. Bios / User Disk / System test

6. STARTER KIT

7. PDG RANDOM CODE

8. CONFIGURATION FOR RELEASE

8.1. SCSI configuration

8.2. Tested Network Card

9. Warnings & Limitations

# FOREWORD

This document is aimed at describing the status of Hardware, Software, Bios, User Disk and Diagnostic.

The  system is released with the following operating systems:

- NT3.51;  NT 4.0

- Novel Netware 3.12; 4.11

- IBM OS/2 Warp Server

- IBM OS/2 2.11

- SCO Open Server 5.02

- SCO Unixware 2.1

The system is released both in Floor Standing  (Box Silver) and Rack configurations.

# System Overview

The main characteristics of Netstrada 5000 are:

- Single and dual Pentium Pro processor

- Processor speed support for 166, 180, 200 and 233 MHz. Only 200 MHz part will be certified .

- 82440FX “Natoma” chipset

- separate 32 bit address/ 64 bit data path

- Eight single module ECC DRAM banks hosted into eight 3.3 V DIMM sockets supporting both FPM (Fast page Mode) and EDO (Extended Data Output) memory protocols

- Memory addressing capacity up to 1 GB

- Up to 9 PCI devices rev. 2.1 compliant including:

- Wide / narrow SCSI channel (AIC7870) on motherboard to connect removable

- Trident 9440-1D video controller on motherboard

- PCI to PCI bridge

- 6 PCI slots for further expansion

- EISA I/O system with 4 expansion slots

- Embedded PC-compatibility I/O support (2 serial, parallel, mouse and keyboard)

- Server management features, including thermal/voltage monitoring and error handling supported by Server View

- System Management Module (SMM-Hobbes) connector

- Flash BIOS support for all the above

The box Silver will provide the following features:

·	Mechanical dimensions: 525x360x548 mm

·	Four removable bays (front accessible, not locked):

·	one 3” ½ 1.44 MB minifloppy bay

·	three removable 5.25”HH (for STU, CD-ROM, DAT). Two of them can be combined to provide one 5.25”FH

·	six 3.5” HH hot-pluggable  wide HDU bays (front accessible, key locked)

·	standard I/O interfaces (VGA, floppy, mouse, keyboard, COM1, COM2, LPT)and a SCSI connector directly accessible on system rear side

·	Two PSU bays on the rear side (second PSU for redundancy)

·	Redundant cooling with 3 + 3  internal fans

·	8 external SCSI connectors (in addition to the controller backtail connector) for large disk array connections

·	Front visible LEDs to report most relevant system status

·	Key for both system on/off switch (same key used for disk bays and electronic card cage locking)

The core logic of the system is implemented on a motherboard card and on a cpu card.

## 3.1 Motherboard Architecture

The motherboard is the main board in the NetStrada5000 system in which the CPU is plugged in and the main system functions are implemented. The motherboard handles three different busses, the primary PCI bus, the secondary PCI bus and the EISA/ISA bus.

The three busses are arranged in a fall-through fashion so that all the I/O access are directed to the primary PCI and where unclaimed accesses are further routed (fall-through) the secondary bus and, if still unclaimed, routed to EISA/ISA bus.

The three bus do not implement any parity protection scheme.

The main bus is the primary PCI that is used by the following elements:

- The CPU/MEMORY card.

- The Trident TGUI9440-1 video controller.

- The LANCE SCSI controller.

- Two general purpose PCI slots.

- The DEC 21052 PCI-to-PCI bridge.

- The INTEL 82430 EISA/ISA bridge.

The secondary PCI bus is obtained expanding the primary bus with  the DEC 21052 PCI to PCI bridge and is used by the following elements:

- The DEC 21052 PCI-to-PCI bridge.

- Four general purpose PCI slots.

The EISA/ISA bus is controlled by the INTEL 82430 EISA/ISA bridge from the primary PCI bus and is used by the following elements:

- Four general purpose EISA/ISA slots.

- The ULTRA I/O controller.

- The Am29F040 Sector Erase Flash Memory.

- The 28C64 8K8 EEPROM.

- The Automatic Server Restart registers.

- The SCSI hot-plug registers.

- Five system specific registers.

The motherboard is used also to generate all the system clocks, to sense the system temperature and the handle the system remote on/off and the system speaker.

### 3.1.1.  Block Diagram

SVGA conn.

256K

x16

RAM

256K

x16

RAM

SCSI conn.

Registers

NVRAM

FEPROM

ASR

HOT-SWAP

Printer

COM 1/2

Floppy disk

Keyboard/mouse

EISA slot 4

EISA slot 3

EISA slot 2

EISA slot 1

Ultra

I/O

controller

EISA

System

component

ESC

PCI

EISA

bridge

PCEB

Secondary PCI slot 4

Secondary PCI slot 3

Secondary PCI slot 2

Secondary PCI slot 1

DEC

PCI-PCI

bridge

SVGA

contr.

SCSI

“LANCE”

Primary PCI slot 2

Primary PCI slot 1

CPU connector

## 3.2. CPU Board

The  CPU card has been designed to handle up to two Pentium Pro microprocessors connected in bi-processor configuration in which each processor makes use of its internal secondary cache memory.

The CPU board includes:

- Up to two PPRO processor

- 82440FX (natoma) chip set

- 2 VRM modules

- 8 DIMM sockets for system memory

- GTL bus termination

- GTL voltage generator

### 3.2.1. PPRO processor

The Netstrada5000 system use up to two Pentium Pro processors running at 66MHz bus host bus frequency. Four jumpers are provided to select the internal processor frequency with a clock multiplier ranging from 2X (120 MHz core speed to 5.5x (330 MHz core frequency) so to provide support for next generation of processors.

Only the 166 and 200MHz processors are planned to be used with 256K or 512Kbyte internal secondary level cache.

Each processors is inserted in a Zero Insertion force Socket 8 and has its own VRM module compatible with Intel specs to provide future upgradability. The VRM module can make use of the processor Voltage ID pins so that it can tune its output voltage properly and there is no need to replace it for future processors (as is the case of Pentium cards).

To guarantee that the processor temperature will not rise above specified limits, a thermal sensor has been placed under each processor so that the OS can monitor the processor local temperature.

### 3.2.2. 82440FX chip set.

The host controller is implemented by the 440FX “Natoma” chip-set that consists of two components, the PCI Bridge/Memory controller (PMC) and the Data bus accelerator (DBX).

These components include the following functions:

- Support for single and dual Pentium Pro Processors at bus frequencies of up to 66MHz.

- 64-bit GTL+ based Host Bus Data interface (no bus ECC support).

- 32-bit Host Address support.

- 32-bit PCI Bus interface.

- 64/72-bit Main memory interface (with ECC support).

- Extensive Data buffering between all interfaces for high throughput and concurrent operations.

### 3.2.3. System memory.

Eight 168 pins memory connectors for 3.3V DIMM have been included that memory capacity can range from 8 Mbytes to 1 Gbytes ECC protected. The eight memory sockets must be filled in the following order: SC1, SC2, SC3, SC4, SC6, SC7, SC8 and SC9.

The DIMM module certified for Netstrada 5000 are : 1Mx72, 2Mx72, 4Mx72, 8Mx72 and 16Mx72 Fast Page Mode  symmetrical and asymmetrical addressing.

## 3.3. Mechanical Structure

Olympus can be offered in floor-standing solution and in rack mount solution

The system is hosted in the Silver box (wide body tower box), providing support for all the basic electronics and peripherals used for the fans and power supply redundancy. The chassis  is the same of that used by the SNX460RS product.

It is composed of a wide body tower, with two sections, one dedicated to the electronic boards and one separate section for mass storage bays and PSU.

It will basically offer:

·	Mechanical dimensions: 525x360x548 mm

·	Four removable bays (front accessible, not locked):

·	one 3” ½ 1.44 MB minifloppy bay

·	three removable 5.25”HH (for STU, CD-ROM, DAT). Two of them can be combined to provide one 5.25”FH

·	six 3.5” HH hot-pluggable  wide HDU bays (front accessible, key locked)

·	standard I/O interfaces (VGA, floppy, mouse, keyboard, COM1, COM2, LPT)and a SCSI connector directly accessible on system rear side

·	Two PSU bays on the rear side (second PSU for redundancy)

·	Redundant cooling with 3 + 3  internal fans

·	8 external SCSI connectors (in addition to the controller backtail connector) for large disk array connections

·	Front visible LEDs to report most relevant system status

·	Key for both system on/off switch (same key used for disk bays and electronic card cage locking)

The Olympus system  is hosted in a standard universal 19” x 9U rack-drawer, providing support for all the basic electronics and peripherals. A 24U and a 41U (1U=1,75” or 4,44 cm) tall cabinet are available to offer a complete solution.

The rack mount enclosure has two adjacent but separate sections, one dedicated to the electronic group (on the top) and the other to the peripheral unit (on the bottom ). Both the sections contains the same components as the SNX 460/RS system. The three removable units and the  floppy disk are front accessible storage devices, the 6x3.25” hot pluggable wide HDUs can be accessed  directly from the front of the cabinet by locked door protection;  while internal components can be reached by simply sliding the rack mountable drawer out of the cabinet, releasing the cover screws and then removing the cover.

## 3.4. Resilience and Server Management

Olympus is intended to be a high-availability system with a number of features to enhance its reliability and uptime.  However, the system is not intended to be fully fault-tolerant in that some failures will result in an unintended system shutdown, such as a catastrophic DRAM or controller failure,  CPU execution error, etc.

Resilience features of Olympus include :

- DRAM ECC protected

- Systema Automatic Restart

- hot-swap disk bay with raid support

- redundant fans (optional) and temperature monitoring

- redundant power supply (optional)

- An increase of reliability can be accomplished with use of external parts such as:

- Dual-host

- Uninterruptible Power Supply (UPS)

## 3.5. Power Supply

The Olympus power supply is the same as in the Olivetti Lark and Condor systems, which provides 450 watts of regulated voltages.  In addition to the normal PC-compatible voltages and Power-Good signaling, this power supply will also provide up to 27.4 amps at 3.3 volts.  An input signal from the motherboard allows the shutdown of the DC voltages;  this is useful to protect the system when fault conditions are detected, such as over temperature.

A second PSU can work in parallel with the main one to provide redundancy .

# 4. SYSTEM  BOARDS

## 4.1. SYSTEM CORE

NetStrada 5000 hardware system core is composed of the following boards:

- CPU                            UC 2007    cod. 210932W

- Motherboard                BA 2320    cod. 212812L

- VRM module               AL 2023

## 4.2. I/O SUBSYSTEM

NetStrada 5000 I/O subsystem is composed of the following boards:

- SCSI WIDE Back panel   IF 2046    cod. 210830C-00

- Swap Board                     IF 2031     cod. 562384W-02

- Terminator Board             IF 2024     cod. 562237S-00   (for duplexing configuration)

- jumper board                    IF 2022     cod. 562232V-00

- Fans board                       IF 2015    cod.  562145W-00

The power supply expansion board for redundant configuration for NetStrada 5000  in Silver Box is:

- IF 2035      cod.  562408F-00

for NetStrada 5000 in rack solution is:

- IF 2034      cod.  562407W-00

# 5. Bios / User Disk / System test

No major problem has been reported for bios, user disk, the released versions

are the following:

- Bios			1.04

- User Disk Diagnostic 1.04

- System Test	         1.04

- Configuration Utility 1.05

# 6. STARTER KIT

The starter kit code is:   214266A

It includes the following :

- CDROM Orchestra rel 1.6

- Floppy that includes Orchestra boot  and release notes

- User disk diagnostic

- Getting Started

- Last Minute Information

# 7. PDG RANDOM CODE

The PDG random code released are as follow:

- NW50200x32Y0  A4678 R  (1 PPRO, 32Mbyte Memory, UW controller,floor standing )

- NW50200x64U0  A4679 J   (1 PPRO, 64Mbyte Memory, UW raid controller, floor standing )

- NR50200x32Y0  A4680 G  (1 PPRO, 32Mbyte Memory, UW controller, Rack mount)

- NW50200R64Y0xxS  A4681 V   (2 PPRO, 64Mbyte Memory, UW  controller, floor standing, NT Exchange )

- NW50200  A4689 D ( Empty) (floor standing)

- NR50200  A4690 A ( Empty) (rack mount)

- PEM 241W/RS      A3987 P

- PEM 240W/RM    A6114 N

# 8. CONFIGURATION FOR RELEASE

- Operating System

The following O.S. have been fully certified:

- NT3.51;  NT 4.0

- Novel Netware 3.12; 4.11

- IBM OS/2 Warp Server

- IBM OS/2 2.11

- SCO Open Server 5.02

- SCO Unixware 2.1

- Bios			1.04

- User Disk Diagnostic 1.04

- System Test	         1.04

- Configuration Utility 1.05

- ORCHESTRA release 1.6

- Hardware configuration

The hardware configuration released is as follow:

Pentium PRO 200Mhz/256K cache

DIMM Modules : Fast Page Mode 1Mx72; 2Mx72 ;4Mx72 ;8Mx72 ; 16Mx72

HDU  : 2Gbyte UW  Seagate ST32171WC 7200 RPM 1”

4Gbyte UW  Seagate ST34371WC 7200 RPM 1”

Video Ram : 1Mbyte

DPT UW  for Raid configuration

SCC PCI 114UW (Katana)

CDROM  SCSI 8x Panasonic

DAT DDS2

STU 1Gbyte

CMUX  8-32 (EISA Stallion)

## 8.1. SCSI configuration

- Default : removable connected to the SCSI embedded on MB, HDU connected to Katana board (plugged in slot 9)

- Duplexing: removable connected to the SCSI embedded on MB, one set (up to three) of HDU connected to one Katana board (plugged in slot 9) , one set (up to three) connected to a second Katana board (plugged in slot 5)

- Raid : removable connected to the SCSI embedded on MB, HDU connected to the DPTUW board (plugged in slot 9)

- PEM : pheripheral expansion module can be connected only to the DPT UW board. The transfer rate must be set to 20Mbyte/sec max.

## 8.2. Tested Network Card

The table below shows the status of  the boards tested with the released OS

| Windows NT 3.51 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZX312 3C905 100Mbps | PCI 2 dev10 irq 10 DC00 PCI 1 dev 8 irq 11 DCC0 | 1.03 | v1.85 12/9/95 v1.00.04 6/26/96 | BBS: SF00510.ZIP 1/22/96 BBS: 3C90XN.EXE v1.1 9/5/96 | OK OK (1) |  |  |  |  |  |  |
| OC-2185 + OC-2315 100M | PCI 2 irq 10 PCI 2 dev 8 irq 11 ECC0 | 1.03 | v1.04 6/21/96 v1.43 1/16/96 | BBS: ETHMPPCI.EXE 6/25/96 BBS: MP-2315.EXE 1/16/96 | OK OK |  |  |  |  |  |  |
| 3C509B |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| Intel PRO10 |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| IBM 16/4 Tring ISA |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| 3C619C |  |  |  | Embedded WNT3.51 | NOK (2) |  |  |  |  |  |  |
| 3C900 |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| 3C905 |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| DE530CT |  |  |  | Embedded WNT3.51 | OK (3) |  |  |  |  |  |  |
| OC2315 |  |  |  | Embedded WNT3.51 | OK (4) |  |  |  |  |  |  |
| AT2450FT | PCI 1 |  |  | Embedded WNT3.51 | OK (5) |  |  |  |  |  |  |
| Racal PCI T2 | PCI 1 |  |  | Embedded WNT3.51 | OK (6) |  |  |  |  |  |  |
| ZX314 |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| 3C770A |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |
| OC3135 |  |  |  | Embedded WNT3.51 | OK |  |  |  |  |  |  |

| Windows NT 4.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allied Ethernet PnP ISA |  |  |  | Embedded WNT4.0 | OK |  |  |  |  |  |
| OC-2175 |  |  |  | Embedded WNT4.0 | OK (7) |  |  |  |  |  |
| Intel TokenExpr ISA |  |  |  | Embedded WNT4.0 | OK |  |  |  |  |  |
| Intel EthFlash32 |  |  |  | Embedded WNT4.0 | NOK (13) |  |  |  |  |  |
| IntelPRO100eisa |  |  |  | Embedded WNT4.0 | NOK (14) |  |  |  |  |  |
| OS/ WARP Server |  |  |  |  |  |  |  |  |  |  |
| ZX312 OC-3118 |  | 1.03 | v2.10 12/31/95 v7.42 8/13/96 | BBS: SF00510.ZIP 1/22/96 BBS: OLITOKOS.EXE 2/11/95 | TO BE DONE |  |  |  |  |  |

| ZX312 OC-3118 |  | 1.03 | v2.10 12/31/95 v7.42 8/13/96 | BBS: SF00510.ZIP 1/22/96 BBS: OLITOKOS.EXE 2/11/95 | TO BE DONE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OC2163 |  |  |  | Embedded OS/2 WARP | NOK (15) |  |  |  |  |  |
| 3C771A |  |  |  | Embedded OS/2 WARP | OK |  |  |  |  |  |
| SCO Open Server 5.02 |  |  |  |  |  |  |  |  |  |  |
| ZX312 OC-3118 | PCI 2 dev 10 irq 11 DC00 irq 9 I/O A20 dma 5 no pnp | 1.03 | v2.24 1/22/96 v1.2.0 4/26/96 | BBS: SF00510.ZIP 1/22/96 BBS: OLITOKOS.EXE12/11/95 | OK NOK (13) |  |  |  |  |  |

| 3C905 100M OC-3137 | PCI 1 dev 8 irq 14 ECC0 PCI 2 dev 11 irq 15 DCC0 | 1.03 | v1.0.0a 8/15/96 v1.2.0 4/26/96 | WWW: 90XSCO5.EXE 8/15/96 BBS: OC3842-0.EXE 4/26/96 | NOK (12) NOK (11) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OC-2325 | / | / | Not available | driver SCO | / |  |  |  |  |  |
| SCO UW 2.1 |  |  |  |  |  |  |  |  |  |  |
| ZX312 NCU9180 | PCI 2 dev11 irq 11 slot1 AT irq 5 | 1.04 | v1.09 3/26/96 / | BBS: SF00510.ZIP 1/22/96 Embedded UW nics v2.1 | OK OK |  |  |  |  |  |
| Novell NetWare 4.11 |  |  |  |  |  |  |  |  |  |  |
| OC-3137 3C900 ZX312 NE2000+ NE3200 3C590 | PCI 1 dev10 irq14 DD00 PCI 1 dev10 slot 3 irq 3 I/O 300 slot 2 irq 5 | 1.03 1.04 1.04 1.03 1.03 1.03 | v3.34 1/3/96 v1.0 6/14/96 v2.35 12/29/95 / / | BBS: OCTTOK162.EXE 1/3/96 Embedded NW 4.11/ BBS: 3C90X.EXE 9/5/96 BBS: SF00510.ZIP 1/22/96 Embedded NW 4.11 Embedded NW 4.11 Embedded NW 4.11 | OK (8) OK (9) OK (9) OK (10) OK OK OK |  |  |  |  |  |
| 3C679 |  |  |  | Embedded NW4.11 | OK |  |  |  |  |  |
| Novell NetWare 3.12 |  |  |  |  |  |  |  |  |  |  |
| NE3200 ZX312 NE2000+ | irq 5 PCI 2 irq 3, I/O 300 |  | / v2.35 12/29/95 / | Embedded NW 3.12 BBS: SF00510.ZIP 1/22/96 Embedded NW 3.12 | OK OK OK |  |  |  |  |  |

- : with board on PCI bus 2^ the diagnostic program fails in “FIFO loopback”. On 1^ PCI bus OK

- : board does not work

- : diagnostic program fails. OS loading OK

- : the board works only on PCI bus 1^

- : the board works only on PCI bus 1^

- : the board works only on PCI bus 1^

- : the board works only if it is not configured in Plug&Play modality

- : the board works only on PCI bus 1^

- : the system hangs if  the board is inserted in slot 7 (irq shared with DPT); the driver ion PCI slot 1 is not loaded . The problem is either present with the Embedded driver or the BBS one. Many “Hardware lost interrupt are shown during heavy network loads.

- : if the board is inserted in slot 5 (irq shared with SCSI embedded) the driver is not loaded

- : fail during OS loading. The problem is present both on primary and secondary PCI bus

- : no fail, but the system is isolatded from the network.

- : the diagnostic program is correctly executed, but the system hangs at start-up

- : the diagnostic program is correctly executed, but the system hangs at start-up

- : the diagnostic program is correctly executed, but the system hangs at start-up

# 9 - Warnings & Limitations

With DPT raid controller is mandatory to use the Uninterruptible Power Supply  (UPS)

On RAID 5 applications with HDUs distribuited across the SCSI channels (Main + Pem) is mandatory turn on/off the system by only UPS, and the push-button (on/off) must be locked by the key on “ON” position.

SCO Open Server 5.0 ,in the installation phase, needs the CD-rom and the boot HDU connected by the same SCSI channel in the case of homogeneous controller

No CD bootable support

Video PCI card must be plugged  into Primary PCI bus.

Because there are more devices and PCI slots than PCI interrupts, embedded SCSI controller must share interrupt with a PCI slots. The table below shows connection of the interrupt PCI lines.

| PCI interrupts |  | Description |  | Wired to these PCI slots |
| --- | --- | --- | --- | --- |
| INTA# |  | PCI interrupt signal A to PCEB |  | Slot 5 INTA# |
|  |  | also used by onboard SCSI controller |  | Slot 8,10 INTB# |
|  |  |  |  | Slot 7,9 INTC# |
|  |  |  |  | Slot 6 INTD# |
| INTB# |  | PCI interrupt signal B to PCEB |  | Slot 6 INTA# |
|  |  | also used by onboard VGA controller |  | Slot 5 INTB# |
|  |  |  |  | Slot 8,10 INTC# |
|  |  |  |  | Slot 7,9 INTD# |
| INTC# |  | PCI interrupt signal C to PCEB |  | Slot 7,9 INTA# |
|  |  |  |  | Slot 6 INTB# |
|  |  |  |  | Slot 5 INTC# |
|  |  |  |  | Slot 8,10 INTD# |
| INTD# |  | PCI interrupt signal D to PCEB |  | Slot 8,10 INTA# |
|  |  |  |  | Slot 7,9 INTB# |
|  |  |  |  | Slot 6 INTC# |
|  |  |  |  | Slot 5 INTD# |