Current Monitor Offer

| CRT Size | Model | Random ( Olicode ) | Dot Pitch (mm) | CRT Mask | Max Hor Freq | Max Vert. Freq | Max Resolution | Pixel Rate | ABS Version Availability | Max Power Consum. | Viewable Area (mm) | Supplier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DSM 70-500 | A5770J ##### ( 213516L ) | ##### 0.28 | ##### Shadow | ##### 54 | ##### 130 | ##### 1024x768@60Hz | ##### 65 MHz | ##### Yes | ##### 80 W | ##### 280x212 | ##### HUNDAY |
| 15” | DSM 70-520 | A5915Q ##### ( 213515K ) | ##### 0.28 | ##### Shadow | ##### 69 | ##### 120 | ##### 1280x1024@60Hz | ##### 110 MHz | ##### Yes | ##### 90 W | ##### 282x212 | ##### GOLDSTAR |
|  | DSM 80-525 | ##### G0478Q ( 213521R ) | 0.28 | Shadow | 70 | 150 | 1280x1024@60Hz | 95 MHz |  | 75 W | 262 x 196 | HUNDAY |
|  | DSM 80-501 | ##### G0427V ##### ( 213520U ) | ##### 0.28 | ##### Shadow | ##### 54 | ##### 100 | ##### 1024x768@60Hz |  |  | ##### 80 W |  | ##### LITE-ON |
|  | DSM 70-740 | A5735T ##### ( 213518W ) | ##### 0.28 | ##### Shadow | ##### 69 | ##### 100 | ##### 1280x1024@60Hz | ##### 108.5 MHz | ##### Yes | ##### 110 W | ##### 323x243 | ##### LITE-ON |
| 17” | DSM 70-750 | A5963Y ##### ( 213517M ) | ##### 0.25 | ##### Trinitron | ##### 92 | ##### 150 | ##### 1280x1024@85Hz | ##### 160 MHz |  | ##### 130 W | ##### 321x237 | ##### NOKIA |
|  | DSM 80-745 | ##### G0477F ##### ( 213522J ) | ##### 0.26 | ##### Shadow | ##### 95 | ##### 150 | ##### 1600x1200@75Hz | ##### 205 MHz |  | ##### 130 W | ##### 300x225 | ##### HUNDAY |
| 21” | DSM 60-210 | A6191L ##### ( 213510S ) | ##### 0.26 | ##### Shadow | ##### 107 | ##### 170 | ##### 1600x1200@85Hz | ##### 250 MHz |  | ##### 180 W | ##### 400x298 | ##### FIMI |
|  | DSM 80-215 | G0720L ( 213523K ) | 0.26 | Shadow | 115 | 160 | 1600x1200@90Hz | 250 MHz |  | 150 W | 380x285 | FIMI |

MONITORS OVERBOXING    with AUDIO BASE STATION

| Stock Denomination |  | Random |  | ABS Type |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ABS1 / DSM 70-500 | A5716Y |  | ABS1 |  | 15” 0.28 d.p., Flat, 54 KHz, Universal Power Supply, DDC1/2B, Power Saving, ISO 9241-3 , MPRII, Without Power Cable + Audio Base Station |  |
| ABS2 / DSM 70-520 | A5715X |  | ABS2 |  | 15” 0.28 d.p., Flat,69 KHz, Universal Power Supply, DDC1/2B, ISO 9241-3 ,TCO 95, Without Power Cable, OLIVETTI case with Audio Base Station Plus |  |
| ABS2 / DSM 70-740 | G0353J |  | ABS2 |  | 17”, 0.28 d.p., Flat,69 KHz, ISO 9241-3 TCO 92, DDC1/2B, Universal Power Supply,Power Saving, Without Power Cable, Audio Base Station Plus |  |

MAIN DIFFERENCES BETWEEN ABS1 AND ABS2

| ITEM | ABS1 | ABS2 |
| --- | --- | --- |
| Front Panel command appearance | Label and icons in rilief , not colored | Label and icons not rilief , black colored |
| Other front panel writing | None | Label “ Active Crossover System “, black colored |
| Adhesive label | None | Removable plastic label, 3 colors |
| Woofer speaker | 3 W , 5 inch , shielded | 5 W , 5 inch , shielded |
| Mid-tweeter speaker | 2 x 3 W, 2 inch , shielded | 2 x 5 W, 2 inch , shielded |
| Global power | 3 Wrms (power supply = 9V ) | 13 Wrms (power supply = 10V ) |
| Mid-tweeter power amplifier stage | Single-end | Bridge |

Old Monitor List

| CRT Size | Model | Random ( Olicode ) | Dot Pitch (mm) | CRT Mask | Max Hor Freq | Max Vert. Freq | Max Resolution | Pixel Rate | ABS Version Availability | Max Power Consum. | Viewable Area (mm) | Supplier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14” | DSM 60-400 | 98259B ##### ( 212222U ) | ##### 0.28 | ##### Shadow | ##### 50 | ##### 100 | ##### 1024x768@60Hz | ##### 65 MHz | ##### Yes | ##### 80 W | ##### 250x188 | ##### LITE-ON |
|  | DSM 70-500 | A5770J ##### ( 213516L ) | ##### 0.28 | ##### Shadow | ##### 54 | ##### 130 | ##### 1024x768@60Hz | ##### 65 MHz | ##### Yes | ##### 80 W | ##### 280x212 | ##### HUNDAY |
| 15” | DSM 60-510 | A6142S ##### ( 213505S ) | ##### 0.28 | ##### Shadow | ##### 65 | ##### 110 | ##### 1024x768@75Hz | ##### 85 MHz | ##### Yes | ##### 90 W | ##### 282x212 | ##### GOLDSTAR |
|  | DSM 60-520 | A3953U ##### ( 213513R ) | 0.28 | Shadow | 65 | 110 | 1024x768@75Hz | 85 MHz |  |  |  | GOLDSTAR |
|  | DSM 70-520 | A5915Q ##### ( 213515K ) | ##### 0.28 | ##### Shadow | ##### 69 | ##### 120 | ##### 1280x1024@60Hz | ##### 110 MHz | ##### Yes | ##### 90 W | ##### 282x212 | ##### GOLDSTAR |
|  | DSM 70-740 | A5735T ##### ( 213518W ) | ##### 0.28 | ##### Shadow | ##### 69 | ##### 100 | ##### 1280x1024@60Hz | ##### 108.5 MHz |  | ##### 110 W | ##### 323x243 | ##### LITE-ON |
| 17” | DSM 70-175 | A5734S ##### ( 213519X ) | ##### 0.25 | ##### Diamondtron | ##### 86 | ##### 130 | ##### 1280x1024@75Hz | ##### 135 MHz |  | ##### 130 W | ##### 327x244 | ##### MITSUBISHI |
|  | DSM 60-730 | A5375A ##### ( 213514J ) | 0.39 | Shadow | 54 | 95 | 1024x768@60Hz | 65 MHz | Present | 85 W | 323x243 | LITE-ON |
|  | DSM 70-750 | A5963Y ##### ( 213517M ) | ##### 0.25 | ##### Trinitron | ##### 92 | ##### 150 | ##### 1280x1024@85Hz | ##### 160 MHz |  | ##### 130 W | ##### 321x237 | ##### NOKIA |
| 21” | DSM 60-210 | A6191L ##### ( 213510S ) | ##### 0.26 | ##### Shadow | ##### 107 | ##### 170 | ##### 1600x1200@85Hz | ##### 250 MHz |  | ##### 180 W | ##### 400x298 | ##### FIMI |

MONITORS OVERBOXING    with AUDIO BASE STATION

| Stock Denomination |  | Random |  | ABS Type |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ABS / DSM 60-400 | A6235X |  | ABS1 |  | 14” Display Multif., 0.28 d.p., DDC1/2B, ISO 9241-3 / MPRII, Universal Power Supply,48 KHz, Cables + Audio Base Station |  |
| ABS / DSM 60-510 | A6236Y |  | ABS1 |  | 15” 0.28 d.p., MPRII, ISO 9241-3, DDC1/2B, 64 KHz, Universal Power Supply, Power Saving, Optional TCO, Cables + Audio Base Station |  |
| ABS / DSM 70-500 | A5716Y |  | ABS1 |  | 15” 0.28 d.p., Flat, 54 KHz, Universal Power Supply, DDC1/2B, Power Saving, ISO 9241-3 , MPRII, Without Power Cable + Audio Base Station |  |
| ABS2 / DSM 60-510 | A5374H |  | ABS2 |  | 15” 0.28 d.p., MPRII, ISO 9241-3, DDC1/2B, 64 KHz, Universal Power Supply, Power Saving, Cables + Audio Base Station Plus |  |
| ABS2 / DSM 70-520 | A5715X |  | ABS2 |  | 15" 028 d.p.Flat-65KZ-Universal Power Supply-DDC1/2B-Power Saving-ISO 9241-3, TCO 95/OLIVETTI CASE/ Without Power Cable, Audio Base Station2 |  |
| ABS2 / DSM 60-730 | A5375A |  | ABS2 |  | 17”, 0.39 d.p., MPRII, ISO 9241-3, DDC1/2B, 54 KHz, Universal Power Supply, Power Saving, Without Power Cable, Audio Base Station Plus |  |

Approval MARKS

| MARKS |  |  |  |  |  |  |  |  |  | MODELS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60-400 |  | 70-500 |  | 70-520 |  | 70-740 |  |  | 70-175 |  | 60-730 |  | 60-510 |  |  | 60-520 | 70-750 | 60-210 |
| TCO ‘92 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | X |  |  |
| TCO ‘95 |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |  | X | X |
| Blue Angel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| MPR II | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| ISO 92241-3 | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| GS | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| EPA Nutek | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| CE-NEMKO- DEMKO- FEMKO- SEMKO | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| CSA | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| FCC | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |
| UL | X |  | X |  | X |  | X |  |  | X |  | X |  | X |  |  | X | X | X |