Troubleshooting Guide for Using the Creative Labs CT4171 Audio Card on Computers on Which OS/2 Warp 4.0 is Installed.

- - If the CT4171 board is not recognized as PnP by the operating system OS/2 Warp 4.0 and thus, during the normal installation procedure, a screen is displayed with the audio item set to “none”, proceed with the installation procedure without attempting to modify the part of the screen concerning the audio device.

- Once you have reached the end of the operating system installation procedure, after the computer has bootstrapped for the last time, insert the floppy disk containing the OS/2 Warp 4.0 audio driver installation program in drive “A”.

- Double click on the “System Setup” icon and then on “Command Prompts” and “Os/2 Window”.

- Move to the prompt A:> on the floppy disk by typing “A:” and run the “Install” program, confirming the default resource selections (including the disabling of PnP on the card) and select “Sound Blaster 16” - Install.

- Restart the computer.

- Interrupt the computer bootstrap cycle by pressing “ALT+F1” (during the POD phase in which a small white square followed by “OS/2” appears in the top left-hand corner of the screen) and then press the “F2” key.

- At this point, when the system prompt C:> is displayed, open the system configuration file config.sys by typing “e config.sys”.

- Change the line that loads the audio driver to the following :

- DEVICE=C:\MMOS2\SB16D2.SYS /P /C:1 /D:1 /H:1 /I:5 /A:220 /B:8 /M:330 /N:SBAUD1$

- Save the file, close the editor program and restart the computer again by pressing “CTRL+ALT+DEL”.

The audio board will now be correctly installed and the problems listed below will disappear.