PC Cross Zap Manual

Copyright 1986, 1987, 1994 Hypersoft

PO Box 99685, Raleigh NC 27609

1

READ THIS FIRST

~~~~~~~~~~~~~~~~~~

Notice:

This program is capable of writing to your floppy

disks. It is therefore possible to overwrite and

lose valuable data if you are not careful. Please

be sure to retain backup copies of all disks

considered irreplaceable.

Please read, and make sure you thoroughly

understand this manual. No responsibility can be

accepted for damaged or lost data through misuse

or mistakes on the part of the user.

Trade Marks:

IBM PC and PC-DOS are registered trade marks of

International Business Machines Inc.

TRS-80 is a registered trade mark of Tandy

Corporation.

MS-DOS is a registered trade mark of Microsoft

Inc.

VERY IMPORTANT NOTICE

Due to the way many TRS-80 Operating Systems

format disks, it is quite common for the first

sector on the disk to occur too soon after the

index hole for an IBM computer to read it

properly. If this happens you will get a "Sector

Not Found" error message when PC Cross-Zap tries

to read such a disk. To prevent this, simply take

a small gummed label or disk write protect tab and

stick it on the disk jacket so it covers the index

hole aperture ( the small circular hole about

3/16" diameter about 1" South-East of the disk

center). This will stop the PC floppy disk

controller from resetting when the index hole

comes round. Remove this if you intend to use the

disk in a TRS-80 or you need to reformat it. Many

modern disk drives will not operate if the index

hole is covered, in that case you must make the

hardware mod described in the file problem.doc

2

PC Cross-Zap

==============

Manual V1.2 03/05/87

(c) Copyright Hypersoft 1986

Introduction.

PC Cross-Zap ( or PC-XZ for short ) is a program for

owners of IBM-PC and compatible computers that allows users

to access the files on floppy disks made on TRS-80

computers. Features of PC-XZ include:

o File Transfer

Allowing copying of files from TRS-80 to PC or

from PC to TRS-80 type floppy disks.  Files can be

copied as-is (Image mode) or converted on the fly

(ASCII and Basic modes). In the Basic transfer mode,

TRS-80 Basic programs are converted to a form

compatible with BASICA or GWBASIC.

o Disk Format.

Using PC-XZ you can format a disk that will then

be readable on your TRS-80 by your favorite DOS. All

double density DOS formats are supported including

TRSDOS 2.7/8, TRSDOS 1.3, DosPlus 3.4 and 3.5, LDOS

5.1, NewDOS-80 V2.0, MultiDOS, TRSDOS 6.2. Double Sided

and 35, 40 and 80 track operation are supported as

appropriate. Since PC hardware will not handle single

density only double density formats are supported.

Several Model I operating systems support double

density through the use of a single density track zero.

PC-XZ supports this format to the extent that you can

read and write files to such disks as long as the files

are not on track 0. You cannot format disks like this

however.

o Disk Directory.

This will display either a short form or a full

directory of your TRS-80 disk. A short form directory

is also available for PC/MS-DOS disks.

o Disk Navigator.

In Disk Navigator mode you can examine, modify or

repair disk sectors of all types of disks including

TRS-80 and MS-DOS disks.  Special features are provided

to identify the meanings of bytes in directory entries,

3

GAT, HIT and FAT sectors. An option to automatically

repair damaged GAT and HIT sectors is also provided.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

4

o Shell Command.

The Shell Command allows you to temporarily to

leave PC Cross-Zap and execute any DOS command or other

PC program providing you have enough memory.

o Save Configuration.

The Save Configuration Command allows you to save

the current default settings, including Alien format

specification for automatic restart next time the

program is used.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

5

1.0 Hardware Requirements.

==========================

PC Cross-Zap will run on any IBM-PC or compatible

computer that has the following minimum features:

o Standard PC or MS-DOS 2.0 or higher operating system.

o Minimum of 256 Kilobytes of memory.

o At least one 40 track 5.25" floppy disk drive.

o Standard floppy disk controller.

o DMA controller. (Model 1000 owners please note.)

o Monochrome or Color Graphics Adapter.

The program will run more smoothly with two drives or

at least one drive and a hard disk. A RAM disk can be used

if you have sufficient standard, extended or EMS memory.

You need 256k of memory for all functions except the Shell

Command which needs at least 384 Kilobytes for useful

operation.  If you run any memory resident programs

simultaneously with DOS and PC Cross-Zap you will need

proportionately more memory.

Versions 1.0F and later support High Density floppy

disk drives of the type found on AT type computers like the

Tandy Model 3000. These drives are optional equipment on

most AT's. If you have one you can read, write and format 40

or 80 track TRS-80 type floppy disks. It is not generally

advisable, however, to write to floppy disks on an 80 track

drive if the disk was originally formatted on a 40 track

drive.

PC Cross-Zap also supports regular 80 track drives

which you can add to your PC just like the standard

equipment 40 track drives. PC-XZ will automatically double

step if you select a 40 track TRS-80 format in an 80 track

drive.

The program has been tested and found to run on all of

the following: Standard IBM PC, XT and AT, Tandy Models 1000

(with DMA), 1000EX, 1000SX, 1200, 3000, and a number of

other compatibles. In fact it should run on all compatibles.

1994 comment:

Modern 386 & 486 machines may need to be run in non-turbo

mode (i.e. the slowest clock rate).

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

6

2.0 Getting Started.

=====================

PC Cross-Zap is supplied as a single file XZ.EXE on a

floppy disk. The first thing you should do is make a backup

and put the original away in a safe place.

You may copy the program to a disk with an operating

system on or to your hard-disk if you have one. To run the

program, simply type:

XZ <Enter>

If you are running it from a floppy disk you can remove the

disk containing PC-XZ freeing your drives for file transfer.

If you have an AT with High Density (1.2 Meg) drives see

section 3.8 on specifying floppy drive types.

If you are running PC-XZ for the first time, you may

want to set up a configuration file. This is done by making

various menu selections and then, from the main menu

selecting number 8 from the main menu. This will create a

file called XZCONFIG.DAT in your current disk directory.

When you run PC Cross-Zap it will display choices in

the form of menus.  For the most part you can select the

action you want by a single key stroke as indicated in the

menu. The Escape Key, <Esc> in the menus will take you to

the next higher menu or to DOS if you are in the Main Menu.

Where the function you select requires a typed answer

it may offer you a default as shown in parenthesis (). To

select the default simply press the Enter Key on your

keyboard.

Before we proceed, it would be helpful if the Reader

understands the meanings of various terms used in this

manual. Please skip this section if you think you know all

this information. If you find something later that you don't

understand return here to see if it is explained.

The Menus and the Keyboard.

All the Menus display a list of options, either

numbered or lettered. To select an option simply press the

appropriate number or letter key.

The function keys F1 to F10 are also used for various

special operations so where you see a reference to say F10

this means the key with F10 printed on it.

The Escape key referred to in the menus as <Esc> is

usually marked Esc.  The Return or Enter Key may be marked

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

7

with a name such as 'Retrn' or a graphic symbol such as an

arrow which points down and left.

Memory and Disk Terminology

o ASCII   The American Standard Code for the Interchange of

Information. Used by computers to store text

files. Each character and printable symbol plus

other needed control characters is assigned a

numerical code in the range 0 to 255.

o Bit     The smallest unit of memory storage equivalent to

one cell in a memory chip. The cell can take one

of two possible states: either 'off' or 'on'

represented numerically as 0 or 1.

o Byte    A unit of memory storage capacity. One Byte is 8

bits and can store one text character.

o Nibble  A unit of memory storage equal to half a byte or 4

bits.

o Hex     Hexadecimal (base 16) numbering system is used by

computers and users to represent programs and

data. Convenient because an 8 bit byte can be

represented by just 2 hexadecimal digits.

Hexadecimal counts 0, 1, 2, 3, 4, 5, 6, 7, 8, 9,

A, B, C, D, E, F, 10, 11, 12 etc.  The largest 8

bit number is FF hex or 255 decimal. Hex numbers

are usually shown with an H appended. E.g. 10H

would be 16 decimal.

o Track   The path traced on a floppy disk by the recording

head as the disk spins beneath it. A typical disk

has 40 tracks. Each track can hold up to 6250

bytes of recorded information using double density

recording. Tracks are numbered from 0 so the

highest track on a 40 track disk is number 39.

o Sector  A segment of a track. The size of a sector is

measured in bytes. Not all the track is available

for useful information. Gaps are recorded between

the sectors with special control and sector label

information. Each sector on the disk is identified

by its own unique label. Typical sector sizes are

512 bytes for MS-DOS and 256 bytes for TRS-80 type

DOSs.

o Side    Disks have two physical sides both of which may be

recorded upon. Most MS-DOS systems use both sides.

Until the advent of the TRS-80 Model 4D all

official TRS-80 systems used single sided formats.

Many third party operating systems provide the

capability of double sided operation. Side 0 is

the back (opposite the label), side 1 is the

front.

o Granule Or Gran for short. A TRS-80 unit of disk storage.

Typical Gran sizes are 3, 5 or 6 sectors.

Allocation of space on disks is by Grans so the

smallest amount of space occupied by a file is 1

Gran. Example: on a TRSDOS 1.3 disk which is

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

8

single sided and has 18 sectors per track numbered

1-18, the granule size is 3. Granule 0 would be

track 0 sectors 1-3, Gran 1 would be sectors 4-6

etc.

o Cluster The MS-DOS unit of disk space. On floppy disks a

cluster is 1 sector on single sided disks and 2

sectors on double sided disks. On hard disks a

cluster can be as large as 16 sectors (8192

bytes). Cluster number 2 is always the first

cluster after the directory.

o GAT     Granule Allocation Table. A map of disk space kept

by TRSDOS to speed finding space to put new or

expand existing files. Space is measured in Grans.

o HIT     Hash Index Table. A table kept by TRSDOS that

speeds finding whether a file of a given name

exists in the directory. Each filename is encoded

or 'Hashed' into a one byte code called a

Hashcode.

o FAT     File Allocation Table. A Map of disk space kept by

MS-DOS to speed finding space to put new or expand

existing files. Space is measured in Clusters.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

9

3.0 The Main Menu.

===================

The Main Menu is your key to selecting all the

functions and sub-menus of PC Cross-Zap. It appears when you

first start the program after you have chosen an alien disk

format. If you are in any sub-menu you can return to the

Main Menu by pressing the Escape key. To leave PC-XZ and

return to DOS simply press Escape again.

Following is a detailed description of the Main Menu

functions and options.

3.1 Select Format. (press key '1')

This allows you to select a new alien disk format. Only

one alien disk format at a time may be active. For the

purposes of the Disk Navigator feature (see later) MS/PC-

DOSs are also included in the list. Whenever you select an

operation that works on an alien disk the chosen format will

be assumed. If the disk is not of the assumed format then

you will either get an error message if the disk is

unreadable or you may corrupt the disk if you try and write

to it. It is your responsibility as the user to make sure

that the disk is of the correct format. If you are not sure

try displaying the directory or looking at it with Disk

Navigator before proceeding to any other operation.

Attempting to read a disk will not harm it, even if it is of

the wrong format.

The Select Format option will display a screen of

alternative alien disk formats. Each has a code number or

letter by the side of it. To choose your format type the

appropriate number or letter. When you do so a window will

open displaying your choice and asking if you want to

proceed. For example, if you have selected TRSDOS 6.2 single

sided format, the following should be shown:

Format Selected: Model IV TRSDOS 6.x   ss

Y if OK,  F7 for info,  F8 to revise.

Pressing any key but Y, F7 or F8 will return you to the

format selection.  Pressing Y for Yes will activate the

format and return you to the Main Menu.  Pressing F7 will

open a window which will show you details of the format such

as number of tracks, sides, sectors,  directory track etc.

The format in each case defaults to the standard for

that DOS however the F8 option allows you to modify the

directory track number and the number of tracks. For

instance the LDOS standard is 40 tracks with the directory

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

10

on track 20 and you might want to change these to 35 and 17

respectively.

3.2 File Transfer. (Press '2')

Pressing the '2' key brings up the File Transfer sub-

menu. This has a range of options allowing copying of files

to and from alien disks.

3.2.1 Copying Alien to PC

To copy files from a TRS-80 type floppy disk to an MS-

DOS disk use this option. Shown on the screen will be the

default disk drives. Unless otherwise specified, files will

be copied from the disk in the default alien drive to the

disk in the default MS-DOS drive using the default path

(MS-DOS subdirectory) as shown.

When the Copy Alien to PC option is chosen you will be

prompted for a filespec thus:

Enter Filespec: [drive:]filename/ext  [-option]

Items in square brackets [ ] can be omitted in your reply.

For example, you can enter the following forms of filespec:

filename/ext             e.g.  test/bas

drive:filename/ext       e.g.  b:patient/dat

drive:filename/ext -opt  e.g. b:postage -i

Wildcards are permitted in filenames, for instance

a:*/* would cause the copying of all files on the disk in

drive A. a:xy*/bas would copy all files who's names begin

with xy and whose extensions are /BAS. Note all characters

are converted to upper case inside PC-XZ.

There are three possible modes when copying from TRS-80

to MS-DOS. These modes are: ASCII, Basic and Image. They can

be selected by appending a -A, -B or -I to the filename as

shown in the example above.

ASCII transfer mode converts TRS-80 text files to MS-DOS

format, appending a line feed (0AH) to each carriage return

(0DH), converting multiple spaces to TAB characters where

possible, converting the [ character to ^ and appending a

1AH end of file mark. The MSB (most significant bit) or each

byte is masked out. The file length is also correctly

recorded in the directory.

BASIC transfer mode converts standard Compressed Format TRS-

80 BASIC files to Compressed Format MS-DOS files readable by

BASICA or GWBASIC. Spaces are inserted where necessary and

the syntax is adjusted including replacing PRINT @

statements with LOCATE X,Y. Certain other statements that

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

11

have no direct counterpart in PC Basics or that might give

trouble are flagged by tacking on a ! character. This forces

BASIC to give an error message and tell you where it

occurred so that you can quickly find and correct the line.

In addition a :'@#$% statement is appended to each line with

one or more potential errors.

Image Transfer mode transfers files as-is without any

modification. Use this for Binary files, Data files etc.

3.2.2 Copy PC to Alien

Again, unless otherwise specified, files will be copied

from the default drive and path (subdirectory) of the MS-DOS

system to the default TRS-80 disk as shown in the defaults

at the bottom of the File Transfer Menu screen.  When you

select the Copy PC to Alien option you will be prompted for

the files to copy thus:

Drive:\Path\Spec

At this point you can reply with any standard MS-DOS

filespec. Wildcards are allowed and the Drive and Path are

optional. If you omit either, the defaults will be used.

Note that the filenames in MS-DOS remain the same when

the file gets copied to TRSDOS except that the period (.),

separating the body of the name & the extension, gets

replaced by a slash (/). For example MYFILE.BAS becomes

MYFILE/BAS.  A problem can arise because some MS-DOS

filenames are not legal in TRS-DOS.  In TRS-DOS only the

characters 0-9 and A-Z are permitted whereas MS-DOS also

allows other symbols such as '-' in the name.  TRS-DOS names

must start with a letter so such names as 66/DAT are

illegal. Any files with such names as this must be renamed:

e.g. make 66.DAT into F66.DAT before copying. See use of the

SHELL command.

Valid copy modes are ASCII and Image. Basic files are

not converted in the PC to TRS-80 direction. ASCII files are

converted in the reverse manner to that described in Section

3.2.1 above. TAB characters are expanded to spaces.

3.2.3 List Alien File.

This option allows you to read text files on your TRS-

80 disks without copying them. To call this option press the

'3' key and you will be prompted for a filename. At this

point you should enter a valid name of a file on the disk in

the default TRS-80 drive or you may include a drive letter

as well. Valid responses are, for example:

myfile/asc

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

12

b:maillist/dat

The file will list to the screen of your PC. You may suspend

the listing by pressing any key.  To resume listing press

the Enter Key or to abort press the Escape Key.

3.2.4 Display Directory of Alien Disk

This will display a short form directory of the alien

disk in the alien default drive. There are no options.

3.2.5 Display Directory of MS-DOS disk.

As 3.2.4 above but for the MS-DOS default disk.

3.2.6 Change Defaults

The current defaults for alien and MS-DOS drives are

shown at the bottom of the File Transfer Menu Screen. To

change these press '6' and you will be asked, item by item,

for replacements. As each question is asked you will be

given the current value, shown in parenthesis. To accept

this without revision press the 'Enter' key.

===========================

3.3 (Main Menu) Directory

Pressing '3' at the Main Menu gives a short-form directory

of the alien drive just like the File Transfer Menu. However

you can also type

DIR   or   DIR -a

DIR gives a detailed directory listing showing file size, in

Records and Grans, End Of File, File Date and other

attributes. Only visible files are shown.

DIR -a shows all files including System and Invisible types.

===========================

3.4 (Main Menu) Format Alien Disk.

This option allows you to prepare a new data disk for

your TRS-80 type DOS.  The disk will be formatted in the

currently selected alien format.  All alien formats can be

used except for the fact that some TRS-80 Model I formats

have a single density track 0.  Since the floppy disk

controller on the PC will not read, write or format in

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

13

single density you will not be able to make such disks on

your PC that will be acceptable to your Model I DOS. If you

try and format any of these formats (marked in the menu with

a *) then track 0 will be formatted in double density. You

can still use these disks to make backups and repairs of

your old Model I disks providing you only ever intend to

read them on your PC under PC Cross-Zap.

To format a disk, press '4' at the Main Menu and then

follow the questions listed below.  Note pressing Enter at

any question means 'Use the Default shown'.

Volume Name: The default is DATADISK. Enter any suitable

name, 8 characters maximum or press 'Enter' to accept the

default name 'DATADISK'.

Volume Date: The default is the current MS-DOS system date.

Press 'Enter' to accept this or type the date in the format

MM/DD/YY.

Number of Tracks: The default is the number of tracks

defined in the current alien format selected.

Drive to format: The default is always A. Press 'Enter' to

accept this or any letter corresponding to an existing valid

floppy disk drive.  High density drives are NOT supported.

Most PCs have one or two drives so the answer should

normally be 'A' or 'B'.

Insert disk in drive and press any key when ready (or <Esc>)

At this point, pressing 'Escape' will abort the format and

return you to the Main Menu. Pressing any other key will

start formatting the disk.  Note that the program does not

check that the disk is blank so it is your responsibility to

check this before proceeding.  Failure to do so could result

in loss of valuable data or programs.

Once formatting has started it will proceed to

completion unless an error is encountered. The only way to

abort the formatting procedure is to open the disk drive

door which will cause an error message to be displayed.

Select 'A' to abort.

Your completed disk will be a data disk with BOOT/SYS

and DIR/SYS files only. Unlike many disks made on TRS-80s it

will not need a write protect tab covering the index hole.

If you intend to move data back and forth on a regular basis

it is suggested that you do it in a disk formatted on PC

Cross-Zap.

===========================

3.5 Disk Copy (Key '5' on Main Menu)

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

14

This provides a sector by sector copy of a whole disk.

It requires the destination disk to be preformatted. It is

slow and is not recommend for general purpose disk copying.

To use, press '5' at the Main Menu. You will be asked

for source and destination disk drives. They may be the same

although this is even slower since the transfer buffer is

only 16 kBytes & therefore you will have to swap disks many

times.

Disks Formatted and Copied using PC Cross-Zap will not

need the index hole covering.

===========================

3.6 Disk Navigator

Disk Navigator is a major subsection of PC Cross-Zap,

designed to allow the user to read, modify and write sectors

of floppy disks in any format listed in the Format Selection

Menu.  This is particularly useful if you no longer have

access to your TRS-80 and have problem disks.

Special attention has been paid to providing the user

with guidance when viewing sector data.  See Chapter 4 of

this manual for full details.

===========================

3.7 Shell Command (Key '7' on Main Menu)

The Shell Command allows you to temporarily leave PC

Cross-Zap and execute any series of DOS commands after which

you can return to the program in exactly the same state as

you left. The Shell command causes DOS to run another copy

of COMMAND.COM while leaving PC Cross-Zap in memory in

suspended animation. When you are ready to return from DOS

simply type the command 'EXIT'.

Note that the Shell Command is not fully supported in

releases of MS and PC-DOS earlier than 3.0. As a result,

while it will work in PC Cross-Zap, you will have to reboot

your computer when you finally return to DOS.

To use the Shell Command, press '7' at the Main Menu

and you will be given a prompt. At this point you can type a

single line DOS command such as DIR C:/W which will execute

and return immediately. ALternatively, if you want to

execute a series of DOS commands, press the 'Enter' key at

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

15

the Shell prompt. The screen will clear and you will be in

DOS until you type 'EXIT' to return.

Examples of use of the Shell Command include:

o    Renaming Files

o    Change Subdirectories

o    Copying MS-DOS files

o    Listing or Printing MS-DOS files

o    Determining free disk space.

===========================

3.8 Save Defaults (Key '8' on Main Menu)

If you select this option then PC Cross-Zap will create

a file called XZCONFIG.DAT on your MS-DOS disk. This file

contains information about the configuration of your system,

the default drives you have selected and the current Alien

Format selected. Each time you run PC-XZ it looks for this

file and, if it finds it, the information contained will be

loaded automatically, saving you the trouble of

reconfiguring the program every time you run it.

You can, from the main menu, tell PC-XZ what kind of

drives you have. To do this type: D=X where D is a valid

floppy drive letter and X is 40 or 80 for regular 40 or 80

track floppy disk drives or X is hd for a high density

drive. Use X=0 to remove a specification altogether. For

example, b=40 sets drive B: to be a 40 track drive, a=hd

sets drive A: to be a high density AT type drive.

If you have an AT type computer, PC-XZ automatically

analyzes the equipment status registers and determines the

number and type of floppy drives.

===========================

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

16

4.0 The Disk Navigator

========================

Disk Navigator is essential for examining, modifying

and repairing disk sectors. The Disk Navigator Menu gives

the following options:

1)   View/Edit Disk Sectors

2)   View/Edit Directory Sectors

3)   View/Edit FAT/GAT sectors

4)   Fix Bad GAT sector

5)   Fix Bad HIT sector

6)   Change Default Drive.

4.1 View/Edit Disk Sectors (Press '1')

This allows you to view the data in a specific sector.

To use, make sure your disk is in the default drive ( or

change the default as necessary using option 6.) and press

'1' at the Disk Navigator Menu. You will be prompted:

Enter Track, Side, Sector:

Reply (for example) 1,0,3<Enter> to read sector 3 on side 0

(back) of track 1. Note tracks are numbered from 0, side 0

is the back, side 1 is the front.

Once you have made your selection the sector will be

read into memory and displayed. A flashing cursor points to

the byte under examination. Use the arrow keys to move it

about. Lower right is the Relative Byte Indicator that shows

the position of the cursor relative to the start of the

sector.

In sector examine mode the following keys are active:

Up Arrow:      Moves cursor up one line (-16 bytes)

Down Arrow:         Moves cursor down one line (+16 bytes)

Right Arrow:        Moves cursor right 1 char (+ 1/2 byte)

Left Arrow:         Moves cursor left 1 char (- 1/2 byte)

Home Key:      Moves cursor to top left hand corner

End Key:       Moves cursor to lower right hand corner

PageUp or ;         Move up in memory 256 bytes. If end of

Sector exceeded, read next sector.

Pagedn or -         Move down 256 bytes. Read next sector if

top of screen exceeded.

Shft lft-arrow Read same sector, Side 0

Shft rgt-arrow Read same sector, Side 1

Shft up-arrow  Read next higher sector

Shft dwn-arrow Read next lower sector

Shft PageUp         Get 1st sector of next higher track

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

17

Shft PageDn         Get 1st sector of next lower track

T              Select New Track, Side, Sector

M              Turn on Modify Mode

Z              Zap - Write sector to disk.

Esc            Return to Disk Navigator Menu.

Once Modify Mode is selected you can change sector data

in either Hex or ASCII modes. In Hex mode you type any of

the keys 0-9 or A-F and modify the bytes a nibble at a time.

In ASCII mode you type any printable key character and the

byte equivalent to that is placed at the current cursor

location.

In Modify mode the following special keys are active:

Function Key F2     Modify Mode OFF

Function Key F3     Switch to Hex Modify Mode

Function Key F4     Switch to ASCII Modify Mode.

While in modify mode you are changing only the copy of the

sector data in computer memory. To modify the data on disk,

select F2 to exit modify mode and then Z to write the sector

back to disk.

===========================

4.2  View/Edit Directory Sectors.

This is identical to View/Edit Disk Sectors except for

the following:

1)        By choosing this option PC Cross-Zap automatically

selects the correct side, track and sector for the

start of the directory.

2)        Displayed below the sector is specific information

about the byte under examination. Each directory

sector takes (typically) 32 bytes. The number of

the entry and the byte offset within the entry are

shown on the lower left below the directory sector

number.

3)        The following special keys may be active in

View/Edit Directory Sectors:

F7   Show Disk Format Information

F8   Select New Track, Side, Sector

F9   Return from Viewing Extent/Cluster

F10  View 1st sector of Extent or Cluster

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

18

When the cursor is on an Extent Descriptor pair within

a directory entry, pressing F10 will take you to the track,

side and starting sector of the relevant extent or cluster.

Pressing F9 will return to the same place you left from.

===========================

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

19

4.3 View/Edit FAT/GAT

Like the View/Edit Directory option this will take you

to the point where you are viewing the first sector of the

FAT (MS-DOS) or GAT (TRS-DOS). In the case of TRS-80 type

DOSs, pressing PageUp will take you up one sector so you are

looking at the HIT sector.

When viewing the GAT sector of a TRS-80 disk the following

may be observed:

1)        The allocation bytes are identified and the

logical cylinder number is given. A table is

displayed showing which granules are used and

which are free.

2)        A similar table is available when the cursor is

over the bytes corresponding to the track/sector

lockout table.

3)        Bytes corresponding to the disk name and date are

identified as are those for the AUTO command.

When viewing the HIT sector of a TRS-80 disk:

1)        Each byte corresponds to a directory entry.  The

byte is the hashcode of the filename. The location

of the byte in the table is determined by the

directory sector number and the entry number

within that sector. Three different formulae apply

according to whether the DOS is TRSDOS 2.7/8,

TRSDOS 1.3, or any other DOS. The correct formula

is used and the Directory sector and Entry are

given.

2)        Pressing the F10 key will take you immediately to

view the corresponding directory entry. While

there pressing F9 will return to the same location

in the HIT sector.

When viewing the FAT (File Allocation Table) of an MS-DOS

disk:

1)        Each 3 byte group describes the allocation or use

of two clusters-worth of space on the disk.

Because of the scrambled way this is done (see the

Chapters on disk formats) PC Cross-Zap decodes

these for you. Wherever the cursor is, corresponds

to one of 3 bytes in a 3 byte group. Those 3 bytes

are highlighted in two colors so that you can see

which parts of which bytes are used to describe

the two clusters. See Chapter 6 on MS-DOS formats

for more information.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

20

2)        Press F5 for 1st sector of the 1st cluster.

Press F6 for 2nd sector of the 2nd cluster.

3)        When viewing either cluster (having pressed F5 or

F6) pressing F9 will return you to the same place

in the FAT that you left from.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

21

5.0 TRS-80 Disk Formats

=========================

This chapter gives details of how data is organized and

recorded on TRS-80 type disks. A knowledge of this

information is essential if you are going to use the Disk

Navigator feature of PC Cross-Zap.

Information is recorded on floppy disks using a

magnetic recording head similar to a tape recorder. The

information, called data, is recorded in circular rings

called tracks. Each track can hold up to 6250 bytes

(standard 5.25" floppy disk, single sided, double density).

However, because of control information only 4608 of these

bytes are available for user programs and data. These 4608

are split into 18 groups of 256 bytes, each group being

called a sector. Some of the control information is used to

label each sector so the operating system can find it's

unique label called an Address Mark. Preceding each sector

one special byte called the Data Mark is recorded. Normally

this is FB hex but, on TRS-80 disks can be F8 hex on some

sectors.

All TRS-80 formats are based on the original TRSDOS

developed for the TRS-80 Model I. This was a 35 track single

sided single density format. The directory was in the middle

of the disk at track 17. The directory track sectors used a

different address (0FAH)mark to all others (0FBH). When

double density formats were introduced the same principle

was followed except that the directory track address mark

used 0F8H for the address mark. The exception is Model III

TRSDOS 1.3 where the marks are reversed: All sectors EXCEPT

the directory have an F8 data mark while the directory

sectors have an FB.

Typical TRS-80 disks have 35, 40 or 80 tracks, and may

be single or double density, single or double sided. We will

only discuss double density here since these are the only

formats accessible to PC Cross-Zap.

-----------------------

5.1 The Boot Sector.

The first sector of the first track of all TRS-80 disks

is used for the Boot Sector. This would normally contain a

loader program so that the TRS-80 can get the Operating

System into memory. The TRS-80 ROM contains a routine that

loads this sector into memory at 4300H (4200H Model I) and

then jumps to the start at 4300H. The first 2 or 3 bytes are

set up so that the computer ignores them, typically they are

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

22

4300: 00       NOP       ; No Operation

4301: FE 11    CP   11H       ; Compare A

with 17

The DOS loader can tell from the 3rd byte that the directory

track is on track 17 (decimal). This is the only essential

piece of information that must be in the Boot sector for it

to be properly recognized by DOS. Two exceptions to the

above may be found: TRSDOS 1.3 omits the NOP and DOS+ 3.4

sets most significant bit of the byte containing the

directory track to a 1 so it appears as 91H instead of 11H.

When DOS makes a data disk it puts a simple program in

the boot sector that loads, clears the screen and pronounces

itself 'Not a System DIsk' or 'This is a Data Disk'. However

this is not necessary for any other purpose.

On Model I disks the boot sector is sector 0 and is in

single density. Self booting disks must have at least sector

0 in single density so the most common solution for Model I

double density disks is to make the whole of track 0 in

single density and all the rest double density. As is

mentioned elsewhere in this manual, the PC and hence PC

Cross-Zap cannot read single density so you will not be able

to access anything on track 0 of such disks.

-----------------------

5.2 The Directory.

The Directory of a disk provides a record of what files

are on the disk, what space is used for each and where, and

how much free space is left. On a TRS-80 disk, for each file

there is an entry describing where the file is, it's size,

date, passwords and other information.

For most types of TRS-80 operating system the directory

occupies a complete physical track. In double density this

means 18 sectors (36 for some double sided formats). The

directory consists of 3 parts:

1)        The GAT sector containing

- A Granule Allocation Table (free space map).

- A Track Granule Lockout Table.

- The Disk Name, Date and Master Password.

- The Disk AUTO Command.

- Other Misc Information.

2)        The HIT (Hash Index Table) sector containing

- A quick means for DOS to determine whether a

particular file is on the disk or not. Saves the

DOS reading the entire directory.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

23

3)        The Directory proper. This occupies from 8 to 28

sectors, each sector is 256 bytes and can hold 8

file entries (excepting TRSDOS 1.3 which holds 5

entries). Each entry is 32 bytes (48 for TRSDOS

1.3).

A 32 byte directory entry contains the following

information:

Byte                Use

00H       File Type and Access Level.

01H       Misc Flags and part of date.

02H       Rest of date (Day and Year)

03H       End Of File (EOF) byte.

04H       Logical Record Length

05-0CH    File Name - 8 characters

0D-0FH    File Extension - 3 chars.

10-11H    Access Password

12-13H    Update Password

14-15H    No of Records in File.

16-17H    Extent #1 descriptor

18-19H    Extent #2 descriptor

1A-1BH    Extent #3 descriptor

1C-1DH    Extent #4 descriptor

All DOS types except TRSDOS 1.3:

1E-1FH    Extended Entry flag & link pointer

TRSDOS 1.3 only:

1E-2DH    Extent #5-#12 descriptors

2E-2FH    Unused.

The Extent Descriptors define where the file is on the

disk. It may be broken into several fragments or Extents.

Each fragment is a whole number of grans in size and the

Extent descriptor specifies where the extent starts and it's

size in grans. If a file is broken into more than 4

fragments additional directory entries known as FXDEs (File

eXtended Directory Entries) are used. The exception is

TRSDOS 1.3 which uses 48 bytes per directory entry giving

the possibility of 12 extents in one entry. FXDEs are not

used in TRSDOS 1.3.

An FXDE looks just like a regular directory entry

(FPDE, File Primary Directory Entry) except that bytes 02H

through 14H are unused. Byte 01H is used as a reverse link

pointer indicating where the previous FXDE or FPDE is to be

found.

We will now discuss the meanings of the bytes in the

Directory Entry in more detail and indicate the differences

between DOSs if of any significance.

Byte 0H: File Type and Access Level

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

24

Bit

7    0=FPDE, 1=FXDE

6    0=User file, 1=System file

5    LDOS reserved, MultiDOS VFU % flag, others not

used.

4    1=Active file, 0=Deleted (KILLed) file

3    0=Visible, 1=Invisible file

2 )  File Access Level:

1 )  000 Full   001 Kill   010 Rename   011 Not used

0 )  100 Write  101 Read   110 Exec     111 Access

Byte 01H Misc Flags and Month Date

Bits

7    0 Shrinkable,  1 Non Shrinkable    )

NewDOS: 0 File may expand          ) Not

6    0 File backed up, 1 not backed up  ) used

NewDOS: Deallocation flag          ) by

5    LDOS reserved, DOS+ not used       ) Multi

NewDOS: Updated flag               ) DOS

4    LDOS only: MOD flag                ) Not

3    )                                  ) used

2    ) Month         0000=no date       ) by

1    ) file          0001=January       ) NEWDOS

0    ) created       1100=December      )

Byte 02H: Rest of Date

Bit       All DOSs except    TRSDOS 1.3    NEWDOS

7    )                        )         )

6    ) Day     00=no date     ) Year    )

5    )  of     01=1st         ) offset  ) Not

4    ) Month   1F=31st        ) from    ) Used

3    )                        ) 1900    )

)                        )         ) 00

2    ) Year    00=1980        ) 00=1900 )

1    ) from    07=1987        ) 56=1986 )

0    ) 1980                   )         )

Byte 03H  EOF End of File

This byte indicates where, in the last sector of the

file, the file ends. For example, 00 means the file ends

exactly at the end of the last sector, 47 means that the

first 47H bytes only of the last sector are meaningful.

Bytes 48H to FFH do not belong to the file.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

25

Byte 04H  LRL Logical Record Length.

This byte tells DOS what size each record is for Random

file access. For instance, if you have a mailing list

program where each record is 80 (decimal) characters, the

LRL will be 50H. This is used only by the TRS-80 type DOS to

handle reading and writing one record at a time. Physically

on the disk, the records are stacked end to end in a

continuous stream. PC Cross-Zap does not need or use this

byte in transferring files. Note that other DOSs such as

MSDOS have no way of indicating in the directory what the

LRL of a file is.

NewDOS 80 does not use this byte, except when the file

is created. It is included for compatibility only.

Bytes 05 to 0FH  File Name and Extension.

The first 8 bytes contain the file name, left justified

and padded with spaces. The next 3 contain the filename

extension, left justified and padded with spaces. Only the

characters A-Z and 0-9 are permitted and the first character

of both name and extension must be a letter. The extension

can be omitted in which case it will appear as all spaces.

Bytes 10-11H  File Update Password.

These 2 bytes represent the encrypted password that

permits users to update files. The code for no password is:

TRSDOS 1.3:         EF5C

TRSDOS 2.7/8:       CD40

All others:         9642

To remove password replace with the correct code above.

Bytes 12-13H  File Access Password

As above but this password, depending on the access

code in Byte 00, restricts use of the file. Same codes as

the Update Password above.

Note PC Cross-Zap ignores passwords for file transfer.

Bytes 14-15H  Number of Records in File.

This field indicates the last sector written in the

file expressed Least significant byte first. If EOF (Byte

03) is not zero then add 1 to number of records to get the

total.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

26

All DOSs count the number of records from zero except

TRSDOS 1.3 and 2.7/8 which count from 1. Thus 0C00 means 12

sectors for all DOSs except TRSDOS 1.3 and 2.7/8 where it

would mean 11 sectors.

Bytes 16-17H  The 1st Extent Descriptor.

Byte 16H  contains the starting cylinder number (or lump

number for NewDOS). If this byte is FF then this

extent is not used.

Byte 17H  Bits 0-4 represent the number of contiguous grans

in this fragment of the file. Note that 0=1 gran,

1=2 grans etc.  EXCEPTION (as usual), TRSDOS 1.3

and 2.7/8 where the count is from 0. E.g. 1=1

gran, 2=2 grans etc.

Byte 17H  Bits 5-7 indicate the number of the 1st granule on

the track or lump where this fragment of the file

starts. Offset from 0, i.e. 0=starts on 1st gran,

1=2nd gran etc.

Example:  0322 means the file starts on Track 3, granule 3

and this fragment is 5 grans in size. (Not TRSDOS

1.3 or 2.7/8)

A Note about Relative Sectoring.

--------------------------------

Here we come to a major difference between NewDOS 80

(also Model I DoubleDOS) and all other DOSs.  NewDOS 80 uses

logical units called LUMPS instead of physical tracks. A

lump is normally 10 sectors = 2 grans and you must calculate

the true physical position from the beginning of the disk

(track 0). For instance the directory of NewDOS almost

always starts on Lump 17 (decimal) that is 170 sector from

the beginning of the disk. If the disk is a mixed density

disk then ignore track 0 and start counting from the

beginning of track 1. Thus the directory of a single sided,

double density, Model I NewDOS disk will actually start on

physical track 10, sector 8. ( 9*18+8 = 170 ).

In all other respects NewDOS uses the Extent Descriptor

in the same way as other DOSs. It is only necessary to

convert the lump number in Byte 16 to an equivalent physical

track number and sector and then count forward by the number

of grans indicated by bits 5-7 of Byte 17H to find the start

of the extent on the disk.

Bytes 18-19H  The 2nd Extent Descriptor.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

27

As for 16-17H except if Byte 18H=FFH then no more

extents.

Bytes 1A-1BH  The 3rd Extent Descriptor.

As for 18-19H except if Byte 1AH=FFH then no more

extents.

Bytes 1C-1DH  The 4th Extent Descriptor.

As for 1C-1BH except if Byte 1CH=FFH then no more

extents.

Bytes 1E-1FH  FXDE pointer   (Not TRSDOS 1.3)

If Byte 1EH is FFH then no more extents. Extent 4 was

the last.

If Byte 1EH is FEH then at least one FXDE is present

with extent descriptors describing more fragments of the

file.  In this case Byte 1FH points to the sector and entry

containing the FXDE.  Bits 3-7 give the directory sector

number, and bits 0-2 give the number of the entry in the

sector where the FXDE will be found.  Note DOS tries to put

the FXDEs in the same sector as the FPDE wherever possible.

Bytes 1E-2DH  Extents 5 to 12 for TRSDOS 1.3 only

TRSDOS 1.3 does not use FXDEs, instead it uses 48 bytes

for each file entry allowing these additional 8 Extent

descriptors making a maximum of 12 in all. Bytes 2E-2FH are

not used.

-----------------------

5.3 The Granule Allocation Table (GAT Sector)

GAT bytes 00-BFH

The principle use of the Granule Allocation Table is to

provide a map of disk use, Granule by Granule. This map

helps DOS quickly determine what space is used and what is

available when a new file is being added to the disk.

In fact the GAT proper contains no unique information

that could not be obtained from the directory sectors.

However to get that information DOS would have to read the

entire directory which would be time consuming or would use

a considerable amount of memory. PC Cross-Zap does this when

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

28

it reconstructs the GAT by reading all directory sectors in

the GAT Repair feature of Disk Navigator.

The GAT uses one byte for each track (or lump with

NewDOS).  On all DOSs that allow double sided operation

(except NewDOS) a track includes all the sectors on the

front and back. Each bit of each byte represents the status

of one of the grans in the track or lump. Bit 0 for the 1st

gran, bit 1 for the 2nd and so on. In some cases where there

are only 3 grans per track then only 3 bits are used. If the

gran is used then the bit is set to a 1, and if not used

then it is a 0. All unused bits are set to 1.

With a few exceptions the GAT table occupies Bytes 00-

5FH of the GAT sector. A similar table is kept at 60-BFH and

this is used as a granule  lockout table to disable access

to sections on the disk that were found to be bad at format

time. Bits are set to a 1 if the granule is locked out.

Where the disk has too many tracks or lumps to be

represented in 60H bytes then the lockout table is not used.

An example of this is NewDOS 80 double sided which has 144

lumps (on a 40 track ds disk) and uses bytes 00-8FH for the

GAT.

GAT bytes C0-CFH  Miscellaneous.

Bytes C0-CA.  Not used except TRSDOS 2.7/8 puts an ID code

of 27H at C0. If the DOS supports a hard disk it

may use all the bytes from 00 to CA for the

Granule Allocation Table (202 tracks max).

Byte CB   Contains a code which indicates the type of DOS.

00 TRSDOS 2.7/8, 16/17 MultiDOS, 21 DoubleDOS, 32

TRSDOS 1.3, 34/35 DosPlus, 51 LDOS, 62 TRSDOS 6.x,

82 NewDOS 80

Byte CC   No of tracks on disk in excess of 35. Exception:

MultiDOS where it is number of logical cylinders.

Not used by NewDOS.

Byte CDH  LDOS: bits 0-2 Grans/Cyl-1, bit 5=1 if 2 sided,

bit 6=1 id DDEN.

DOS+ 3.5 Bit 5 only, =1 if 2 sided disk.

MultiDOS: number of physical cylinders.

NewDOS, TRSDOS = 0

Bytes CE-CFH Disk Master Password:

When the disk master password is "PASSWORD"

Password code is:   TRSDOS 1.3:    D38F

TRSDOS 2.7/8:  13CC

All others:    E042

GAT bytes D0-DFH  Disk Name and Date.

The first 8 bytes are used for the disk name. Normally

limited to printable text characters.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

29

The 2nd 8 bytes are used for the disk creation date in

the format MM/DD/YY

GAT bytes E0-FFH  DOS  AUTO Command

32 bytes are available for a DOS automatic startup

command which gets executed immediately DOS has finished

booting. If the 1st byte at E0H is an 0DH then there is no

AUTO command.

-----------------------

5.4 The Hash Index Table (HIT Sector)

The HIT sector of a TRS-80 Disk provides a quick way

for DOS to determine if a file exists and if so, which

directory sector it is in.  This design helps to reduce the

amount of memory needed by directory operations at the same

time allowing fast and efficient disk access.

Each location in the HIT sector's 256 bytes corresponds

to a possible directory entry. This imposes a maximum number

of 256 files in any TRS80 directory no matter how big the

disk or how many sectors are allocated to the directory. The

location in the HIT is mapped from the directory sector

number and entry according to one of three possible

formulae:

Let S = Directory Sector Number    (0 .. 27 max)

E = Entry number within sector (0 .. 7  max)

Then for  TRSDOS 1.3:         H = E+5S

TRSDOS 2.7/8:       H = E+8S

All others:         H = S+32E

H is the address of the byte in the HIT table.

The value of the byte, the Hashcode, is computed from the

corresponding filename according to the following simple

algorithm:

Start with hashcode byte=0

Loop 11 times doing the following:

Get next character of filename

Exclusive Or into Hashcode

Rotate Hashcode byte circularly 1 bit left

Loop till done

If result=0 then set Hashcode=1

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

30

If there is no filename or the file was deleted then the

Hashcode byte is set to 0. File Extended Directory Entries

(FXDE) take on the same Hashcode as the Primary Entry

(FPDE).

The hashcode algorithm does not always give a unique

result. Several different names can give rise to the same

code. When this happens it is called a hash collision. When

DOS tries to find a file it computes the hashcode and

searches the HIT table. If it finds a match it checks the

corresponding directory sector. If the desired filename is

not present because of a hash collision, it goes back to the

HIT and searches for any further occurrences of the hashcode

until it has either found the file or exhausted the HIT

table.

Some locations in the HIT table are never used because

of the way directory sector entries are mapped. NEWDOS 80

uses an unused HIT byte at location 1FH to store a number

which indicates how many sectors (in excess of 10) are used

in DIR/SYS which comprises the GAT, HIT and directory

sectors. Legal values are 0, 5, 10, 15 and 20 (decimal).

===================================

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

31

5.5 TRS-80 Disk Formats Summary

=================================

Model    DOS  Tracks Sides Track DIR  1st  Sector Dir  Grans

0 Den DAM Sector skew Track /Lump

I   DOS+ 3.4   40    1    S    F8    0      6   17

I   DoubleDOS  40    1    S    F8    0      6   10     2

I   LDOS/Sole  40    1    S    F8    0      3   17

I   NewDOS 80  40    1    S    F8    0      6   10     2

I   NewDOS 80  40    1    S    F8    0      3    5     2

I   NewDOS 80  80    2    S    F8    0      6   39     8

I   TRSDOS 2.8 40    1    S    F8    1      3   17

I   MultiDOS   40    1    S    F8    0      3   17

III  DOS+ 3.4   40    1    D    F8    0      6   20

I/III DOS+ 3.5   40    1    D    F8    0      3   20

I/III DOS+ 3.5   40    2    D    F8    0      6   20

I/III DOS+ 3.5   80    2    D    F8    0      6   40

I/III LDOS 5.1   40    1    D    F8    0      3   20

I/III LDOS 5.1   40    2    D    F8    0      6   20

I/III LDOS 5.1   80    2    D    F8    0      6   40

III  NewDOS 80  40    1    D    F8    0      3    9     2

III  NewDOS 80  40    2    D    F8    0      3    4     2

III  NewDOS 80  80    2    D    F8    0      3   38     8

III/4 MultiDOS 2 40    1    D    F8    0      3   17

III/4 MultiDOS 2 40    2    D    F8    0      3   17

III/4 MultiDOS 2 80    2    D    F8    0      3   34

III  TRSDOS 1.3 40    1    D    FB    1      3   20

4   DOS+ 4.0   40    1    D    F8    0      3   17

4   DOS+ 4.0   40    2    D    F8    0      6   20

4   DOS+ 4.0   80    2    D    F8    0      6   40

4   TRSDOS 6.2 40    1    D    F8    0      3   20

4   TRSDOS 6.2 40    2    D    F8    0      6   20

4   TRSDOS 6.2 80    2    D    F8    0      6   40

Notes: The NewDOS 80 formats above have the following

PDRIVE parameters:

M1 40tk SS DD:  TC=39 SPT=18 GPL=2 DDSL=17 DDGA=2

M1 40tk DS DD:  TC=39 SPT=36 GPL=2 DDSL=17 DDGA=6

M1 80tk DS DD:  TC=79 SPT=36 GPL=8 DDSL=35 DDGA=6

M3 40tk SS DD:  TC=40 SPT=18 GPL=2 DDSL=17 DDGA=2

M3 40tk DS DD:  TC=40 SPT=36 GPL=2 DDSL=17 DDGA=6

M3 80tk DS DD:  TC=80 SPT=36 GPL=8 DDSL=35 DDGA=6

When selecting a NewDOS 80 format you can change the

number of tracks, but do not change the directory track.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

32

TRS-80 Disk Formats Summary (Contd.)

======================================

Model    DOS     Tks Sds GAT Sectors  Directory  Passwords

ID  / Gran  Start  End  Disk File

I   DOS+ 3.4   40   1  34    6      306   323  E042 9642

I   DoubleDOS  40   1  21    5      188   197  9642 9642

I   LDOS/Sole  40   1  51    6      306   323  E042 9642

I   NewDOS 80  40   1  82    5      188   197  E042 9642

I   NewDOS 80  40   2  82    5      206   223  E042 9642

I   NewDOS 80  80   2  82    5     1436  1465  E042 9642

I   TRSDOS 2.8 40   1  27    3      306   323  13CC CD40

I   MultiDOS   40   1  17    6      306   323  9642 9642

III  DOS+ 3.4   40   1  34    6      360   377  E042 9642

I/III DOS+ 3.5   40   1  35    6      360   377  E042 9642

I/III DOS+ 3.5   40   2  35    6      720   737  E042 9642

I/III DOS+ 3.5   80   2  35    6     1440  1457  E042 9642

I/III LDOS 5.1   40   1  51    6      360   377  E042 9642

I/III LDOS 5.1   40   2  51    6      720   755  E042 9642

I/III LDOS 5.1   80   2  51    6     1440  1475  E042 9642

III  NewDOS 80  40   1  82    5      170   179  E042 9642

III  NewDOS 80  40   2  82    5      170   199  E042 9642

III  NewDOS 80  80   2  82    5     1400  1429  E042 9642

III/4 MultiDOS 2 40   1  FF    6      306   323  E042 9642

III/4 MultiDOS 2 40   2  FF    6      612   647  E042 9642

III/4 MultiDOS 2 80   2  FF    6     1224  1259  E042 9642

III  TRSDOS 1.3 40   1  32    3      306   323  D38F EF5C

4   DOS+ 4.0   40   1  60    6      360   377  E042 9642

4   DOS+ 4.0   40   2  60    6      720   755  E042 9642

4   DOS+ 4.0   80   2  60    6     1440  1475  E042 9642

4   TRSDOS 6.2 40   1  62    6      360   377  E042 9642

4   TRSDOS 6.2 40   2  62    6      720   755  E042 9642

4   TRSDOS 6.2 80   2  62    6     1440  1475  E042 9642

Note: Directory start and end given as relative sector

numbers

counted from track 0, side 0, sector 0.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

33

6.0 PC/MS-DOS Disk Formats

=============================

This chapter gives details of how data is organized and

recorded on MS-DOS type disks. A knowledge of this

information is essential if you are going to use the Disk

Navigator feature of PC Cross-Zap.

Throughout this manual, reference is made to MS-DOS.

This is the version of DOS sold with all PC-Compatible

Machines such as the Tandy range. It was written by

Microsoft. The version sold by IBM under the name PC-DOS is

functionally identical and, as far as disk format

considerations are concerned, it is exactly the same so

wherever you read 'MS-DOS' you can be assured that the same

discussion applies for PC-DOS.

-------------------------

6.1 MS-DOS Format Overview

We will restrict our discussion here to formats found

on standard 3.5" and 5.25" standard (not high-density)

floppy disks. The format has four parts:

The Boot Sector.    In regular formats this is always the

first sector (sector 1) of the first track (track

0). It serves to provide a loader for loading the

operating system into memory. It may contain a

table which defines the format parameters.

The FAT.  Or File Allocation Table provides a map of disk

space used by files and also a link list of

clusters assigned to them. Two copies are kept.

Normally the first FAT follows immediately after

the boot sector and the second copy is contiguous

with this.

The Directory. Usually follows immediately after the 2nd FAT

and consists of at least 4 sectors. Each sector is

divided into 32 byte entries, one for each file

name. Each entry has the filename, attributes,

date and time of creation, file size and a pointer

to the starting cluster of the file. Looking at

the corresponding position in the FAT gives the

next cluster which in turn gives the next and so

on.

The File storage area.  The rest of available disk space is

used to store files. The disks is filled from

track 0 upwards. For double sided disks, side 0 is

used first, then side 1, before going to the next

higher track number.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

34

The file storage area is divided up into sectors,

as physically recorded on the disk these are

usually 512 bytes in size. DOS manages them in

terms of units called "Clusters". Typically, on

single sided disks, a cluster is 1 sector or 512

bytes. On double sided disks a cluster is usually

2 sectors or 1024 bytes. On hard disks a cluster

may be as large as 8192 bytes. The first cluster

used, follows immediately after the directory and

is cluster number 2. Subsequent clusters are

numbered sequentially from this.

6.2 The Directory.

The Directory of an MSDOS disk consists of a number of

512 byte sectors divided into 32 byte sections. Each section

describes one file on the disk. We will now discuss the

meaning and use of each of the 32 bytes.

Byte(s)        Meaning and Use

0-7       Main Body of Filename, 8 characters Max, left

justified and padded with spaces. If entry

not previously used then Byte 0 = 0. If entry

deleted then Byte 0 = E5H

8-10      Extension part of name, 3 characters maximum,

left justified and padded with spaces.

11        File attribute byte. This defines the nature

and use of the file. Meanings of the bits:

Bit 0  If 1 then file is Read Only

1  If 1 then file is Invisible

2  If 1 then file is a System File

3  If 1 then this is The Volume Name

4  If 1 then file is a Sub-Directory

5  If 1 then file has not been archived

6  Not defined

7  Not defined

12-21     Unused. Reserved for future use.

22-23     Time of file creation, Byte 22 bits 0-7, Byte

23 bits 8-15. Hours = bits 11-15,

Minutes = bits 5-10, Seconds = bits 0-4.

24-25     Date of file creation, Byte 24 bits 0-7, Byte

25 bits 8-15. Year=1980 + bits 9-15,

Month = bits 5-8, Day = bits 0-4.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

35

26-27     Starting Cluster in the file. Byte 26 is the

least significant byte, 27 is most.

28-31     Files size in bytes. 28 is least significant

byte, 31 is most significant.

-------------------------

6.3 The File Allocation Table.

The File Allocation Table or FAT provides a map of disk

use, how much disk space is free and a linked list of

pointers showing each cluster of each file is located.

The FAT is held on the disk as a table of 12 bit

cluster pointers, one for each possible cluster on the disk.

Looking at a typical FAT with PC Cross-Zap you will see  a

somewhat confusing situation. This is because each

successive group of 3 bytes is used to hold two 12-bit

cluster pointers in a scrambled fashion. Looking at a

typical example you might see: 03 40 00. This must be

decoded as follows: take the middle byte and split it into

two nibbles, the 4 and the 0. Take the 2nd nibble (0 in this

case) and put it on the front of the 1st byte to make 003.

Take the 1st nibble (4) and put it on the end of the 2nd

byte to make 004. Diagrammatically this can be represented

as

3 bytes in FAT:        03   40   00

||   ||   ||

||   ||___||

||____|   |||

|||        |||

2 clusters:           003        004

-------------------------

6.4 An Example

Let us take a more complete example, the first part of

an actual FAT display on a double sided MS-DOS 2.11 disk

looked like this:

Address                     FAT  Bytes

---------------------------------------------------------

0000      FD FF FF 03 40 00 05 60 00 07 80 00 09 A0 00 0B

0010      C0 00 0D E0 00 0F 00 01 11 20 01 13 40 01 15 60

0020      01 17 80 01 19 A0 01 1B C0 01 1D E0 01 1F 00 02

0030      21 20 02 23 40 02 25 60 02 27 80 02 29 A0 02 2B

0040      C0 02 2D F0 FF FF 0F 03 31 20 03 FF 4F 03 35 60

0050      03 37 80 03 ......

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

36

To make it readable, first we group it into threes:

-----------------------------------------------------------

FD FF FF | 03 40 00 | 05 60 00 | 07 80 00 | 09 A0 00

0B C0 00 | 0D E0 00 | 0F 00 01 | 11 20 01 | 13 40 01

15 60 01 | 17 80 01 | 19 A0 01 | 1B C0 01 | 1D E0 01

1F 00 02 | 21 20 02 | 23 40 02 | 25 60 02 | 27 80 02

29 A0 02 | 2B C0 02 | 2D F0 FF | FF 0F 03 | 31 20 03

FF 4F 03 | 35 60 03 | ......

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

37

Then we rearrange the 2 halves of each middle byte:

Cluster            Decoded FAT table contents

Address

---------------------------------------------------

0000      FFD  FFF | 003  004 | 005  006 | 007  008

0008      009  00A | 00B  00C | 00D  00E | 00F  010

0010      011  012 | 013  014 | 015  016 | 017  018

0018      019  01A | 01B  01C | 01D  01E | 01F  020

0020      021  022 | 023  024 | 025  026 | 027  028

0028      029  02A | 02B  02C | 02D  FFF | FFF  030

0030      031  032 | FFF  034 | 035  036 | .....

At each cluster address in the table is a 3 digit Hex

(12 bit binary) number. If the number is non zero then the

cluster is used or otherwise reserved. If the number is from

FF8H to FFFH then this cluster is the last in the file. If

it is from FF0H to FF7H then the cluster is bad and locked

out. Otherwise the number indicates the next cluster in the

file. The first 2 entries are never used except as a DOS

type identifier. For standard 40 track floppy disks the

following is true:

Ist byte in FAT     Disk type

FC        Single sided 9 sectors/track

FD        Double sided 9 sectors/track

FE        SIngle sided 8 sectors/track

FF        Double sided 8 sectors/track

The corresponding directory entry for the FAT example above

looked like this:

Address                     Data

-----------------------------------------------------------

0000  4B45 524D 4954 2020 4558 4520 0000 0000  KERMIT  EXE

0010  0000 0000 0000 1180 480B 0200 00AF 0000  .............

0020  4D53 4B55 524D 4954 494E 4920 0000 0000  MSKERMITINI..

0030  0000 0000 0000 326F 310B 2E00 1001 0000  .............

0040  .......

So, on this disk the first file is KERMIT.EXE, its size

is 0000AF00H (=44800 decimal) bytes and it's first cluster

is number 0002 (from bytes 26,27 = 001AH,001BH above).

Referring back to the descrambled FAT at location 2 we

see the number 003. This means the next cluster is number

003. Looking at location 003 we see 004 which means cluster

004 is next and so on. If we follow through to the end we

see that at location 02C is 02D and at 02D is FFF which

means cluster 02D is the last cluster in KERMIT.EXE.  So our

file occupies clusters 002 to 02D inclusive or 44 clusters.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

38

Each cluster on this disk is 1024 bytes so the file occupies

45056 bytes of disk space. The end of file is part way

through the last cluster.

It should be noted that DOS 3.0 and higher is capable

of writing disks with 16 bit allocation in the FAT entries.

This is only likely to be found on Hard Disks. With the

standard 12 bit scheme, up to 4080 clusters can be specified

which is more than adequate even for a 1.2 Meg AT High

Density disk. However, with a large hard disk this forces

large cluster sizes - for instance a 20 Megabyte disk would

force the use of 8 kbyte clusters which is very inefficient

with large numbers of small files.  With 16 bit FATs the

cluster size can be reduced to say 2K resulting in much less

wasted disk space.

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.

39

PC Cross-Zap Manual - Copyright (c) 1986 Hypersoft.