How to Read those "Problem" TRS-80 disks using PCXZ.

Copyright 1987, 1994 Hypersoft

PO Box 99685, Raleigh NC 27609

Some disks made on TRS-80s may seem hard or impossible to read

on your PC using PC Cross-Zap (PCXZ).  The fundamental reason for

this is that the Floppy Disk Controllers used in the TRS-80 and IBM-

PC are of different designs and have some significantly different

operating characteristics.  Depending on the TRS-80 DOS used, it is

possible to format disks which have at least one sector per track

which cannot be read on a PC. This happens when the sector starts or

ends too close or overlaps a line drawn radially from the center of

the disk through the index hole.

Each time the disk makes one complete revolution, a light shines

through the index hole and a photodetector sends a pulse to the disk

controller.  If this occurs while your PC controller is reading or

writing a sector then the operation will be aborted and the sector

will be lost or corrupted.  There are several possible ways to get

round this problem.

1) Cover the index hole on the disk. Use a piece of opaque tape or a

disk write protect tab and cover the small hole in the disk jacket

on one side. This prevents the index pulses being generated and

thus allows the difficult sector to be read without the PC

controller aborting. Be careful the tape does not come off inside

your drive. As long as you have good disks and are not trying to

format a disk, the PC is perfectly happy if you cover the index

hole. However some newer types of floppy disk drive will not let

you use the "Cover the Index Hole Method". They have sophisticated

electronic circuitry which times the intervals between index

pulses and, if the interval is not 200 milliseconds +/- 3% then

the drive says "No disk in drive", it then sets the ready line low

and blocks transfer of data. With this type of drive, if you cover

the index hole on your disk, PCXZ will then display a message

which says "No Disk in Drive or Drive Door Open". In this case you

must use one of the alternative methods below.

2) Change the option jumpers on the disk drive electronics board. You

need a User's Guide for this. Look for settings that allow the

drive to work with hard sectored disks. If you are successful you

will now be able to cover the index hole and read/write your disk.

3) Change to a different make or model of disk drive. Drives from

Qume, Shugart and Tandon are suggested. Do not use the TEAC 55

series.

4) Use a TRS80 disk formatted under PCXZ for file transfer. Disks

made this way do not need the index hole covered. You cannot make

disks with a single density track 0 for Model I's; instead you

must format a Model III type disk. This method has the

disadvantage that you must still have your TRS80 and you have to

make two transfers.

5) Use a modified disk drive cable. Insert a switch in the line

carrying the index hole signal from the drives to the disk

controller. Whenever you want to read TRS-80 disks, open the

switch. At all other times close the switch. This method is the

best and works regardless of the type of drive you have and does

not require you to cover the index hole on your disks. Overleaf a

diagram shows how to make or modify your own cable. Note that some

computers (e.g. Tandy Model 1000) do not twist the cable between

the two drive A and B connectors.  We can supply adapter cables

with a built-in switch on an extension lead ready made for $15 +

$2 s/h. Please specify the make and model of computer, and if

possible what type of connector your machine uses at the disk

controller end. It will be either a 34 pin card edge connector or

a 34 pin header connector (2 parallel rows of 17 pins spaced 0.1"

apart).  We can also supply a special switched adapter cable for

external disk drives using the external DB37 connector available

on some computers. Price - $18 + $2 s/h.

Note: The Index Hole Cover or Adapter Cable does not work with other

TRS80 file transfer programs

Copyright 1987, 1994 Hypersoft