INSTRUCTIONS for HYPERCROSS III

CP/M and PC FILE TRANSFER for

the TRS-80 and MAX-80

V3.0A last revised December 30th 1986

Copyright 1985 M.J.Gingell, Hypersoft Raleigh NC USA

Notice

Hypersoft and the  Author offer  this program  on  an as-is  basis only  and

assume no responsibilty for any  loss or consequential damage resulting from

its use or misuse. If you  have a problem, please submit  a report using the

standard form provided at the end of  this manual and we will exert our best

efforts to resolve it for you.  You must be the  original purchaser and have

submitted a  registration form if  you did  not buy it  directly from us. In

addition we will be pleased  to add additional formats  to the repertoire of

this program.  For this  we need  a sample  disk of  the format in question,

properly formatted with a few arbitary files placed on the disk to establish

the sector  interleave sequence  and other  parameters. Please  use the form

provided and submit it with the disk and specified remittance.

Program Overview

This manual describes one program that  comes in several different versions.

It is designed  to allow you  to move data  and program files freely between

your own TRS-80 or MAX-80  disks and those  of a number  of other computers.

You  do  this using  only  the hardware  in  your TRS-80  or  MAX-80 and the

supplied program thus eliminating the need for modems, terminal programs and

access to a second non TRS-80 computer.

The program runs as  a /CMD  file under  your own  TRS-80 DOS  and will

allow you to COPY files  to and from the foreign  DOS, and also to DIR, KILL

and FORMAT. You can therefore format a blank  disk and move files to it that

can subsequently be read  by, for example, a Kaypro  or an IBM PC. Note that

you cannot run  Z80 machine  language programs  on, for  example, an  IBM PC

which uses an 8088  microprocessor. The same is  true in reverse except that

many CP/M computers use  8080 or Z80  microprocessors and the  TRS-80 uses a

Z80  which  is  compatible  with  8080  code.  CP/M  code  would  need  some

modification before it could be  used on a TRS-80,  particularly with regard

to I/O.

You can however copy  a BASIC, PASCAL,  FORTRAN, C or  other high level

language program  and run it  with very little  modification unless it makes

extensive use of  memory  mapped graphics  or  has system  machine  language

calls. You can even  move an assembly language  source file to a new machine

where  you  can  translate  it  to  be  compatible  with  the  code  of  the

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page   2

microprocessor using one of the  translator programs now available. Data and

text files of  many  types can  also  be  usefully moved  between  machines.

Medical records for  example, could be  prepared using a  TRS-80 and sent to

another department which uses  IBM PCs. Manuscripts  could be prepared  on a

TRS-80 and  sent to  a publisher  who only  has Sanyo  CP/M computers in his

office. Machine code for a 68000 CP/M computer  could be generated on a TRS-

80 using our XAS68K  cross assembler and then put  on a disk readable by the

target machine.

The program comes in versions for 4 machines: SX1/CMD, SX3/CMD, SX4/CMD

and SMAX/CMD for TRS-80  Models I,  III, 4/4P  and the  MAX-80 respectively.

SX1/CMD  will also  run on  the Genie  I and  the LNW80  I and  II. With the

program itself is a database file HPRX/DAT which includes definitions of the

formats  of  the alien  disks   (NOTE:  on  the MAX-80  this  file is called

HXMX/DAT).  HPRX/DAT  will  be  different  depending  on  whether  you  have

purchased the PC only, CP/M only, or combined CP/M and PC version. The model

III version will  run on  a  Model 4    or 4P  in model  III  mode under  an

appropriate model III  DOS. Except for  this, each version  will only run on

the model specified provided with  the minimum hardware as  given in Section

1.

Also included on your  disk is a  BASIC program CFGMAKE/BAS,  to create

customized configuration files for  Hypercross. This makes a custom tailored

configuration file called SXCFG0/DAT which tells Hypercross what the default

settings should be at  startup. It even allows  you to start with predefined

alien formats.

You will need a  TRS-80 type DOS system  disk in logical drive 0 except

when doing alien to alien file copying (XFER). With LDOS, DOSPLUS and TRSDOS

6.x you can redefine logical drive 0 to be any physical drive, hard disk, or

RAM disk. Apart from that you will be  able to transfer files between a TRS-

80 disk in any drive and an  alien disk in any drive except logical drive 0.

Hypercross uses your own  DOS to  handle transfers to  and from  your TRS-80

disks and its own internal routines when accessing the alien disk. Therefore

if your system is set  up to use double  sided, 80 track, RAM  or hard disks

then these features should continue to  work. The only restriction is if you

have program drivers in high  memory. Hypercross III honors  the high memory

limit set by your  DOS. If you use high  memory then the available space for

the file transfer buffer will be decreased and copying speed will be reduced

because it will be done in more, smaller blocks.

TRS-80  is a registered trademark of Tandy Corp

CP/M    "  "    "          "      "  Digital Research, Inc.

Z80     "  "    "          "      "  Zilog

MAX-80  "  "    "          "      "  Lobo Systems

MS-DOS  "  "    "          "      "  Microsoft Corp.

IBM PC  "  "    "          "      "  International Business Machines

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 1                Getting Started                          Page   3

1.0 Getting Started

To run the program  you will need the  following hardware as a minimum:

TRS-80 Model I, III or 4/4P with 48k of memory (64k model 4/4P) and two disk

drives. Model 1 users also need  a doubler and a DOS which should preferably

be double density although this is not  essential. The program tests for the

type of Model  I doubler in  use if any  and adapts itself accordingly. This

includes the doubler made by Radio Shack.

Some users have special driver routines in high memory. Hypercross will

automatically check the high memory vector and  adjust for it. Provided your

high drivers do not exceed 2048 bytes (HIMEM is greater than F7FFH) then you

will have 8192 bytes for your  transfer buffer. If your  high memory is less

than F800H then you  will be allowed only  4096 bytes and copying speed will

be reduced. If your  high memory limit  is below E800H  then Hypercross will

not run at all.

The model I version also runs on the  LNW80 models 1 and 2 provided the

clock speed is set at standard 1.77 MHz.

Among the formats you  can read  and write  to are  a number  of double

sided types. For this you will need  a double sided drive as drive number 1,

2 or 3 (or 0  with some DOSs).  On Model I's  drive 3 is  not permitted with

double sided drives as  the drive 3 select  line is used for side selection.

Similarly to use  the 80  track CP/M  formats you  need an  80 track  drive,

double sided if appropriate.

If you have the  capability of  reading 8  inch disks,  i.e. an  8 inch

drive and a disk controller to match you  may also be able to use the 8 inch

standard CP/M ss sd option. A suggested controller for the Model III or 4/4P

is the DX3D  from Holmes Engineering in  Salt Lake City.  Owners of  a newer

Model 4 or a 4P will not be able  to take advantage of this because the disk

controller is integrated on the main board and you cannot replace it with an

8 inch capable  one. The  4P has  always  had an  integrated controller  and

cannot have external or 8 inch drives.

#-1TRS-80 DOS formats#-0

The following TRS-80 disk operating systems are supported:

Model I / LNW80:

LDOS               Single/Double density

NewDos/80 V 2.0    Single/Double density

DOSPlus 3.4,3.5    Single/Double density

MultiDos 1.6       Single/Double density

TRSDOS 2.3         Single density only.  *

TRSDOS 2.7 or 2.8  Double density only.

* Note DIR and TAG of TRS-DOS not available.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page   4                   Getting Started                     Section 1

Model III:

DosPlus 3.4,3.5

LDOS

MultiDos 1.6 and earlier

NewDos/80 V 2.0

TRSDOS 1.3

Model 4 and 4P

All above Model III DOS's in III mode

TRSDOS 6.x

DosPlus IV

MultiDos 1.7 64/80 not supported.

MAX-80:

LDOS

The program is supplied as a command  file (/CMD) on a data disk and to

run it you will need to move it to your system disk. This will usually be in

drive 0  but, in some  cases can  be in another  drive. Assuming your system

disk is drive 0, insert your DOS disk  in that drive and reset the computer.

Insert your Hypercross disk in drive 1 and type:

For the Model I: (35 track sd data disk)

COPY SX1/CMD:1 :0

COPY HPRX/DAT:1 :0

With NewDOS/80 you will have to set the PDRIVE for drive 1

appropriately (also applies to Model III NewDos).

For the Model III: (35 track sd data disk)

With TRSDOS 1.3 use the TRSDOS CONVERT utility thus:

CONVERT  (you will be prompted for drive numbers)

For other DOSs on the Model III:

COPY SX3/CMD:1 :0

COPY HPRX/DAT:1 :0

For the Model 4/4P: (40 track dd data disk)

COPY SX4/CMD:1 :0            COPY HPRX/DAT:1 :0

For the MAX-80: (40 track dd data disk)

COPY SMAX/CMD:1 :0            COPY HXMX/DAT:1 :0

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 1                     Getting Started                      Page   5

In  case  of difficulty  with  the  master files,  backups  of each are

provided as SXn/BAK and HPRX/BAK. Note that on  the model 1 the program will

be supplied on a single density TRSDOS  2.3 35 track data disk, on model III

it will be on a single density 35 track data disk and on the 4/4P it will be

on a 40 track Model 4 double density data disk. For the MAX-80 it will be on

a 40 track double density data disk readable by MAX-80 LDOS.

On some  systems and  some DOSs it  is possible  to redefine the system

disk as another drive. This is true for LDOS  on the Model III and TRSDOS on

the Model 4/4P.  For instance, suppose you  have a hard  disk as drive 5 you

can redefine it with the command:

SYSTEM (SYSTEM=5)

This swaps drive 5 and drive 0 logically and then will make all system calls

to drive '0'  which is now  the hard disk.  If you wish  to do this then you

should place a copy of Hypercross and the format data file on whatever drive

you have selected as your system drive.

To run the program, type  the filename - excluding  the /CMD i.e. type,

for instance:

SX1

The program will load and run and its first action will be to try and load a

configuration file called SXCFG0/DAT. This file, if found will automatically

configure Hypercross for  your DOS type,  your disk drive configuration, two

predetermined alien disk formats and some other default settings.

If the configuration file can't  be found then Hypercross will ask what

DOS you are running and what  density (model I only).  The program will then

offer you a menu of formats. You  must choose one at this point although you

may change it later using the Change Configuration option.

Hypercross maintains a table of your drives showing how many tracks are

on each drive and  which of two  alien formats  is allocated to  that drive.

Once Hypercross  III is running,  you may place  a disk in  a drive and copy

files to and from it, DIR it,  FORMAT it and KILL files  on it. You can copy

alien to TRS-80, TRS-80 to alien, and alien to alien. If you specify a drive

by number (0-7) then Hypercross knows it is a TRS-80 type disk and relies on

your DOS  to know what  kind of drive  it is (density,  sides, no of tracks,

size etc.). If you use a  letter for the drive  number (A-D corresponding to

physical drives 0-3) then Hypercross knows it is an alien disk in that drive

and goes to its own drive table to see which alien format is to be used.

When  accessing  TRS-80  disks  (copying  to  or  from  them  getting a

directory etc.,) you MUST retain a  disk in the system  drive with a minimum

operating  system. The  selected alien  formatted disk  can be  in any other

drive but the system drive (logical drive 0).

Note that if you put  a disk in a drive  and then you try and access it

and it is not in the expected format it may hang the system. This is true if

the disk is  TRS-80, CP/M or PC.  In general it  should not hurt anything if

this happens as the first operation  is to access the  directory of the disk

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page   6                      Getting Started                      Section 1

which is a read  operation. At that point, if  the directory is on the wrong

track or the  correct number  and type  of sectors  are not  found then  the

program will  either abort  or hang,  trying to  find a  sector that doesn't

exist.

Hypercross III  has a  special feature  which checks  the format of the

disk first to see if it is the expected  one. If not it will not proceed. If

you are trying to read a PC format  disk it will tell you that the format is

incorrect and what  it most  probably is.  A common  mistake is  to try  and

access a double sided  format disk in a single  sided drive or with a single

sided format selected. The check  procedure is not  infallible however. With

CP/M disks it checks to  see if the sector size  is correct and if there are

sufficient sectors on  a track.  Because  of the  large  number of  possible

formats  there  is  no  sure  way  of  automatically  determining  a formats

correctness.

Type HELP or  H to see  a summary of  available commands. These will be

described in more details in Chapter 3. For the most part they must be typed

exactly as shown although you may insert extra spaces between words.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 2                      The Main Menu                       Page   7

2.0 The Main Menu

When Hypercross III  has loaded and  initialized itself it will display

the main menu screen. The menu screen looks like this:

TRS80 <--> PC & CPM    HyperCross V 2.0A       Copyright 1985

Hypersoft, PO Box 51155, Raleigh NC, USA   (919) 847 4779

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Drive - B C D is format F1  Kaypro II       40 SS DD  USER1 0

Drive A - - - is format F2  PC/MS DOS 1.1   40 SS DD

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Enter command or select from menu:

DA  Directory of alien disk     TA  tag alien default disk

DT  Directory of TRS-80 disk    TT  tag TRS-80 default disk

T   Copy TO alien from TRS-80   F   Copy FROM alien to TRS-80

FA  Format default alien drive  C   Set configuration

H   Help                        X   Exit to DOS

>>

Below the heading,separated by  dividing lines,  are two  lines showing

the two currently  selected alien formats,  F1 and F2  and which drives they

are allocated to. You can copy between your TRS-80 disks and either of these

two alien formats or directly alien to alien.

Below the format lines is a menu of 10 quick command options and a '>>'

prompt where Hypercross  is waiting for  you to type  something. If you type

anything but a valid command, Hypercross will repaint the screen.

At the >>  prompt you can either  type a command  or a quick entry from

the menu. The  commands are  fully typed  commands just  like DOS  commands.

These are described in detail in the next chapter.

#-1The Default Drives and changing the configuration#-0

Before you can use these commands you must understand about the default

drives and formats allocated  to them. Any  drive can contain  an alien disk

and any drive can contain a TRS-80 disk. Hypercross normally knows what kind

of disk is in a drive because  when you type a command  you must use a digit

(0 to 7) to signify the  drive if it is a TRS-80  disk and a letter (A to D,

corresponding to 0 to  3) if  it is an  alien disk.  However when you  use a

quick menu command, Hypercross  goes to a table  and looks up which drive is

the 'default' TRS-80 drive and  which is the 'default'  alien drive and uses

them in any operation.

To  see  which  drives  are   the  default  drives,  go   to  the  'Set

configuration' page by  pressing C and  <enter>. You will  see a screen like

this:

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page   8                       The Main Menu                      Section 2

Drive - B C D is format F1  Kaypro II       40 SS DD  USER1 0

Drive A - - - is format F2  PC/MS DOS 1.1   40 SS DD

Drive Tracks Format Skip Default

A    0    40    2    N    TRS80

B    1    40    1    N    Alien

C    2    40    1    N

D    3    80    1    Y

Choose from one of the following:

A,B,C or D Set drive config.      X  Exit to command level

1  Select new format F1           3  Select default TRS80 drive

2  Select new format F2           4  Select default alien drive

>

The first two lines are the same as shown on the main menu and tell you

the two alien formats currently selected. Next  is a table showing, for each

of your four floppy drives, A  to D (corresponding to physical drive numbers

0 to 3),  how  many tracks  are  on  each drive  and  what alien  format  is

allocated to that drive.  If you select drive  C, in the example shown, this

would be physical drive 2 which has 40  tracks and is allocated to format F1

which is KAYPRO II  (40 track, single sided,  double density). The skip flag

is N for No skip.  If you had  selected the 80  track drive D  then the skip

flag would be Y for Yes skip  because you have selected a 40 track format in

an 80 track drive.

In the Default column  of the drive table  you will see 'TRS80' against

the A (0) drive  and 'Alien' against  the B  (1) drive. This  indicates that

whenever you  use the  quick menu commands  the default  TRS80 drive will be

logical drive 0 and the default  alien drive will be  physical drive 1. Note

that logical drive 0 means the drive  your TRS-80 type DOS thinks is drive 0

from its own internal  drive table.  With some DOSs  you can  reassign drive

numbers, even assigning a RAM disk or hard disk as drive 0.

You can redefine the default drives, the selected formats and the drive

format allocations all  from the short  menu shown at  the bottom of the Set

Configuration screen. To return to the main menu type X.

Note  that all  the  parameters in  the  set configuration  menu can be

predefined in a configuration file SXCFG0/DAT  which is loaded automatically

when Hypercross  runs. You  use the BASIC  program supplied  on your disk to

make a configuration file. See Chapter 6 for details on running this program

and making your own configuration files.

The Quick Menu

At the bottom of the main  menu screen is the quick menu. The following

items are shown in the quick menu:

DA  Directory of Alien disk     TA  Tag Alien default disk

DT  Directory of TRS-80 disk    TT  Tag TRS-80 default disk

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 2                      The Main Menu                        Page   9

T   Copy TO alien from TRS-80   F   Copy FROM alien to TRS-80

FA  Format Default alien drive  C   Set Configuration

H   Help                        X   Exit to DOS

By typing the code for the item desired  and pressing the enter key the

option  selected  is  executed immediately.  The  default  drives are always

assumed. If you want to change the default go to change configuration option

C.

Most of the quick menu commands are short versions of the full commands

shown in  the next chapter.  Here is  a brief description  of each one. Note

that the equivalences assume, for the sake of example, that the default TRS-

80 drive is 0 and the default alien drive is B (:1).

DA   Directory of Alien disk

Equivalent to:      DIR :B

DT   Directory of TRS-80 disk

Equivalent to:      DIR :0

T    Copy TO alien from TRS-80

Equivalent to:      COPY filename:B filename:0 -switch

- you will be asked for the filename and switch

FA   Format default Alien drive

Equivalent to:      FORMAT :B

H    Help

Displays help page.

TA   Tag Alien default disk

Equivalent to:      TAG :B

TT   Tag TRS-80 default disk

Equivalent to:      TAG :0

F    Copy FROM alien to TRS-80

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  10                       The Main Menu                      Section 2

Equivalent to:      COPY filename:0 filename:B -switch

- you will be asked for the filename and switch

C    Set Configuration

Displays Set Configuration Screen at which time

you can enter a sub command to change the drive

specifications, the default drives and the

selected alien formats. Sub Commands are:

A,B,C or D - set drive specification for drive 0,1,2,

or 3 respectively. Allows you to redefine the number

tracks and allocate alien format F1 or F2 to the

drive.

1  Select new format F1 - allows you to change the

alien format F1.

2  Select new format F2 - allows you to change the

alien format F2.

3  Select default TRS-80 drive - allows you to

redefine which drive will be used as the default

TRS-80 drive in the quick menu commands.

4  Select default alien drive - allows you to

redefine which drive will be used as the default

alien drive in the quick menu commands.

X  Exit the configuration set up screen and return

to the main menu screen.

X  Exit to DOS

Causes Hypercross to stop and returns control to

DOS. Make sure you have a system disk in drive 0.

The Program Commands

The program commands are described in  the next chapter. You can type a

command any time you see the  '>>' prompt. If you mistype  a command or just

hit the enter key the main menu screen will be redisplayed.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 3                     Program Commands                      Page  11

3.0 Program Commands

This  section gives  detailed  information on  the  use of  each of the

available commands in your program. In each command, if the disk is a TRS-80

disk you must use a number from 0 to  7 for the drive. If the disk is a CP/M

or PC disk you must use a letter for the drive. Thus drive 1 would be called

drive B, drive 2 drive C and so on. This is how the program knows whether to

expect a TRS-80 or alien  disk in any  drive at any time.  Don't forget that

drive 0 must contain a DOS system at all times if you are executing commands

which access TRS-80 type disks.  With some DOSs  drive 0 can  be assigned to

any physical drive, even  a RAM based disk  emulator such as MEMDISK. If you

do this then you  must refer  to the  TRS-80 DOS disk  by the  logical drive

number that the DOS  thinks it is in  and to the alien  disk by a letter A-D

corresponding to the true physical location of the drive.

DIR

This command allows you to  view the directory  of one of  the disks in

your drives. Note you must have selected the correct format before you start

or the program may hang trying to find sectors that don't exist on the disk.

When you try and  read the directory of  an alien disk, Hypercross will test

to see if the disk format  matches the one specified for  the drive that the

disk is in. If it does not it will give an error message. In the case of MS-

DOS formats the program will tell you what the format actually is so you can

reselect the correct one.

Example:

DIR :1

Explanation:

Read the directory from the TRS-80 disk in drive 1.

Requires a system disk in drive 0 always.

Note: the colon (:) is mandatory.

Example:

DIR :B

Explanation:

Read the directory from the CP/M or PC disk in

drive 1. The format assumed will be the one

specified in the drive table.

COPY

This command allows you to  transfer files between your own TRS-80 type

DOS disks and an alien DOS disk. The  direction of transfer is determined by

the  sequence  of  drives.  The  source  drive  is  defined  first  and  the

destination second. The drive containing your TRS-80 DOS disk is referred to

by number (0-7)  and the drive  containing the alien  disk is referred to by

letter.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  12                      Program Commands                     Section 3

Four types of copy are possible.  ASCII for text files, BASIC for Basic

files, IMAGE for most other kinds of file,  and for fixed record length TRS-

80 data files the -R transfer. With ASCII transfer the file is assumed to be

an ASCII (text)  file and  is converted  so that  it reads  properly on  the

target   machine.  This   includes  fixing   the  carriage-return  line-feed

differences, the end of  file mark and  the non standard  TRS-80 characters.

Use this for files created with a text editor, BASIC files saved with the -A

ASCII option and assembly language source files.

The BASIC transfer option is available for copying from TRS-80 disks to

CP/M or MS-DOS  disks only.  It  allows you  to copy  Basic  files saved  in

standard  (tokenized) format  directly form  your TRS-80  disk to  a CP/M or

MS/PC-DOS disk converting the  file automatically  while it is  copied. With

this  option it  is not  neccessary  to convert  the Basic  program to ASCII

first. As the file  is copied  over, spaces  are inserted  where neccessary,

THENS are  inserted if  needed, PRINT  @ is  converted to  LOCATE X,Y.  Some

expressions or tokes that  would cause trouble  are flagged by  inserting an

exclamation  ! mark.  See the  Appendix on  converting BASIC  files for more

details.

With IMAGE  transfer the  file is  copied over  exactly as  is, with no

changes whatsoever.   Use this  for binary  files and  data files  including

special record length ASCII files. Also use this mode if in doubt.

The -R transfer is like IMAGE  transfer except that it  is for files of

fixed record  length, not  the normal  256 byte  logical record length (LRL)

TRS-80 records but  some  other  lesser length.  To  use  it type  -R80  for

instance where the LRL  of the file to be  transfered is 80 bytes. This mode

is only useable going from TRS-80 to alien DOS.

You can type the  command in full together  with a valid switch option.

If you omit the switch the  currently selected default will  be used. If you

omit the destination file  name the source name will  be used but with a '/'

substituted for a '.' or a '.' for a '/' as appropriate.

A switch option tells Hypercross which  copy mode to use on the file as

it is copied. If you  type -I then  an exact image is  copied across without

any changes. If you  type -A then the file  will be transfered in ASCII mode

with appropriate corrections for  differences in  ASCII file formats  on the

different computers. If you type -B then the file will be treated as a Basic

file and converted  according to  the rules  of the  source and  destination

types of  Basic. If you  type -R  followed immediately by  a number then the

file will be transferred  in IMAGE  mode but  will be  opened on  the TRS-80

source disk as a file where  the LRL=number. The number may be anywhere from

1 to 255.

Note that if  you want to  move BASIC programs  then you can either use

the -B transfer option or the -A option. If  you use the -A option (You must

use this if going to or from CoCo Basic or if copying FROM CP/M or MS-DOS to

TRS-80) then the file must be saved in ASCII format first. On the TRS-80 for

instance, you do this by, from BASIC, typing SAVE "filename",A. If you don't

do this then some some of your programs may not be readable as when BASIC is

saved to disk in the ordinary  way, all the commonly used BASIC keywords are

stored as one byte codes called tokens. These  are different on CP/M, PC and

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 3                     Program Commands                      Page  13

TRS-80 computers even though Microsoft probably wrote them all!.

If you are copying files to IBM PC type disks you will probably want to

ensure the date gets  set correctly. Some TRS-80  Dos's allow you to boot up

without setting the date so  set it first  before you run  this program. For

example  in most  DOSs you  simply type  DATE 03/30/84  to set  the DOS date

register to 03/30/84.

If you are  working with CP/M  disks then the  default USER number is 0

unless you select another value. See the  USER command below. Also note that

you can  only specify filenames  that use legal  ASCII characters. Some CP/M

systems use the high bit of  the letters of the  extension (for instance the

COM in BASIC.COM) to signify some special parameter. These are masked off by

the directory read so that they are printable on the screen.

Certain  characters are  legal  in CP/M  file  names but  not in TRS-80

names. Copying from TRS-80  to CP/M or  MS-DOS is  no problem but  going the

other way  you must  observe the following  rules: Only  the letters A-Z and

numbers 0-9 are allowed  in TRS-80  names. The name  and any  extension must

each start with a  letter. You cannot have  embedded hyphens or slashes. For

instance SIG/M.DIR is not permitted.  If a slash  occurs in a  name you must

precede it with a ! escape character.

Example:

COPY  TEXT/DOC:0 TEXT.DOC:B -A

Explanation:

Copy the file TEXT/DOC from TRS-80 DOS in drive 0

to the alien DOS in drive 1 using ASCII transfer

mode. The same name is used.

Example:

COPY  TEXT.DOC:B TEXT/DOC.XYZ:0 -A

Explanation:

Copy the file TEXT.DOC on the alien disk in drive 1

to the TRS-80 DOS disk in drive 0 using ASCII mode

and adding the password XYZ.

Example:

COPY  MYFILE/TXT:0 :B

Explanation:

Copy the file MYFILE/DOC from TRS-80 DOS in drive 0

to the alien DOS in drive 1 using the default transfer

switch. The destination name will be MYFILE.TXT

Example:

COPY  SIG /M.COM:C SIGM/CMD:1 -I

Explanation:

Copy the machine language program SIG/M.COM from the

the disk in drive 2 to the disk in drive 1 making an

an exact image copy. If it runs without changes it

will be a miracle!. Notice the ! escape character.

Example:

COPY  MEDIC/DAT:0 MEDIC.DAT:B -R80

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  14                      Program Commands                     Section 3

Explanation:

Copy the file MEDIC/DAT from TRS-80 dos in drive 0

to the alien DOS in drive 1 using IMAGE transfer

mode but open the source file on the TRS-80 disk

using a Logical Record Length of 80.

Note that in the  quick menu  there are two  options for  quick copying

between a TRSDOS  disk an alien disk  in drive B. The  same name is used for

source and destination file. Just type  3 to transfer to the  alien DOS or 8

to transfer from the DOS and  follow the prompts. Just give the filename, no

drive numbers. The  drives used  will be  the  defaults shown  in the  drive

table.

ERA or ERASE or KILL or REMOVE

This  command  is  allowed  in  four  forms  corresponding  to the same

functions on CP/M, PC's and TRS-80's respectively. Again, use a number for a

drive containing a TRS-80 disk and a letter for a drive with an alien disk.

Example:

ERASE  BIGONE.COM:C

Explanation:

Delete the file called BIGONE.COM from the alien disk

in drive 2.

Example:

KILL  MYFILE/BAS:0

Explanation:

Delete the file MYFILE/BAS from the TRS-80 DOS disk in

in drive 0.

FORMAT

This command  allows you  to put  a blank  disk in  drive A,  B, C or D

(physical drives 0, 1, 2 or 3 repectively) and  format it so that it will be

readable on a non  TRS-80 computer. Drive 3 is  not permitted on Model Is if

you have double sided drives.

On the command  FORMAT <drive no.>  the computer will  prompt you for a

disk in the selected  drive and remind  you of the  currently selected alien

format for  that drive.  After each track  is formatted  it will be verified

before proceeding. If an  error is found  you will be  asked if you  want to

Retry, Skip or Abort. Type the  appropriate letter to select your choice. If

you skip over the  error i.e. ignore  it then  this information will  NOT be

recorded anywhere on the disk and  you may subsequently get errors on a file

that spans the  bad  area.  In  general  it is  unwise  to  use  disks  that

consistently exhibit bad formatting characteristics.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 3                     Program Commands                     Page  15

Note that FORMAT does  not duplicate the  code often found  in the boot

sectors of  PC or  CP/M disks as  this might  infringe copyright. Because of

this some  disk  checking  programs  may  indicate  that  the  disk  is  not

initialized. However this is not a serious problem as the disks can still be

accessed and files copied on  the PC or  CP/M machine in the  normal way. In

general it is probably  safest to use disks  formatted by Hypercross as data

disks only. This is  particularly true for  CP/M where some  machines format

the system tracks differently from the user area.

A few computers use checking of the  boot sector to determine what kind

of disk  is inserted. If  this is the  case then they  may reject disks made

using Hypercross. In that case it  is suggested you will  have to format the

disk on the target machine and then use Hypercross to transfer the files.

Hypercross does NOT  check  to  see  if  the  target  disk  is  already

formatted. If it is then the  original data will be destroyed. In general it

is good practice to use new  blank or bulk erased  disks to prevent residual

data  from  previous  use  interfering  with  the  new  recording.  This  is

particularly important when using disks in an 80 track drive with a 40 track

format selected.

Example:

FORMAT :B

Explanation:

Format the disk in physical drive 1.

HELP

This command displays a short summary of commands as an aid to memory.

Example:

HELP

Explanation:

Display help page.

LOAD

This command allows you to load a pre-existing file from a TRS-80 disk.

The file is in ASCII format  and contains a tagged  file listing as produced

by the  'S' subcommand  of the TAG  command. This  is useful  if you want to

repeatedly copy the same list of files.

Example:

LOAD TAGLIST/DAT:0

Explanation:

Load the tagged list of files from the TRS-80

disk in drive 0 and go to the TAG display

mode.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  16                      Program Commands                     Section 3

R

This command returns  you to  the tagged  display mode  if you  wish to

repeat the copying of  a tagged list or  if you inadvertently exited the TAG

mode using the X sub-command.

Example:

TAG :B

X

R

Explanation:

Tag the list of files in drive B, exit to the

main menu (X) and return to the tagged list (R).

SELECT1 and SELECT2

These commands allow you to directly select a new alien format from the

main menu without going through the set configuration option. SELECT1 sets a

new Format F1 while SELECT2 sets a new F2. You may type the command alone in

which case the  menu  will be  displayed  or you  can  type a  number-letter

combination specifying the menu page and  item on that page. This command is

useful if you want to run Hypercross from within a DO file.

Example:

SELECT1 3B

Explanation:

Replace the current format F1 with format 3B

(Item B on page 3 of the format menu).

TAG

The TAG command allows you to  read a disk directory into memory and to

tag or mark all the  files you want  to copy or  delete and then  to copy or

delete them all  in one go. The  TAG command puts you  into the tag mode and

displays a menu of subcommands which allow you to do various things with the

directory of the disk  you have just read.  After typing the TAG command the

directory of the specified disk will be read and you will see the following:

Tag Commands:

-----------------------------------------------------------

A     Tag for ASCII copy         I     Tag for image copy

Rlrl  Tag Random, lrl<256        */ext-sw Tag all /ext files

U     Untag (remove tag)         *U    Remove all tags

K     Kill file on source        S     Save tag list in file

:D    Copy files to D or Kill    -     Step back one entry

?     Display this list          X     Exit (no action)

------------------------------------------------------------

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Files on source drive :B user number 0

File        Tag  User

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 3                     Program Commands                      Page  17

filename.ext      0-->

Each time you hit the enter key the  next file in the directory will be

listed, together with any existing tags  and the '-->' prompt, until you get

to the end  when it  will  display the  first filename  again.  If you  have

selected a drive with a disk in  a CP/M format you will be shown the current

user number selected for that format (see  the USER1 and USER2 commands) and

against each filename you will see  the user number for that file. Remember,

you can only copy  or delete  files which  have a  user number  matching the

currently selected one.

At the '-->' prompt you can type a  tag command from those shown in the

tag menu. This allows you to tag the files you want to copy or delete and to

issue other commands described in detail below.

Example:

TAG  :B

Explanation:

Get directory of alien disk in drive B

The TAG subcommands

At the --> prompt you  can type any of  the following subcommands. They

will be executed immediately without waiting for the enter key to be struck.

A     Tag for ASCII copy

At the --> prompt, simply type an A and the filename will be tagged for

copy in ASCII mode. If you  step back using the '-'  command and look at the

file name again you will see the tag:

filename.ext  A   -->

I     Tag for image copy

This puts an 'I' against the filename marking it for image copy mode.

Rlrl  Tag Random, lrl<256

This allows you to tag  the file for  copy as one  whose logical record

length (LRL) is not 256. Example: R80 would indicate that the file has a LRL

of 80. This applies  only to files  on TRS-80  disks. CP/M and  MS-DOS disks

have no way of recording the LRL in the directory.

*/ext-sw Tag all files having extension 'ext' with tag 'sw'

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  18                      Program Commands                     Section 3

This allows  you to tag  all the  files in the  directory that have the

same extension. For example */BAS-I  would tag all the  files which have the

extension /BAS with the I tag. As  in regular tagging 'sw' is a switch which

can be A, I, or Rlrl (where lrl is a number from 1 to 255).

U     Untag (remove tag)

This allows you to  remove the  tag from  the filename  currently being

prompted.

*U    Remove all tags

This removes all tags from all files.

K     Kill file on source

This marks the file with a 'K'  tag which indicates that the file is to

be killed #-1from the source directory#-0 when you give the :drive command.

S     Save tag list in file

This allows you to save the names of all the tagged files in a file for

later recall  and re-use. You  can only save  to a TRS-80  type disk and you

must have one in drive 0 when you give this command. You will be asked for a

filename to save into. See the 'LOAD' command to get the filename list back.

:D    Copy files to D or Kill

When you issue the :D command you must  put a figure 0 to 7 or letter A

to D in place of the 'D'. This  indicates the destination drive for copying.

Thus  if  you had  tagged  files on  drive  B  and then  given  a :0 command

Hypercross would start executing a  copy of all tagged  files from the alien

disk in drive B to  the TRS-80 disk in drive  0. If it encounters any K tags

these will be used to remove files from the source disk (B in this example).

If you had typed :A instead  then Hypercross would execute an alien to alien

file transfer (XFER) from  the alien disk  in drive B  to the alien  disk in

drive A. No TRS-80 system disk is needed while this is happening.

-     Step back one entry

At any time  you can step  backwards through the  file list by pressing

the '-' key. To step forwards use the Enter key.

?     Display this tag menu list.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 3                     Program Commands                      Page  19

At any time,  pressing  the  '?'  key  will reprint  the  menu  of  TAG

commands. You will also restart at the top of the file list.

X     Exit (no action)

The 'X' command returns control to  the main menu without any action on

the tagged list. The list is still in  memory unless you execute another TAG

command and you can return to it with the 'R' command.

---------------------------------------------------

USER1 and USER2

This command, only available in  CP/M allows you to  change a file flag

called USER1 or  USER2. At any  time you will  only be able  to copy or kill

files for the USER number  you are currently set  to. Permissible values for

user are 0 to  15. The default value  is 0 and most  files will have this as

their attribute. The  current user  number  is always  shown  when you  have

selected a CP/M format. You will  see it shown as a single hexadecimal digit

to the right of the selected format on the main command screen.

USER1 specifies the user number in force when accessing any drive which

is allocated to format F1. Similarly USER2 is for format F2. This allows you

to  allocate  different  user  numbers  for  two  drives  and  to copy files

allocated to one user to another user.

Note that under CP/M a DIR  will only show the files for the particular

USER setting you are  using. In  this program #-1all#-0  files are  shown together

with their USER parameter so you  can select which ones you want to transfer

or delete.

Example:

>>USER1 7

>>USER2 3

Explanation:

Set USER1 atribute to 7 and USER2 to 3

the result will be displayed thus:

Drive - B C D is format F1  Kaypro II    40 SS DD   USER1 7

Drive A - - - is format F2  Kaypro II    40 SS DD   USER2 3

In this case both format  F1 and F2 happen to  be the same but the user

numbers are different. This would allow you to copy files from a Kaypro disk

in drive A with user # 3 to a Kaypro disk in drive B with user # 7.

XFER - alien to alien file copy

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  20                      Program Commands                     Section 3

This command is  similar to the  COPY command except  it is for copying

files from one alien format to another. A  switch is not neccessary and will

be  ignored. All  files are  copied in  -I mode  - that  is exactly, without

modification. If XFER  is called  from the  TAG command  then files  will be

copied -I whether they are tagged with an A or an I.

Hypercross will use  the  drive  table  to  determine  the  source  and

destination formats. They may be the same if you wish. If the format is CP/M

then the user number will be  that set by USER1 for  format F1 and USER2 for

format F2.

Example:       Let us assume that drive A has been assigned

format F2 with user number 1 and drive B to

format F1 with user number 0

XFER  OLDNAME.XYZ:B NEWNAME.ABC:A

Explanation:

Copy the file called OLDNAME.XYZ from the disk

in drive B with format F1, user number 0 to

the disk in drive A, format F2, user number 1,

renaming the file NEWNAME.ABC

Example:

XFER  MYFILE.DOC:A :C

Explanation:

copy the file called MYFILE.DOC on drive A

to drive C using the same name MYFILE.DOC

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 4                  Technical Information                    Page  21

4 Technical Information

Bytes, Sectors, Grans, Blocks and Clusters

File sizes are  described  by different  disk  operating systems  using

different units of measurement. Among the more important to us are:

Byte  almost the smallest unit of measurement, in fact a byte is an byte bit

binary number which can take  on 256 different values sufficient to describe

all the text characters  plus some for  graphics and other  purposes. A text

file usually uses one byte per character of text.

Sector - a sector is a portion of a  track of recorded data on a floppy disk

and consists of a number of bytes,  typicaly 128, 256, 512 or 1024. A sector

is the minimum amount of information that can be read from, or written to, a

disk. A sector on a standard TRS-80 DOS is 256 bytes.

Grans - TRS-80 operating  systems always  read and  write sectors  in groups

called granules. Files  are therefore always  multiples of the granule size.

Consult your DOS manual for definite information on the size of a granule in

your case. Typically, however, granules are three, five or six sectors, each

sector being 256 bytes.

Blocks or Clusters - another name for a group of sectors. MS and PC DOS uses

the name 'cluster' which is  only 1 sector  in size on single  sided and two

sectors on  double sided disks.  CP/M uses 'blocks'  which are usually 1024,

2048 or 4096 bytes long. Files on CP/M  and PC DOS's are multiples of blocks

or clusters long.

--------------------------

End of File Markers

The end of a file can never be guaranteed  to end exactly at the end of

a sector, gran, block  or cluster so some other  means is used to show where

the end actually is.  On the TRS-80  the directory information  contains and

EOF value which says  which byte in the final  sector is the last byte. Also

there is a  number given  showing the  total  number of  records (=256  byte

sectors) in the  file. There is  some inconsistency in  this with some DOS's

counting sectors from 0 and some from 1.

On CP/M  files are  always multiples  of 128  bytes long. The directory

contains an entry defining the  number of 128  byte records in  the file and

nothing  else is  available. So  users  have to  resort to  some other means

within the file. Text files  are terminated with a  Control-Z character (hex

1A)  and, if  the  ASCII file  transfer mode  is  selected, this  is used by

Hypercross to determine the proper end of file.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  22                   Technical Information                   Section 4

IBM PC files have a four byte directory entry defining the exact length

which can be used for data  files. On text files however  this is not always

exact and may be rounded up to  the nearest whole sector or cluster. In this

case the exact end of file is marked by a Control-Z as with CP/M.

Some TRS-80  text editors use  a 00 byte  to mark the  end of the files

they create. To allow for this, when copying  from PC or CP/M files in ASCII

transfer mode, the  program automatically puts  at least one  00 byte on the

end instead of the CTRL-Z mark.

--------------------------

ASCII File Differences

There  are  some differences  between  the way  text  (ASCII) files are

stored on disk by the TRS-80 and CP/M or PC disk systems. Apart from the end

of file  mark as described  above a major  difference is the  way the end of

each line is marked. On the TRS-80 a single carriage-return code (hex 0D) is

used. In CP/M  and PC files  a line feed  is added (hex  0A). If you were to

copy a text file  from say  CP/M to  TRS-80 without  making any  changes you

would find your printer double line spacing if you were to try and print it.

The ASCII file copy switch takes care of this for you automatically removing

the 0A's  when copying  to TRS-80  formats and  inserting 0A's  in the right

places when copying from TRS-80 to CP/M or PC.

Another problem  with the  TRS-80 is that  four of  the ASCII codes are

used in a non standard way. These are the codes:

Hex     True ASCII       TRS-80 uses it for

for        Model I        Model III/4

5B          [         Up arrow       Up arrow

5C          \         Down arrow

5D          ]         Left arrow

5E          ^         Right arrow

To correct for this the -A switch  will cause the copy program to change any

occurence of 5B hex in a  TRS-80 text file to  a 5E when copying  to a PC or

CP/M disk. The reverse will be done  when copying the other way. If you want

to use ASCII transfer but  do not want  this substitution to  take place you

can  select  this  as  an  option  when  you  run  CFGMAKE/BAS  to customize

Hypercross at run  time. You  can also  elect to  convert TAB  characters to

space characters when  copying from alien to  TRS-80 format since the TRS-80

does not recognize TABs.

--------------------------

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 4                  Technical Information                    Page  23

Load File Formats

Apart from differences  in ASCII file  formats there are differences in

the way other  types  of files  are  stored on  disk  by different  computer

operating systems. In particular the TRS-80 stores /CMD files using a scheme

which indicates where each section of the file is to be loaded in memory and

the address where the program execution is to start.

By contrast CP/M load format is pretty dumb. All machine language files

load and start running  at 0100 Hex and  must contain some kind of relocator

if they are to run elsewhere in memory.  Since CP/M programs are written for

8080 or Z80 microprocessors they should, in many cases, be adaptable to TRS-

80 operation.

--------------------------

Spread sheet file formats

If you use VISCALC on your TRS-80 you will be able to transfer these to

the PC using the COPY or TAG command.  VISICALC VC files are in ASCII format

so use the -A switch when copying them. You can then read them straight into

VISICALC on your PC or compatible. If you  prefer Lotus 123 then Lotus has a

Visicalc to 123 translation program  VCWKS.EXE which allows you to translate

Visicalc .VC files to Lotus .WKS file format.  You can also go back from WKS

to DIF format using Lotus's file translator.

--------------------------

Using JCL DO control files

Most TRS-80 operating  systems  allow  you  to build  a  file  of  text

commands and then to  execute it as if  the commands had been typed directly

from the keyboard. These  files generally have  the extension /JCL  (for Job

Control Language) and are executed with  the DO command. You can use this to

automate a sequence of operations with Hypercross. For instance, suppose you

want to format  a blank disk  in drive 1 as  an IBM PC  disk and then copy a

file to it and return to DOS you could make a JCL file such as this:

HYPERCROSS

FORMAT :B

COPY FILE/DAT:0 FILE.DAT:B -A

X

If this file was  called say MAKEPC/JCL you  would execute it by typing

DO MAKEPC from DOS command level. It would first load and run Hypercross and

then it would format thhe  disk in drive B (:1).  Note the blank line in the

file corresponding to the <enter> key being  pressed when Format asks you to

insert a disk. It would then do a file copy and exit to DOS.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  24                     Formats Supported                     Section 5

5 Formats Supported

This section  details the formats  that are available  at the time this

manual was printed. Your Hypercross disk contains a file which contains some

or all of these depending on the version you ordered. You can upgrade at any

time by returning  your old  disk  plus the  price  difference between  your

version  and the  new  one plus  $5. There  are  four major  versions of the

program:

A) A PC version containing the first 4 PC/MS-DOS formats

in the list below.

B) A CP/M version containing 40 single sided CP/M formats

selected from the list below.

C) The XT version containing 90 of the CP/M and MS-DOS

formats below.

D) The XT+ version containing all the formats below.

Note SS = single

sided, DS = double sided, SD = single density, DD = double

density, 35,40,80 etc = number of tracks.

Note: Model I users need a doubler for DD formats.

All users need 2 sided drives for DS formats.

You need 80 track (96 TPI) drives for 80 track formats.

The 8 inch format needs an 8 inch drive and supporting

hardware.

IBM PC (PC-DOS, MS-DOS) formats:

Type            Sides Density Tracks Sectors Bytes

PC/MS-DOS 1.1        SS    DD    40       8       512

PC/MS-DOS 2.0        SS    DD    40       9       512

PC/MS-DOS 1.1        DS    DD    40       8       512

PC/MS-DOS 2.0        DS    DD    40       9       512

Burroughs            DS    DD    80      10       512

DEC Rainbow          SS    DD    80      10       512

IBM 5550 (Japan)     DS    DD    80       8       512

Jformat t  Tall Tree SS    DD    40      10       512

Jformat tt     "     SS    DD    40      10       512

Jformat b      "     SS    DD    40       5      1024

Jformat e      "     SS    DD    40      10       512

Jformat et     "     SS    DD    40      10       512

Jformat f      "     SS    DD    40       5      1024

Jformat q      "     DS    DD    80       8       512

Jformat o      "     DS    DD    80       9       512

Jformat r      "     DS    DD    80      10       512

Jformat rt     "     DS    DD    80      10       512

Jformat n      "     DS    DD    80       5      1024

Tandy 2000           DS    DD    80       9       512

Televideo 1603       DS    DD    80       9       512

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 5                    Formats Supported                      Page  25

CP/M formats:

Type            Sides Density Tracks Sectors Bytes

Aardvark             SS    DD    35      10       512

Access Matrix        SS    DD    40       9       512

Access II            DS    DD    40       9       512

Action Discovery 500 DS    DD    80       8       512

Altertext            SS    DD    40      18       256

Altos                DS    DD    80       9       512

Ampro Little Board   SS    DD    40      10       512

Ampro Little Board   DS    DD    40      10       512

ATR 8000             SS    DD    40      10       512

Avatar               SS    DD    40      10       512

Avatar TC10          DS    DD    40      10       512

Avatar               DS    DD    80      10       512

Casio FP 1000        DS    DD    40      16       256

Chameleon            DS    DD    40       8       512

Cifer 2683           DS    DD    40      10       512

CCS                  SS    DD    35       5      1024

Columbia MPC         DS    DD    40       8       512

Cromemco CDOS        SS    DD    40      10       512

Cromemco CDOS        DS    DD    40      10       512

Cromemco Z-2         SS    SD    40      18       128

Cromemco Z-2         SS    DD    40      10       512

Cromemco             DS    DD    40      10       512

Davidge              DS    DD    40       5      1024

Digilog              DS    DD    35       5      1024

DEC VT180 Robin      SS    DD    40       9       512

DEC Rainbow          SS    DD    80      10       512

Eagle                SS    DD    80       5      1024

Epson                DS    DD    40      16       256

Epson Multifont      DS    DD    40      16       256

Epson  QX10          DS    DD    40      10       512

Florida Graphics     DS    DD    80      16       256

Fujitsu 16 S         DS    DD    40       8       512

Gemini               DS    DD    80      10       512

Genie III            DS    DD    80      18       256

Groupil III          DS    DD    40       4      1024

HP 125               DS    DD    40      16       256

IBM PC CP/M-86       SS    DD    40       8       512

IBM PC CP/M-86       DS    DD    40       8       512

Idea Bitelex         SS    DD    40      16       256

IMS 5000             DS    DD    80       5      1024

Insight IQ 120       SS    DD    40      16       256

ISM                  DS    DD    40      10       512

ITT 3030             DS    DD    70      16       256

Kaypro II            SS    DD    40      10       512

Kaypro 4 & 10        DS    DD    40      10       512

LNW-80/Team          SS    DD    40      18       256

LNW-80/Team          DS    DD    40      18       256

Lobo Max-80          SS    DD    35      18       256

Lobo Max-80          SS    DD    40      18       256

Lobo Max-80          DS    DD    40      18       256

Lobo Max-80          DS    DD    80      18       256

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  26                     Formats Supported                     Section 5

Max-80 CPM 3.0       SS    DD    40      10       512

Max-80 CPM 3.0       DS    DD    40      10       512

Max-80 CPM 3.0       DS    DD    80      10       512

Micral 9050          DS    DD    40      16       256

MicroBee             DS    DD    40      10       512

Molecular 9000       DS    DD    40       9       512

Monroe 2000          DS    DD    80       8       512

Morrow MicroDecison  SS    DD    40       5      1024

Morrow MD3 MD11      DS    DD    40       5      1024

NCR Decision Mate    DS    DD    40       8       512

NEC PC-8001          SS    DD    40      16       256

NEC PC-8801          DS    DD    40      16       256

Nelma Persona        SS    DD    40       9       512

Northern Telecom 803 DS    DD    80      10       512

Novel                DS    DD    35      16       256

Octopus              DS    DD    40       5      1024

Octopus              DS    DD    80       5      1024

Olympia ETX II       SS    DD    40       9       512

Olympia EX 100       DS    DD    40       9       512

OKI IF800 Lifeboat   DS    DD    40      10       512

Osborne-I            SS    SD    40      10       256

Osborne-I, Exec      SS    DD    40       5      1024

Osborne Osmosis      SS    DD    40      10       512

OSM Zeus             SS    DD    40       8       512

OSM Zeus 2/3/4       DS    DD    80       8       512

Otrona               DS    DD    40      10       512

Pied Piper           DS    DD    80      10       512

PMC-101 CPM 3.0      SS    DD    40       5      1024

PMC-101 CPM 3.0      DS    DD    40       5      1024

Reynolds TC1000      SS    DD    40       5      1024

Sanyo 1000           DS    DD    40      16       256

Sanyo 1250           DS    DD    80      16       256

Sanyo 2000           SS    DD    80      16       256

SD Computers         SS    SD    40      18       128

Sharp YX3200         DS    DD    40      16       256

Superbrain           SS    DD    35      10       512

Superbrain D         DS    DD    35      10       512

Systel II            SS    DD    40       9       512

Systel IV            DS    DD    40       9       512

Teletek              SS    SD    35      18       128

Teletek              SS    DD    35      18       256

Teletek              SS    DD    40      18       256

Teletek              DS    DD    35      18       256

Televideo 802        SS    DD    40      18       256

Televideo 802-6      DS    DD    40      18       256

Televideo 1603       DS    DD    80       9       512

Televideo Turbo      DS    DD    40       5      1024

TI CP/M 80           SS    DD    40       8       512

Topper Beehive       SS    DD    40      10       512

Toshiba 100/200      DS    DD    35      16       256

TRS80 1 CPM 1.4      SS    SD    35      18       128

TRS80 1 FEC          SS    SD    40      18       128

TRS80 1 FEC          SS    DD    40      18       256

TRS80 1 Omikron      SS    SD    40      18       128

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 5                    Formats Supported                      Page  27

TRS80 1 Omikron      SS    SD    80      18       128

TRS80 1 Omikron      SS    DD    40      28       128

TRS80 1 Omikron      SS    DD    80      28       128

TRS80 3 Holmes       SS    DD    40      10       512

TRS80 3 Holmes       DS    DD    40      10       512

TRS80 3 Hurricane    SS    DD    40       5      1024

TRS80 3 MM Shuffle   SS    DD    40      10       512

TRS80 3 Omikron      SS    DD    40       5      1024

TRS Montezuma 1.30   SS    DD    40      18       256

TRS Mtzuma 1.30      DS    DD    40      18       256

TRS Mtzuma 1.32      SS    DD    40      18       256

TRS Mtzuma 1.32      DS    DD    40      18       256

TRS Mtzuma 1.4x      SS    DD    40      18       256

TRS Mtzuma 1.4x      DS    DD    40      18       256

Mtzuma 2.2x System   SS    DD    40      18       256

Mtzuma 2.2x System   DS    DD    40      18       256

Mtzuma 2.2x System   SS    DD    80      18       256

Mtzuma 2.2x System   DS    DD    80      18       256

Mtzma 2.2x data disk SS    DD    40      10       512

Mtzma 2.2x data disk DS    DD    40      10       512

Mtzma 2.2x data disk SS    DD    80      10       512

Mtzma 2.2x data disk DS    DD    80      10       512

TRS80 Mod 4 CPM 3.0+ SS    DD    40       8       512

Wang MAWS            DS    DD    40      16       256

Xerox 820-II         SS    SD    40      18       128

Xerox 820-II         DS    SD    40      18       128

Xerox 820-II         SS    DD    40      17       256

Xerox 820-II         DS    DD    40      17       256

XOR-100 512          SS    DD    40      10       512

XOR-100 512          DS    DD    80      10       512

Zenith-Heath H89     SS    SD    40      10       256

Zenith-Heath H89     SS    DD    40      16       256

Zenith-Heath H89     DS    DD    40      16       256

Zenith-Heath H89     DS    DD    80      16       256

Zenith-Heath Z90     SS    DD    40      16       256

Zenith Magnolia      SS    DD    40       9       512

Zenith Magnolia      SS    DD    80       9       512

Zenith Z100          SS    DD    40       8       512

Zenith Z100          DS    DD    40       8       512

Zenith Z100 8re      DS    DD    40       8       512

Zorba                DS    DD    40      10       512

Zorba Z2000          DS    DD    40       5      1024

Zorba Q              DS    DD    80      10       512

Zorba Z2000Q         DS    DD    80       5      1024

8 inch CPM standard  SS    SD    77      26       128

We are always on  the look-out for  new formats to  add to the  list. If you

have access to a format not listed here, format a disk and put some files on

it and send  it to  us together  with  a completed  copy of  the New  Format

Submission form  and we  will endeavour to  include it.  If possible we will

return your original  disk with  a new  copy  of the  file transfer  program

containing all new additions to date  including yours. If the disk is double

sided, we need  it to  be more  than half  full with  files so  that we  can

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  28                     Formats Supported                     Section 5

determine how  the back side  is recorded.  There are many  ways this can be

done and we need  a disk with something written on the back to be certain.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 6                   Configuration files                     Page  29

6 Making a Configuration File

Included on your Hypercross  disk is  a program CFGMAKE/BAS  for making

configuration files  for Hypercross. If  a configuration  file is present on

the disk when  Hypercross  runs, it  will  be loaded  and  it will  set  the

following parameters:

o   The type of DOS you are running under.

o   The default file transfer switch.

o   The initial alien formats F1 and F2.

o   The default Alien and TRS-80 drives.

o   The number of tracks on each drive.

o   The alien format allocated to each drive.

o   The drive stepping rate.

You can have  more than one  different configuration file  - or none at

all. If there is  no file SXCFG0/DAT  on your disk  or if you  hold down the

Enter  key Hypercross  will  start and  ask you  for  your DOS  type and the

principal alien format F1. All drives will be  set to 40 tracks and assigned

to alien format F1. The drive stepping rate will be 40/30 mS and the default

drives will be 0 (TRS-80) and B (Alien).

The  configuration file  loaded will  be  SXCFG0/DAT if  it is present.

However, if you  hold down  one  of the  keys  1 to  9  while Hypercross  is

loading,  Hypercross will  try and  load  an alternative  file SXCFG1/DAT to

SXCFG9/DAT depending on  the key  held  down. Thus  you can  have  up to  10

different  configurations loaded  automatically  according to  which key you

hold down.

If you have  the file SXCFG0/DAT  present on your  disk and you want to

bypass it, simply hold down  the Enter key while  Hypercross is loading. You

can also hold down  a key from 0 to  9 corresponding to a non-existant SXCFG

file. For example, if SXCFG9/DAT is not on  your DOS then holding down the 9

key will force Hypercross to try and load SXCFG9/DAT and, not finding it, it

will bypass the configuration file.

Running CFGMAKE/BAS

CFGMAKE/BAS is a BASIC program  so you will  have to load  basic to run

it. First, transfer  CFGMAKE/BAS to a  DOS disk which  has Disk BASIC on it.

Then you must run BASIC and load CFGMAKE/BAS.  The exact form of the command

depends slightly on  the DOS you  are using. For  most BASICs you can simply

type:

BASIC              to load and run BASIC

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  30                    Configuration files                    Section 6

RUN "CFGMAKE/BAS"  to run CFGMAKE/BAS

CFGMAKE uses one  disk file, and  some DOSs require  you to specify how

many files are to be allowed. With Dos Plus you will have to run Basic thus:

BASIC -F:1

Once CFGMAKE/BAS is running it will ask you a series of questions. From

your answers it will create the configuration file. At any time you can type

B to step back to the previous question  if you want to revise your answers.

CFGMAKE  gives  you detailed  explanations  for each  question  and needs no

separate step by step discussion here. When you are done it will ask you for

a file  number from 0  to 9.  This allows you  to create  up to 10 different

configuration files. Which one  is loaded  will depend on  which key  0-9 is

held down while Hypercross  loads. If no key is  held down at load time then

SXCFG0/DAT will be loaded if present.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 7                     Trouble Shooting                      Page  31

7 Trouble Shooting

After selling Hypercross for some time a  pattern is starting to emerge

in the kinds of problems many people  have when using it for the first time.

In this section most  of these are  listed with  the cause and  cure. Please

read this first before calling or sending disks in the mail.

* I can't read my Model III disk.

Model III  disks are distributed  in single density  35 track format s

that they can be  easily read  by all the  available DOSs,  NEWDOS, DOSPLUS,

LDOS etc.  However, if you  have TRSDOS  1.3 you will  NOT be  able to get a

directory of you distribution Hypercross disk. TRSDOS  will hunt for a while

and then give an  error message: ** ERROR  04 ** Operation aborted. You will

have to run CONVERT to transfer  the files to your  DOS. WARNING: If CONVERT

has been deleted  from your TRSDOS 1.3  disk you will  get a read error when

the DOS goes looking for it on drive 1. See the section on getting started.

* Wrong format selected

If you put an alien disk  in your system and  tell Hypercross the wrong

format  unpredictable  things can  happen.  Because of  the  wide variety of

formats there is no infallible way of automatically detecting what the alien

format actually is. You must know exactly. Near enough is not good enough. A

format selected in error may appear  to work but on  examining the files you

may find they are scrambled.

Some MSDOS formats are  verified by  Hypercross and  a warning  will be

given if the disk  is not in the selected  format. With CP/M the sector size

and number of sectors on  a track are  checked and an error  message will be

given if these do not match the specifications of the one selected.

Some users have tried to  select 80 track or  double sided formats when

they  only have  40  track or  single  drives. You  MUST  have hardware that

supports the format selected.  The only exception  to this is  that you can,

with care, read and write 40 track  formats in an 80 track drive. However if

you write to a 40 track disk it must have been either new or bulk erased and

you must format it and write to it ONLY in the 80 track drive.

* Double Sided Drives & MSDOS versions

One of the most common problems of MS-DOS users is putting double sided

format disks in a single sided disk drive. Users typically select format 1D,

double sided MS-DOS 2.x and are  surprized when they get a garbage directory

displayed. If you have a  stock TRS80 model 1,  3, or 4 it  will have single

sided drives  and you  MUST choose  format 1A  or 1B.  You cannot read, even

partially, double sided  disks. You  must use  single sided  disks for  file

transfer. On the PC you can format a disk single sided with the command:

FORMAT D:/1

where D is the drive letter (typically A or B).

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  32                      Trouble Shooting                     Section 7

* Can't load Basic program on the PC.

It is common on the TRS80 for  Basic programs to be called by a name of

1-8 letters with no /BAS  or other /extension  after the name.  When you say

LOAD "filename" on the TRS80 this gives no problem. However many people have

found when they copy this over to a PC disk they can't load it into BASIC on

the PC.  "Where is it  ?", they say,  "I can list  it on my  PC from DOS but

BASIC says File not found".

The answer is simple, BASIC on the  PC is too clever for it's own good.

If you try and save or load  a file on the PC  it automatically checks for a

.extension  after  the  name.  If  it   can't  find  one  it  puts  in  .BAS

automatically. So, if you say LOAD "MYFILE" on the PC it treats it as if you

had typed LOAD "MYFILE.BAS" and promptly says it can't find it. To get round

this say: LOAD "MYFILE.", note the period after the name.

* Montezuma CP/M format problems

A lot of confusion has arisen because of Montezuma's unfortunate choice

of release  numbers for their  CP/M. When Digital  Research wrote CP/M their

first major release was  called CP/M 1.4  and their second  and most popular

was CP/M 2.2. More recently Digital Research has introduced CP/M 3.0 plus.

Now Montezuma Micro's release  uses Digital  Research CP/M 2.2  for all

versions to date. Unfortunately, in the course of development Montezuma went

through 'Versions' numbered  1.30, 1.32,  1.40,   and 2.20-2.23.  Each major

release used a  different disk format. Users  are confusing the CP/M version

with the Montezuma  release number.  If  you are  having  trouble reading  a

Montezuma disk check whether you have selected the correct Montezuma release

number and not used the CP/M version number.

* Funny characters in file names.

CP/M and MS-DOS allow  a much wider  range of characters  in filenames.

These  characters include  - ;  /  among others.  Names can  also begin with

numbers. TRS80 DOSs  allow filenames  containing  only letters  A  to Z  and

numbers 0 to 9. In addition names and extensions must begin with a letter.

If you try and copy a file  to a TRS80 disk from  a CP/M or MS-DOS disk

using the F or TAG commands you  may, because of an illegal file name get an

ERROR 30H message.  For instance  trying to  copy a  file 48TPI.DOC  it will

convert to 48TPI/DOC which is illegal  on the TRS80 because it begins with a

number. You will have to change the name, for example like this:

COPY 48TPI.DOC:B TPI48/DOC:0 -A

* File not found on TRS80 disk.

Trying to copy  a file from a  TRS80 disk to an  alien disk may give an

error indicating file not found. because of  an invisible password. The COPY

command will allow you to copy filenames with passwords as for example:

COPY FILENAME/EXT.PASSWORD:0 FILENAME.EXT:B -I

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Section 7                     Trouble Shooting                      Page  33

* Text files unreadable on MS-DOS or CP/M

If you copy a text file from  a TRS-80 disk to a  PC or CP/M disk using

the -I switch no line feed  will be inserted after each carriage return. The

result will be that if you try and  list the file on the destination machine

all lines  will be printed  on top  of one another  making it unreadable. To

correct this use the -A switch when copying.

* Double spaced lines.

If you get double spaced lines  in a text file when  you copy it from a

PC or CP/M  disk to a TRS-80  disk then you  probably selected the -I switch

instead of the  -A switch.  Text  files on  CP/M  and PC  disks  end with  a

carriage  return and  a line  feed. Each  causes a  line feed  on the TRS-80

resulting in double spacing. Use the -A switch which corrects this.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  34                   Converting Basic Files                 Appendix 1

Appendix 1 - Converting Basic

This version of Hypercross includes  a new feature which  allows you to

transfer Basic  files from TRS-80  disks to MS-DOS  or CP/M disks converting

them to the correct format on  the fly. No other  operation is needed except

to  choose  the  -B  transfer  switch.  This  will  automatically invoke the

conversion of your TRS-80 Basic program to  CP/M or MS-DOS format, inserting

spaces where  needed, converting  PRINT @  statements and flagging potential

trouble spots.

To use, simply type  T to select transfer  from TRS80 to alien and then

choose the B switch. Alternatively you  can use the Tag  mode and mark Basic

files with a B or use the direct copy command thus:

COPY filename:0 filename:B -B

Note that the Basic transfer to CoCo disks is not supported.

The Basic files must have been saved on disk in the normal way, that is

in compressed format. If  you try and list a  Basic file that has been saved

like this you will see mostly garbage with  a few readable text strings here

and there. This is  because Basic uses special  code words called TOKENS for

all the commonly used instructions. Although  Microsoft wrote all the Basics

used by  TRS80s, MS-DOS  and CP/M  Machines they  chose different tokens for

each version. For  instance the  code  DD represents  SQR  on Model  I/IIIs,

VARPTR on the Model 4, OFF on the PC, INKEY$ on CP/M and BACKUP on the CoCo.

Consequently if you copy a file from a TRS-80  disk to say a PC disk then it

will come out with all the keywords  swapped round. For instance "A = B + C"

on the Model III would come out as "A ERR B THEN C" on a PC.

The -B switch, like the A,  I and R switches tells Hypercross what kind

of file you are  transferring. When it  sees the B  it treats the  file as a

compressed Basic file  and automatically converts  the tokens to the correct

ones for the destination  computer. Not only  that, it inserts  spaces where

neccessary and converts  PRINT@ statements to the  correct format for the PC

or CP/M. It also inserts THEN in conditional IF statements where neccessary.

Certain keywords  in the TRS80  either have no  equivalent or mean something

different on the PC or CP/M versions of Basic. These are flagged so that you

can easily find and correct them later.

The different versions of Basic are highly compatible and many programs

can be transferred and run without any problem. However, because programmers

like to be clever  sometimes they may  have done  something which has  to be

modified  by hand.  Check through  the  converted program  for the following

possible trouble spots and,  after fixing those,  run it and  see what error

messages turn up.

Things to check for

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Appendix 1                 Converting Basic Files                   Page  35

The Keywords on  the TRS-80 that  have no equivalent  on the PC or CP/M

are expanded to ASCII  clear text.  In a  few cases  the same  keyword means

something  different on  each  machine. Fortunately  these  are rarely used.

Keywords in this category include  CLOAD, CSAVE, CMD, INP,  OUT, SET, RESET,

POINT,  PEEK,  POKE, ERR  and  SYSTEM.  Since these  cannot  be used without

modification Hypercross flags them with an exclamation mark (

and puts a REM

#$%* statement on the  end of the line.  If you run your transferred program

without modification the  ! will  force basic  to abort  and issue  an error

message without  executing the offending  statement. Basic  will tell you on

what line the error  occurred which  will allow  you to  edit and  rerun the

program quickly.

PEEK and POKE instructions are sometimes  used by TRS-80 programmers to

adjust some parameters of DOS. Below are some of the addresses most commonly

peeked or poked. In some cases  two succsessive adresses are  used to hold a

16 bit number in two bytes, low byte first, high second.

Model I             Model III             Useage

Hex    Decimal        Hex   Decimal

0-2FFF    0-12287        0000-2FFF 0-12287     Basic ROM area

3800-38FF 14336-14591    3800-38FF 14366-14591 Keyboard hardware

3C00-3FFF 15360-16383    3C00-3FFF 15360-16383 Video memory

4020/1    16416/7        4020/1    16416/7     Cursor position

4022      16418          4022      16418       Character under cursor

-          -            4023      16419       Cursor character

4028      16424          4028      16424       Maximum lines per page

4029      16425          4029      16425       No of lines printed + 1

-          -            402A      16426       Printer Tab position

4041-3    16449-51       4217-9    16919-21    Time Secs Min Hour

4044-6    16452-4        421A-C    16922-3     Date Year Month Day

4049/A    16457/8        4411/2    16505/6     Memory Size

Some programs may peek  ROM to determine  what Model the  program is running

on.  For  instance  PEEK(293) returns  73  if  you are  on  a  Model III and

something else on the Model I.

Filenames on MS-DOS or CP/M use  a period to separate the name from the

extension rather than the / on the TRS-80. Also passwords are not available.

Look for references in the  program to "FILENAME/EXT.PASSWORD" and change to

"FILENAME.EXT". Some programs may combine the  name and extension separately

so watch for that too.

Programs on the  TRS-80  will  sometimes  use  graphics  characters  to

enhance the display. Corresponding characters  do not always exist on MS-DOS

or CP/M systems so you may have to tailor these too.

Model I/III Basic allows  you to leave out  the THEN statement after an

IF test. This is only allowed for GOTO  and GOSUB statements in Model 4, MS-

DOS and  CP/M Basics.  The Model  I, III  and Max-80  versions of Hypercross

automatically test the syntax  of conditional  statements and insert  a THEN

where neccessary. Here are some examples:

Model I/III/Max-80           Model 4, MS-DOS or CP/M

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page  36                   Converting Basic Files                 Appendix 1

100  IFA=BRETURN                      100 IF A=B THEN RETURN

100  IF A=B+C LET D=E+F               100 IF A=B+C THEN LET D=E+F

100  IF A=B+C PRINT "A=B+C"           100 IF A=B+C THEN PRINT "A=B+C"

Note however that GOTO and GOSUB do not need a THEN

100  IFA=B+CGOTO200                   100 IF A=B+C GOTO 200

PRINT @ statements are converted correctly only if they are of the form

PRINT @ N where N is a number, not  a symbol or expression. If you have case

like this then they must be changed  manually. The easiest way to this is to

change the statement as folows:

Suppose you have:   PRINT    3*I,"Hello";

This means locate the cursor at 3*I and print Hello.

Change this to:     X=3*I:GOSUB 20000:PRINT "Hello";

At 20000 put a general purpose subroutine like this:

20000 Y=INT(X/64):X=X-Y*64:LOCATE Y+1,X+1:RETURN

For Model 4 use 80 instead of 64.

Hypercross III Manual           -=<*>=-      Copyright 1985, 1986, Hypersoft

Page 37

TABLE OF CONTENTS

-=-=-=-=-=-=-=-=-=-

0   Program Overview............................................... 1

1   Getting Started ............................................... 3

2   The Main Menu ................................................  7

Changing the configuration ...................................  7

The Quick Menu ...............................................  8

3   Program Commands ............................................. 11

DIR .......................................................... 11

COPY ......................................................... 11

ERASE, KILL & REMOVE ......................................... 14

FORMAT ....................................................... 14

HELP ......................................................... 15

LOAD ......................................................... 15

R ............................................................ 16

SELECT1/2 .................................................... 16

TAG .......................................................... 16

USER1/2 ...................................................... 19

XFER ......................................................... 19

4   Technical Information ........................................ 21

Bytes, Sectors, Grans, Blocks and Clusters ................... 21

End of File Markers .......................................... 21

ASCII File differences ....................................... 22

Load File Formats ............................................ 22

Spread Sheet files ........................................... 23

Using JCL DO files ........................................... 23

5   Formats Supported ............................................ 24

PC-DOS and MS-DOS formats .................................... 24

CP/M formats ................................................. 25

6   Making a Configuration File .................................. 29

Running CFGMAKE/BAS .......................................... 29

7   Trouble Shooting ............................................. 31

I can't read my Model III disk ............................... 31

Wrong format selected ........................................ 31

Double Sided Drives & MSDOS versions ......................... 31

Can't load Basic program on PC ............................... 32

Montezuma CP/M format problems ............................... 32

Funny characters in file names ............................... 32

File not found on TRS80 disk ................................. 32

Text files unreadable on MS-DOS or CP/M ...................... 33

Double spaced lines .......................................... 33

8   Appendix: Converting Basic Programs.............................34