501-1638        4/75 (SPARCstation 2) motherboard 16M FCC-A

50MHz CY7C601 @ 40MHz, TI TMS390C601A (602A ?), Sun-4c MMU with

16 hardware contexts. 20MHz SBus.

From left to right, the rear edge of the board has: a 50-pin

SCSI-2 connector; an Ethernet connector; two serial port

connectors (ports A and B from left to right); an 8-pin-DIN

keyboard/mouse connector; and an audio connector.

Connector pinouts and form factors are not known.

Memory consists of up to sixteen 4M x 9 80ns 30-pin SIMMs. The

SIMM slots are arranged in four groups of four. Electrically,

there are four "banks," each of which is composed of one slot

from each group:

Back of machine (nearest SBus connectors)

-------U0311------ 0   0 -------U0322--------

bits    -------U0312------ 2   2 -------U0321-------- bits

24-31  -------U0313------ 1   1 -------U0320--------  0-7

-------U0319------ 3   3 -------U0316--------

-------U0309------ 0   0 -------U0307--------

bits    -------U0310------ 2   2 -------U0308-------- bits

16-23  -------U0314------ 1   1 -------U0315--------  8-15

-------U0318------ 3   3 -------U0317--------

Front of machine (nearest disk connectors)

Banks must be filled in order (0 through 3). THERE IS SOME

DISPUTE OVER WHICH SLOTS BELONG TO WHICH BANKS. The Sun Field

Engineer Handbook for 12/15/93 says that the banks are in order

(i.e. 0, 1, 2, 3 from back to front), but other sources agree

that they are in the interleaved order shown above.

There are a variety of jumpers. All locations are given with

component side up and connector side toward you.