EMULATE.DOC         News and corrections EMULATE 3.00         1987-08-10

The contents of this file is an addendum to the README.DOC file for

MS-DOS 3.20.

GENERAL     EMULATE v3.0 is an enhanced version of the PC ROM BIOS

emulator.

LEVELS      Three levels of emulation are available.

Level 1 corresponds to PC-mode.

Level 2 is a slightly enhanced version of EMULATE v2.0.

Level 3, in addition to level 2, supports direct handling of

the video memory for the monochrome or color/graphics

adaptors. This level of emulation requires 768K of RAM

memory.

Level 2 and 3 stays resident in memory when loaded.

An optional parameter /1, /2 (default if 256K) or /3

(default if 768K) specifies the level of emulation. If level

2 or 3 is already loaded, the level of emulation can not be

changed without resetting the computer. This must be done

with the RESET button on the front of the computer.

NOTE! Use always the RESET button to restart the computer

whenever the emulator is loaded!

PRINTER     A printer filter (cf. PREMUL) is included. An optional

parameter /ON (default) or /OFF defines the state of the

filter. Parameters /+ and /- can be used instead of the

parameters /ON and /OFF.

The printer filter status can be changed directly from the

keyboard. Supershift+Ctrl+Insert activates and Supershift+

Ctrl+Delete deactivates the filter.

A character screen dump (int 05h) is activated with Shift+

Ctrl+F1. Either local or remote printers can be used. No

message is displayed if the screen dump cannot be performed.

The filter is automatically deactivated during graphic

screen dumps.

Full handling of both remote and local printer also for

"normal" printing.

NOTE! EMULATE v3.0 may not be used together with PREMUL!

MODES       Level 2 only supports video mode 7 (25*80 characters).

Characters can only be written to the screen with DOS- or

BIOS-calls.

Level 3 supports video modes 2-7. Direct output to video

memory is emulated. Some programs require a specific video

mode set. Available modes are /MONO for monochrome adapter,

/CGA for graphics or /MONOGR for both.

The video mode can be reset to initial (not restricted) mode

with parameter /INIT. This will also restore some parameters

that application programs may have destroyed.

The different screen modes will work with monochrome as well

as with color screens.

HELP        A status report returned if only the parameter /? is typed.

Helptext is available with the parameter /H or /HELP.

PARAMETERS  Examples of valid combinations:

EMULATE /1              PC-mode

EMULATE                 Load maximum level possible

EMULATE /2 /OFF         Level 2, filter off

EMULATE /3              Level 3, filter on

EMULATE /3 /CGA         Level 3, filter on, cga

EMULATE /?              Status request

EMULATE /h              Help request

ERRORLEVEL  The EMULATE command sets the exit code as follows:

0    Normal completion

1    Syntax error

2    Option conflict

3    Incorrect DOS version

4    Incorrect ROM BIOS version

5    Insufficient memory

6    Level 3 can't be loaded,

if level 2 is already loaded

10    No emulator. No network.

11    No emulator. Network active, not logged on.

12    No emulator. Network active, logged on.

13    Emulator level 1. No network.

14    Emulator level 1. Network active, not logged on.

15    Emulator level 1. Network active, logged on.

16    Emulator level 2. No network.

17    Emulator level 2. Network active, not logged on.

18    Emulator level 2. Network active, logged on.

19    Emulator level 3. No network.

20    Emulator level 3. Network active, not logged on.

21    Emulator level 3. Network active, logged on.

These codes can be used with the batch processing IF

ERRORLEVEL subcommand. The codes 10-21 are only returned

with a status request /?. The network status can be used to

check if the central printer can be used, i.e. the user is

logged on.

KEYBOARD    The key combination Ctrl+Break, calls interrupt 1Bh. The

default action for this interrupt is to insert character 3

(ctrl-C) into the keyboard queue.

INTERRUPTS  The following PC interrupts are emulated:

5h    print screen

9h    physical keyboard

10h    video

11h    equipment-list

12h    memory size

13h    diskette (only for stand-alone computer)

14h    communications

16h    keyboard

17h    printer

1Ah    time-of-day

1Bh    keyboard break

1Ch    timer tick

1Fh    graphic characters

The PC interrupts 15h, 18h, 19h, 1Dh, 1Eh, 67h (EMS) and F0h

(used by the BASICA interpreter) are dummy interrupts.

The COMPIS/SCANDIS interrupts E6h (graphics), EBh (network),

EEh (physical hard disk), EFh (physical terminal), F1h

(flexible disk), F3h (keyboard) and F4h (parallel printer)

are modified.

The hardware interrupts 02h (NMI), 08h (timer 0) and 12h

(timer 1) are used.

VARIABLES   The most important variables located from absolute location

400h to 5FFh in a PC computer are placed at the same

adresses. These variables are:

408h (word)    address of printer

410h (word)    equipment-list data

413h (word)    usable memory size

41Ah (word)    head of BIOS keyboard buffer

41Ch (word)    tail of BIOS keyboard buffer

440h (word)    time out for flexible disk drive

449h (byte)    current video mode

44Ah (word)    screen column width (80)

44Ch (word)    screen regeneration length (4000)

44Eh (word)    screen location offset (0)

450h (word)    current cursor location

460h (word)    current cursor mode

462h (byte)    current display page (0)

463h (word)    port address of 6845 video emulator

465h (byte)    CRT mode setting

46Ch (Dword)   timer count

470h (byte)    timer roll over

478h (byte)    time out for printer

500h (byte)    print-screen indicator

All other bytes from 400h to 5FFh are set to 0 (zero).